March 21, 2012

RE:  CO2 Storage Resources

Hello everyone,

Lynn and Paul asked me to comment on Paul’s porosity approach, which I like, except I don’t follow the twist at the end. So, here goes.

Just a little background on log porosity evaluation in dolomites, as I understand it.  Prior to the mid-80’s, Schlumberger set 2.87 gm/cc as the grain density of dolomite, and then in 1988 they had a spasm and moved it to 2.88, before settling on the modern value of 2.85.  Some variability is to be expected as to iron content in dolomite as it grades towards ankerite.  Basically “field dolomite” is not the museum-grade pure dolomite that we studied in GEOL101.  An old hand in Schlumberger told me that much of the early work on logging in dolomites was made in north Texas which was used to calibrate the neutron logs as well as set the grain density.  In my experience, Arbuckle core sample grain densities plateau at 2.85 where quartz content is minimal. I am guessing that Paul’s value of 2.83 is some kind of Arbuckle average, based on core grain densities and reflecting an average quartz content in the dolomite.  A nice concept because it locks logs and core together and uses a parameter which has a distinct and “constant” physical meaning that is common to logging tools across companies and through time.   Now, it may be that there is a geographic drift in the average grain density caused by lateral changes in chert and calcite content. This could be checked, if necessary, in a “second order model” as a correction but it may be that the effects are insufficient to improve the result and that we are already at “fit for purpose”.

Now, the average of the neutron and density porosities, both calibrated to limestone, is generally a useable approximation to volumetric porosity. A more precise (more accurate??) value would be given by a crossplot of which there are MANY – across companies and across time as different styles of neutron tools came and went. And it doesn’t seem to be getting much better. Look at a Schlumberger LAS file and you get not one, but three neutron porosities, NPHI, NPOR, TNPH.  I am told that TNPH is the best, but if I have a regional database with lots of CNLs to stick with NPHI.  It seems to be a wilderness of mirrors if we fixate on accurate porosities from the neutron logs.

Okay, back to the Arbuckle. We have an estimate of average volumetric porosity by using a grain density of 2.83.  This is different from the limestone-equivalent porosity that would be the conventional density porosity, based on 2.71. So, why are we taking the extra step of averaging this volumetric estimate with the limestone-equivalent neutron porosity?

I would be interested to know the derivation of 2.83 as an Arbuckle grain-density. Whatever number is chosen is locked into average quartz content given by interpolation between 2.65 and 2.85, if you neglect any limestone.  So …  10% quartz? A quick analysis of core grain densities in Wellington 1-32 for an average would be a good check on this number.  A good feature of the density-calibration approach is that there will not be dramatic changes in the look of porosity maps and that any additional tweaking can be made by grain density adjustment if necessary.

Anyway, I know that many of these points are well-known to you all.  I just wanted to lay it all out to see if I am missing something.

Cheers,

John

Page 142, Atlas III

4.2. Saline Formation CO2 Storage Resource Estimates

Saline formations are composed of water-saturated porous rock and capped by one or more regionally extensive low-permeability rock formations. A saline formation assessed for CO2 storage is defined as a porous and permeable body of rock containing water with TDS greater than 10,000 ppm. A saline formation can include more than one named geologic stratigraphic unit or be defined as only a part of a stratigraphic unit. Mechanisms for CO2 storage in saline formations include structural trapping, hydrodynamic trapping, residual trapping, dissolution, and mineralization (Bachu et al., 2007; Kopp et al., 2009b; Xie and Economides, 2009). Structural, hydrodynamic, and residual trapping are initially the dominant trapping mechanisms and are the focus of this methodology.

Saline formations assessed for storage are restricted to those meeting basic criteria including:(1) adequate pressure and temperature conditions in the saline formation to keep the CO2 liquid or supercritical; (2) presence of a suitable seal system, such as a caprock, to limit vertical flow of the CO2 to the surface; and (3) a combination of hydrogeologic conditions to isolate the CO2 within the saline formation.

The storage of CO2 in saline formations is limited to sedimentary basins with vertical flow barriers and depths exceeding 800 meters. Sedimentary basins include porous and permeable sandstone and carbonate rocks. The 800-meter cutoff is an arbitrary attempt to select a depth that reflects pressure and temperature that yields high-density liquid or supercritical CO2. All sedimentary rocks included in the saline formation CO2 storage resource estimate must have seal systems consisting of low-permeability sealing rocks, such as shales, anhydrites, and other evaporates; however, the thickness of these sealing systems is not considered in this methodology. For increasing confidence in a storage resource estimates, other criteria including seal effectiveness (e.g., salinity and pressure above and below the seal system), minimum permeability, minimum threshold capillary pressure, and fracture propagation pressure of a seal system should be considered.

The volumetric equation to calculate the CO2 storage resource mass estimate (GCO2) for geologic storage in saline formations is:

GCO2 = At hg Øtot ρEsaline (2)

The total area (At), gross formation thickness (hg), and total porosity (Øtot) terms account for the total bulk volume of pore space available. The CO2 density (ρ) converts the reservoir volume of CO2 to mass. Rather than using an irreducible water saturation parameter explicitly, the storage efficiency factor (Esaline) reflects the fraction of the total pore volume that will be occupied by the injected CO2. As described in section 5.1., Esaline factors range between 0.40 and 5.5 percent over the 10th to 90th percent probability range. Table 2 summarizes the terms shown in eq 2.

5. CO2 Storage Efficiency for Resource Estimates

Carbon dioxide storage efficiency gauges the fraction of accessible pore volume that will be occupied by the injected CO2. In open systems, the fraction of accessible pore volume is estimated by geologic terms (area, thickness, and porosity) and displacement terms (areal, vertical, gravity, and microscopic displacement) (Lake, 1989). Monte Carlo sampling techniques, as described in Sections 5.1 and 5.2, were used to estimate efficiency factors for CO2 storage resource estimates for both saline formations and unmineable coal areas over the P10, P50, and P90 percent probability range. Efficiency in this methodology is comprised of statistical properties of geologic and displacement parameters.

5.1. Storage Efficiency of Saline Formations

For saline formations, the CO2 storage efficiency factor is a function of geologic parameters, such as area (EAn/At), gross thickness (Ehn/hg), and total porosity (Eφe/ φtot), that reflect the percentage of volume amenable to CO2 sequestration and displacement efficiency components, such as areal (EA), vertical (EL), gravity (Eg), and microsocopic (Ed), that reflect different physical barriers that inhibit CO2 from contacting 100 percent of the pore volume of a given basin or region (Bachu et al., 2007; Doughty and Pruess, 2004; Koide et al., 1992; Shafeen et al., 2004; van de Meer, 1992). Equation 5 describes the individual parameters required to estimate the CO2 storage efficiency factor for saline formations:

Esaline = EAn/At Ehn/hg EØe/ Øtot EA EL Eg Ed (5)

The net-to-total area EAn/At ratio is the fraction of the total basin or region area that is suitable for CO2 storage. The net-to-gross thickness Ehn/hg ratio is the fraction of the total geologic unit that meets minimum porosity and permeability requirements for injection. The effective-to-total porosity EØe/ Øtot ratio is the fraction of total interconnected porosity (Table 4).

The areal displacement (EA) efficiency is the fraction of planar area surrounding the injection well that CO2 can contact. This term is influenced by areal geologic heterogeneity, such as faults or permeability, and by CO2 mobility (Lake, 1989). The vertical (geologic layering) displacement (EL) efficiency is the fraction of vertical cross section or thickness with the volume defined by the area (A) that can be contacted by the CO2 plume from a single well, which can be affected by the aquifer dip and by CO2 buoyancy (Lake, 1989). This term is influenced by variations in porosity and permeability between sub-layers in the same geologic unit. If one zone has higher permeability than other zones, the CO2 will fill this zone quickly and leave the other zones with less or no CO2. The gravity displacement (Eg) efficiency is the fraction of net thickness that is contacted by CO2 as a consequence of the density and mobility difference between CO2 and in situ water. In other words, 1-Eg is the portion of the net thickness not contacted by CO2 because the CO2 rises within the geologic unit. The microscopic displacement (Ed) efficiency is the fraction of water-filled pore volume that can be replaced by CO2 (Lake, 1989). This term is directly related to irreducible water saturation in the presence of CO2. For the areal, vertical, and gravity displacement terms, it is assumed that CO2 fully displaces all in situ fluids. Since 100 percent displacement of fluid is neither theoretically nor technically feasible, the microscopic displacement term identifies the fraction of pore space unavailable due to immobile in situ fluids (Figures 2 and 3). The displacement terms are shown schematically in Figures 2 and 3 and compiled into Table 4.

Efficiency estimates using Monte Carlo sampling are based on statistical properties, such as mean values, standard deviation, ranges, and distributions, that describe geologic and displacement parameters. Little information is known regarding the statistical characteristics of saline formations because geologic parameters and formations are not well characterized (Bachu et al., 2007; Burruss et al., 2009; 2006, 2008, 2010b; Doughty and Pruess, 2004; Gorecki et al., 2009a; Gorecki et al., 2009b; Gorecki et al., 2009c; IEA, 2009). Recently, the International Energy Agency (IEA [2009]) and Kopp et al. (2009a,b) used field data from oil and gas reservoirs and numerical simulations employing relative-permeability data for CO2-brine systems measured in the laboratory (Bennion and Bachu, 2008) to predict appropriate ranges for geologic and displacement parameters for saline formations as a function of lithology. A similar report is also available from Gorecki et al. (2009a; 2009b; 2009c). It was assumed that saline formations do not differ fundamentally from oil and gas reservoirs (IEA, 2009; Kopp et al., 2009a). Table 4 includes values reported by IEA (2009) of the P10 and P90 ranges of geologic and displacement parameters for clastics, dolomite, and limestone lithologies for saline formations.2 The P10 notation reflects that there is a 10 percent probability that the value is less than the P10 value, and the P90 notation reflects that there is a 90 percent probability that the value is less than the P90 value. Because of the difficulty in separating the EA, EL, and Eg displacement terms shown in eq 5 in a heterogeneous scenario, these terms were combined by IEA (2009) into a single volumetric displacement term, EV.

Figure 2: Top-view of injection well and plume area. The area within the irregular shape inside the circle is the areal view of the 3-dimensional CO2 plume (A). The area inside the larger circle (B) is the accessible pore volume for areal displacement. The areal displacement term, EA = net area contacted by CO2 (A) / Total area (B).

Figure 3: Side view of injection well and plume area. The outer vertical dotted lines are defined by the outer areal circle (Depicted by B in Figure 2). The “plume” area enclosed within each interval that is bound by vertical dashed lines represents the numerator of the EL term (area enclosed within C); the denominator is the entire space outlined by the dotted line (area enclosed within D). Within the area bound by the dashed lines, the lower portion is not contacted due to gravity (area depicted by E) and is removed by the Eg term. The Ed term then defines the CO2 displacement efficiency in the plume region.

In this methodology, efficiency, as estimated by Monte Carlo sampling, for saline formations was based directly on the P10 and P90 ranges for net-to-gross thickness Ehn/hg, effective-to-total porosity Eφe/ φtot, volumetric displacement (EV), and microsocopic displacement (Ed) as reported by IEA (2009) (Table 4). Because no documented data for the area EAn/At term are available, it was assumed that CO2 will occupy between 20 and 80 percent of the formation for the purposes of these simulations (DOE-NETL, 2006, 2008).The equation, parameters, symbols, ranges, and description used to calculate efficiency for saline formations are summarized by eq 6 and Table 4.

Esaline = EAn/At Ehn/hg EØe/ Øtot Ev Ed               (6)

The area EAn/At, thickness Ehn/hg, and porosity Eφe/ φtot terms gauge the percentage of volume that is amenable to CO2 sequestration. The volumetric displacement term (Ev) corrects for the effective CO2 plume shape. The microscopic displacement term (Ed) corrects for the accessible pore volume available to CO2.

Efficiency (Esaline) was estimated from the individual terms in eq 6 by Monte Carlo sampling. Each individual term in eq 6 is given by a fraction, p. Various parametric distribution functions, such as normal, uniform, and lognormal, could be used to represent the distributions of the p’s. Currently, there is not enough data available to support assigning a specific distribution function to each of the individual terms in eq 6 at the regional and national scale. Since the p’s are fractions, they are constrained to the range between 0 and 1. Thus, the most appropriate distribution functions will be those that are constrained to the range between 0 and 1. Two distribution functions meeting this criterion and considered in this work are the beta distribution and the log-odds normal distribution. While both distributions are appropriate, the log-odds normal distribution, also known as the logistics-normal distribution (Aitchison and Shen, 1980), was chosen because of its ability to directly integrate the P10 and P90 ranges of geologic and displacement parameters provided by IEA (2009) as presented in Table 4. It was assumed that the individual efficiency terms in eq 6 could all be represented using a log-odds normal distribution at the regional and national scale. From the limited data available (IEA, 2009), all parameters were assumed to be independent since no correlation was found between the parameters. However, parameters may be linked at the site-specific scale.

The log-odds normal distribution transforms a fraction, p, by eq 7 and assumes that the transformed variable can be normally distributed.

X = ln (p/(1-p))     (7)

The distribution is so named because the p/(1–p) term in eq 7 is the “odds” for a fraction or probability p; therefore, ln[p/(1–p)] is the “log odds.” The use of this distribution is referred to as the log odds method when applied with Monte Carlo sampling (Devore, 2004). The transformed variable, X, is then normally distributed and sampled with Monte Carlo techniques. Then, the X value is transformed back to the corresponding p value by eq 8, which is the inversion of eq 7:

p = 1/(1+e-X)      (8)

Since the relationship between eqs 7 and 8 is monotonic, X10 and X90 ranges of geologic and displacement parameters provided by IEA (2009) can be computed directly from P10 and P90 ranges, respectively, using eq 7.

The log odds approach thus transforms p values of a range into corresponding X values of a range. This allows the mean and standard deviation of X to be determined from the X10 and X90 values. The mean and standard deviation of X fully specify its normal distribution, and these moments are then used as input parameters into the Monte Carlo sampling tools. The P10 and P90 values of the ranges presented in Table 4 were converted to X10 and X90 values by eq 7 and are shown in Table 5.

The mean (μx) and standard deviation (σx) are calculated from the X10 and X90 values using standard relationships between the percentiles and moments of a normal distribution

σX = (X90-x10)/(Z90– Z10)                (9)

μX = X10 – σX Z10                         (10)

where Zp is the Pth percentile value of the standard normal distribution. In this case, Z10 equals -1.28 and Z90 equals 1.28. Note that the standard deviation is computed first using eq 9, and this value is then used to compute the mean in eq 10. The values of the moments for X computed using eq 9 and 10 are shown in Table 6.

Monte Carlo sampling, using the commercial program GoldSim, was run using the mean (μx) and standard deviation (σx) values tabulated in Table 6 as input parameters. The respective X values are sampled using normal distributions with a sample size of 5,000 iterations for each. The corresponding values of p are computed using eq 8, and the individual p values are multiplied together to determine the storage efficiency factor E as shown in eq 11:

E = p (EAn/At) p (Ehn/hg) p (Eφe/σtot) p (Ev) p (Ed) (11)

or equivalently,

E = (1/(1 + e-X(EAn/At))(1 + e-X(Ehn/hg))(1 + e-X(Eφe/σtot))(1 + e-X(Ev))(1 + e-X(Ed))

A value of E is thus obtained for each of the 5,000 simulations, and the overall percentiles for the computed E are then estimated. Ranking from smallest to largest, the 500th result corresponds to P10, the 2,500th result corresponds to P50, and the 4,500th result corresponds to P90. These results are shown in Table 7.

The overall efficiency for saline formations ranges from 0.40 to 5.5 percent for the three different lithologies over the 10 and 90 percent probability range, respectively. These efficiency factors are based on documented ranges derived from oil and gas reservoirs and numerical simulations (IEA, 2009). With previous versions of the Carbon Sequestration Atlas of the United States and Canada, geologic and displacement parameters were not based on documented ranges (DOE-NETL, 2006, 2008). These saline formation efficiency factors ranged between 1 and 4 percent over the P15 and P85 percent probability range (DOE-NETL, 2006, 2008). When undocumented ranges for saline formations for previous editions of the Atlas (DOE-NETL, 2006, 2008) were applied using the log odds method described here, the P10, P50, and P90 percent probability ranges were 0.51 percent, 2.0 percent, and 5.5 percent, respectively. While the two sets of input ranges generate similar overall efficiency factors for saline formations, the efficiency factors reported here are based on documented P10 and P90 ranges of geologic and displacement parameters for clastics, dolomite, and limestone lithologies and appropriate distribution functions, log-odds normal in this case, that are constrained to the range between 0 and 1 whereas previous efficiencies were not.

In the case where net-to-total area EAn/At, net-to-gross thickness Ehn/hg, and effective-to-total porosity Eφe/ φtot are known for a region or basin, the geologic efficiency values can be used directly in eq 6. In this instance, only the displacement efficiency factor is needed, which ranges between 7.4 and 26 percent over the 10 and 90 percent probability range (Table 8).

Overall, CO2 storage resource estimates for saline formations are calculated from volumetric parameters (eq 2) and efficiency factors (eq 6) over the P10, P50, and P90 percent probability range (Tables 7 and 8).

GCO2 = At hg φtot ρEsaline           (2)

P10    Esaline = EAn/At Ehn/hg Eφe/ φtot Ev Ed                   (6)

P50     Esaline = EAn/At Ehn/hg Eφe/ φtot Ev Ed

P90   Esaline = EAn/At Ehn/hg Eφe/ φtot Ev Ed

P10 and P90 serve as nominal lower and upper bounds that demark a plausible range of efficiency factors, defined in a consistent probabilistic manner. If the 10th and 90th percentile values of the individual terms are properly specified for the targeted application, such as geologic storage, and the distributions for each term are independent and reasonably represented by the log-odds normal assumption, then the computed 10th and 90th percentile values for efficiency factors are properly estimated. However, because these limits are based on a combination of data with varying quality and expert judgment, the P10 and P90 limits should be interpreted as general, rather than strictly mathematical, limits. That is, with reasonable 10th and 90th percentile limits chosen for each factor, the results provide reasonable 10th and 90th percentile limits for efficiency factors.