November 13, 2002

Mr. Pat Canaday

Vess Oil Corporation

8100 East 22nd–North, Building 300

Wichita, KS  67226

Re:	MARCITsm Polymer Gel Water Shut Off Treatment

Bemis-Shutts Field – Ellis County, Kansas

McCord A #4 Producing Well – Arbuckle Formation

Dear Pat:

Attached is the job log and injection plots for the MARCITsm polymer gel water shut off treatment performed on the McCord A #4 producing well in the Bemis-Shutts Field located in Ellis County, Kansas.  A job recap is presented below.

#### PURPOSE

Use MARCITsm polymer gel technology to 1) decrease water production, 2) lower producing fluid level, 3) improve draw-down on oil-saturated reservoir matrix rock, 4) improve oil recovery and well economics.

#### TREATMENT

TIORCO equipment and personnel arrived on location at 8:30 a.m. on November 12, 2002.  A tailgate safety meeting was held to discuss all potential hazards specific to the jobsite.  The discharge line from TIORCO’s portable polymer pumping unit #14 was then connected to the wellhead, and a suction line was connected to frac tanks that held the mix water.  Finally, the TIORCO portable unit was connected to an electricity source. The following table provides details for each stage of the treatment.

| Stage | Date Begin | Time Begin | Date End | Time End | WC204Ò Polymer Ppm | WC204Ò Polymer Lbs. | WC684Ò X-Linker Lbs. | Gel Bbls. | PSIG Begin | PSIG End | Pump Rate Begin (BPM) | Pump Rate End (BPM) | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 11/12/02 | 10:00 a | 11/12/02 | 7:35 p | 3,500 | 550 | 119 | 448 | Vac | 150 | 0.75 | 0.75 |  |
| 2 | 11/12/02 | 7:35 p | 11/12/02 | 9:37 p | 6,000 | 200 | 43 | 95 | 150 | 335 | 0.75 | 0.75 |  |
| Totals |  |  |  |  |  | 750 | 162 | 543 |  |  |  |  |  |

The treatment was overflushed with 100 barrels of clean lease crude oil.

MARCITsm GEL QA/QC

Representative samples of cross-linked polymer solution were collected during all treatment stages to ensure that the intended gels would ultimately form.  Pre-gel samples were stored at a temperature of 120F in an oven onboard the TIORCO portable polymer injection unit.  All samples indicated that gels formed as intended.  Below is a table that provides gel sample information.

| Sample No. | Treatment Stage | Sample Date | Sample Time | Polymer ppm | Polymer:X-Linker Ratio | Gel Grade | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 11/12/02 | 12:00 p | 3,500 | 40:1 | C-D |  |
| 2 | 2 | 11/12/02 | 8:00 p | 6,000 | 40:1 | G |  |

The gel grading system developed by Marathon Oil Company for MARCITsm and MARA-SEALsm gels is defined as follows:

Grade		Description

A		No detectable gel formed.  The gel appears to have the same viscosity (fluidity) as the original polymer solution and no gel can be visually detected.

B		High flowing gel.  The gel appears to be only slightly more viscous than the initial relatively low viscosity polymer solution.

C		Flowing gel.  Most of the obviously detectable gel flows to the bottle cap upon inversion.

D		Moderately flowing gel.  A small portion (about 5 to 15%) of the gel does not readily flow to the bottle cap upon inversion – usually characterized as a “tonguing” gel (i.e., after hanging out of the bottle, the gel can be made to flow back into the bottle by slowly turning the bottle upright).

E		Barely flowing gel.  The gel slowly flows to the bottle cap and/or a significant portion (>15%) of the gel does not flow to the bottle cap upon inversion.

F		Highly deformable nonflowing gel.  The gel does not flow to the bottle cap upon inversion (gel flows to just short of  reaching the bottle cap).

G		Moderately deformable nonflowing gel.  The gel flows about halfway down to the bottle cap upon inversion.

H		Slightly deformable nonflowing gel.  The gel surface only slightly deforms upon inversion.

I		Rigid gel.  There is no gel-surface deformation upon inversion.

J		Ringing rigid gel.  A tuning-fork like mechanical vibration can be felt after the bottle is tapped.

TIORCO is very interested in monitoring and evaluating the results of this treatment with time.  If you should have questions or comments regarding the job, please do not hesitate to call me in our Ft. Worth, TX office at 817-431-6336, or Randy Prater in our Hays, KS office at 1-785-625-3791.  We greatly appreciate the opportunity to be of service to Vess and look forward to working with you again in the future.

Best regards,

J. Portwood

Southern Region Manager

Attachment

cc:	Joel Stull – Vess (Natoma, KS)

Randy Prater – TIORCO (Hays, KS)

TIORCO - Denver, CO