ArcGIS VOLUMETRIC CALCULATION INSTRUCTIONS:

Note: Due to limited character space, some files were abbreviated.  To avoid confusion, use the filenames that are given.

- Load the “Landfill GPS points” shapefile into ArcMap.

- Create a tension spline interpolation surface (Spatial Analyst) using “Z” (elevation values) in the z-value field.  Then add contours to the spline with 0.5 unit intervals.

- Load the previously created layers into ArcScene.

- Set each layers base height to that of the tension spline.

- In the scene properties dialogue box, click “calculate from extent”.

- Load the “future_fill_surface” shapefile into ArcMap.

- Create a tension spline interpolation surface (Spatial Analyst) using the “future_fill_surface” points with “Z” in the z-value field.  Save it as file name “ffs_tn_spline”.

- Load the “future_fill_surface” shapefile and the newly created “ffs_tn_spline” file into ArcScene.

- Set the layers base heights to that of the newly created interpolation surface.

- To find the volume available, use the cut/fill (3D Analyst) process, entering “ffs_tn_spline” as the before surface and the “tens_spline” as the after surface.  Since the coordinate system is in UTM, and measures in meters, the output cell size needs to be changed to “1”.

- In ArcMap, load “18inch_soil_overlay”

- Create a tension spline interpolation surface (Spatial Analyst) using the “18inch_soil_overlay” points with “Z” in the z-value field.  Save it as file name “ovrly_tn_spli”.

- Load the “18inch_soil_overlay” shapefile and the newly created “ovrly_tn_spli” file into ArcScene.

- Set the layers base heights to that of the newly created interpolation surface.

- To find the volume of 18” overlay surface, use the cut/fill (3D Analyst), process entering “ovrly_tn_spli” as the before surface and the “ffs_tn_spline” as the after surface.  Since the coordinate system is in UTM, and measures in meters, the output cell size needs to be changed to “1”.

* When assessing volume calculations, the “Net Loss” equals the volume available.