Drill Stem Test (DST) Data Entry & Quantitative Analysis Applet  by John R. Victorine

Introduction

The DST web app has 2 sources for importing well data, 1) the user’s PC or 2) the Kansas Geological Survey (KGS) Server & ORACLE Database.  This program allows the user to import Well Header Information and Drill Stem Test Data.

There will be occasions when the user would like to inspect the contents of a LAS file. So, for example, the user may wish to find the identity of the well, the types of logs, depth range, well or log parameters. Alternatively, the user can suspect that the file is not coded correctly in LAS format if the Import LAS function fails, and wishes to examine the file to troubleshoot the problem. The LAS file is an ASCII file and can be read by any text editor, i.e. Notepad, WordPad, TextPad, etc.

To access Drill Stem Test (DST) Data Entry & Quantitative Analysis Applet (http://www.kgs.ku.edu/software/DST/applet.html) go to the top of the web page there is a menu "Main Page|Description|Applet|Help|Copyright & Disclaimer|".  Select the "Applet" menu option a "Warning - Security" Dialog will appear.

This program has to be able to read and write to the user’s PC and access the Kansas Geological Survey (KGS) Database and File Server, ORACLE requires this dialog. The program does not save your files to KGS, but allows you to access the KGS for well information that may be missing in your Kansas logs. The program does not use Cookies or any hidden software it only reads the LAS file for the DST Session and writes a LAS 3.0 File to your PC to save your DST Session and the well data imported. The blue shield on the warning dialog is a symbol that the Java web app is created by a trusted source, which is the University of Kansas.  Select the "Run" Button, which will show the DST "Enter" Panel illustrated below,

# Contents

Drill Stem Test (DST) Data ………………..………………………………………………………………………………………………….. 3 Data Source Panel	………………………………………………………………………………………………………………………... 3

Importing KGS (Database & Server) Data …….…………………………………………………………………………………….… 5 Importing Drill Stem Test Data …………………………………………………………………………………………….………... 5

Example: Wellington KGS 1-32 Digitizing Pressure/Temperature vs. Time Plot ……………..……………………… 9  Add New DST Panel Tab …………….………………………………………………………………………………………………… 12 Pressure Summary Tab ………………………………………………………………………………………………………………… 13 Recovery Tab ……………………………………..……………………………………………………………………………………….. 14 Gas Rates Tab ……………………………………………………………………………………………………………………………… 15

Digitize DST 1 Pressure/Temperature vs. Time Plot ……...…………………………………………………………………… 16 (1) Load DST Image File …………………………………………………………………………..…………………………….…….. 19 (2) Set the Axis Direction …………………………………………………………………………………………………………….. 20 (3) Set Minimum & Maximum Capture Areas ………………………………………………………………………………. 21 (4) Add Pressure Summary Data ……………………………………………………………………………………………….…. 26 (5) Digitize Data - Pressure ……………………………….………………………………………………………..……………….. 27 (5) Digitize Data - Temperature ……………………………………….………………………………………………………….. 29 (6) Normalize Data ….……………………..……………………………………………………………………………………………. 31 (7) Transfer Data …………………………………………………………………………………………………………………………. 32

Horner Plot – Fluid DST ……...……………………………………………………………….……………………………………………… 34 Conventional Horner Type Pressure Buildup Analysis Conditions Assumed for Analysis Procedure 34 Method of Construction of Horner plot - Fluid Drill Stem Test …………………………………………………….. 35 Transmissibility (K*h/μ), Permeability Thickness (K*h), Permeability (K) ……………………………………… 38 Damage Ratio (DR), Skin factor (S), and related quantities ……………………………………………………….…. 39

Horner Plot – Gas DST ……...………..……………………………………………………….……………………………………………… 42 Conventional Horner Type Pressure Buildup Analysis Conditions Assumed for Analysis Procedure 42 Method of Construction of Horner plot - Gas Drill Stem Test ……………..……………………………………….. 43 Transmissibility (K*h/μ), Permeability Thickness (K*h), Permeability (K) ……………………………………… 46 Damage Ratio (DR), Skin factor (S), and related quantities ……………………………………………………….…. 47

Save DST Test Data in Log ASCII Standard (LAS) version 3.0 File ……...………………………………………………… 51

Example: Wellington KGS 1-32 - Previous Session …………………………...………………………………………………… 58 Importing PC Data - Download Well Data to PC – Previous Session ……………………………………………… 58

Drill Stem Test (DST) Control Dialog

Click the "DST Enter” Icon Button, which will show the "Drill Stem Test (DST)" dialog. The dialog below displays an example of the Wellington KGS 1-32 well DST data loaded from the “Prev (Previous) Session” image icon button with the data.  The icon buttons in the Data Source Panel assists the user in loading well data into the DST Applet.

Data Source Panel

The Data Source Panel provides two methods of importing data into the Profile Web App.  The Kansas Geological Survey (KGS) Database and the user’s PC, a Digitizer is also provided to convert a DST Pressure/Temperature vs. Time Image to digital data.  A number of icon buttons are provided to assist the user in importing the specific data type of interest.

Importing KGS (Database) Data

KGS (Database) - Importing Drill Stem Test Data

The Kansas Geological Survey (KGS) has a good collection of well data stored in the ORACLE Database. In this example the user will download the well header information & Drill Stem Test Data from the KGS database. The ORACLE Database is accessed by making Stored Procedure PL/SQL calls to the ORACLE Database from which an Extensible Markup Language (XML) data stream is created containing the well data that is passed back to the web app making the request.

Left Click on the DST Image Icon Button in the Data Source Panel to display the “Search KGS Database for Trilobite DST Data” dialog.

As the Summary image suggests there are 5 methods for searching for the Trilobite Drill Stem Test Data in the KGS Database,

- By API-Number – KGS has a specific format for the API-Number, i.e.SS-CCC-99999 where SS is the state code for Kansas 15, CCC is the county code for Zachary Alexander 1-2 it is 187 for Stanton County and the 5-Digit Well Number for the Zachary Alexander 1-2 is 20851.

- By (Partial) Lease name - The stored procedure used to retrieve the well header information allows the user to enter a partial phrase, in this example Zachary.  The program places a ‘%’ in front and back of the phrase and sends the request to the Database, i.e. “%Zachary%”.

- By Township Range Section – This search is by area in Kansas, this search also allows the user to enter just the Township and Range to search for wells, e.g. to look for the Zachary Alexander 1-2, enter Township as 30 set the S (South) Radio button and Range as 40 set the W (West) Radio button.

- By (Partial) Field name - This search is by area in Kansas, the stored procedure used to retrieve the Drill Stem Test Data allows the user to enter a partial field name phrase, in this example Arroyo Creek.  The program places a ‘%’ in front and back of the phrase and sends the request to the Database, i.e. “%Arroyo Creek%”.

- By County – This search is by area in Kansas, the user can search by county by selecting either the “Counties A-L” for counties that begin with the letter A through L or “Counties M-Z” menu for counties that begin with the letter M through Z. In this example Zachary Alexandra 1-2 is in the Stanton County so the user would select the “Counties M-Z” menu for a list of counties and select “Stanton” menu option to retrieve all wells with Trilobite Drill Stem Data from the KGS Database Table in Stanton County.

The user only needs to enter the above data and select the “Search” Button to display the list of Wells in the Kansas Database that match the search criteria.  In the image below the Field Name “Arroyo Creek” is entered to search for all wells in Kansas with the Phrase Wellington in it. The user searches through the list until they find the well of interest.  In this example it is the Wellington KGS 1-32, which is highlighted.

The user clicks on the “Select” button to transfer the header information to the Header Information Panel on the DST dialog and import the Drill Stem Test Data stored in the KGS Database to the DST Dialog panels.

Example: Wellington KGS 1-32                                                                        Digitizing Pressure/Temperature vs. Time Plot

Import Well Header Information from Kansas Geological Survey (KGS) Database

The Kansas Geological Survey (KGS) has a good collection of well data stored in the ORACLE Database. In this example the user will download the well header information from the KGS database. The ORACLE Database is accessed by making Stored Procedure PL/SQL calls to the ORACLE Database from which an Extensible Markup Language (XML) data stream is created containing the well data that is passed back to the web app making the request.

Left Click on the “Well Info” Icon Button in the Data Source Panel.

This will display the “Search for Data on KGS Server” Dialog, see above image.  This dialog allows the user to search the KGS database for well header information.  In this example, the well of interest will be the Wellington KGS 1-32.  This example assumes the user will eventually digitize the DST Pressure/Temperature vs. Time Plots and eventually saving the data will require the Header Information Panel be filled with expected data and this search will help the user download well header information of the well of interest.

As the Summary image suggests there are 3 methods for searching for the well header information within this dialog,

- By API-Number – KGS has a specific format for the API-Number, i.e.SS-CCC-99999 where SS is the state code for Kansas 15, CCC is the county code for Wellington KGS 1-32 it is 191 for Sumner County and the 5-Digit Well Number for the Wellington KGS 1-32 is 22591.

- By Partial Lease Name – The stored procedure used to retrieve the well header information allows the user to enter a partial phrase, in this example Wellington.  The program places a ‘%’ in front and back of the phrase and sends the request to the Database, i.e. “%Wellington%”.

- By Township Range Section – This search is by location in Kansas, this search also allows the user to enter just the Township and Range to search for wells, e.g. to look for the Wellington KGS 1-32, enter Township as 31 set the S (South) Radio button and Range as 1 set the E (East) Radio button.

The user only needs to enter the above data and select the “Search” Button to display the list of Wells in the Kansas Database that match the search criteria.  In the image below the Lease Name “Wellington” is entered to search for all wells in Kansas with the Phrase Wellington in it. The user searches through the list until they find the well of interest.  In this example it is the Wellington KGS 1-32, which is highlighted.

The user clicks on the “Select” button to transfer the header information to the Header Information Panel on the DST dialog.

Add New DST Panel Tab

The user has to create a DST Data Tab before the user can access the digitizer.  Enter the name for the first DST “DST 1” in the “DST Name / ID / Number (Tab ID)” text field and then tab out of the text field, the “Add” button will them be enabled.  Select the “Add” Button to create the “DST 1” tab with all embedded panels to collect the DST 1 Report Data.

The information that is contained in a drill-stem test report will contain the following,

- Factual data including statistical well information and a description of the testing tools.

- Measured data concerning the recovered fluids and their properties, the time periods the recovered fluids and their properties, the time periods involved and general remarks based on observations during the test

- Recorded pressure and temperature data as a Pressure/Temperature vs. time plot, which will require digitizing to retrieve the data for the Horner Plot and Quantitative Analysis.

- Interpretation calculations, where applicable.

The DST data panels are designed to emulate the Trilobite Drill Stem Test Report Format as an example.  The GEMINI Project created a DST Database table was created so donated Trilobite Test Data could be loaded into the table.   In this example the DST report was provided but the digital drill stem test data is not just the Temperature/Pressure vs. time plot.  This web app does not replace the original DST Report, but provides a method for saving the data in a format that can be shared with other programs.  Once the DST Report data is transferred to this web app and the DST plot is digitized the data can be saved to a Log ASCII Standard (LAS) version 3.0 file.

The next three panels illustrate the Trilobite Drill Stem Test Report pages and the corresponding Drill Stem Test Dialog Panels for the “DST 1” Tab.

The DST 1 Tab Panel contains General Information Panel that displays will hold the General Information in the Drill Stem Test Report.  The lower have of the DST 1 Tab Panel contains 4 tabs, Pressure Summary, Recovery, Gas Rates and Shut-In Pressure Data panels.  The first three is for DST information; the Gas Rates panel is only filled if the DST Report was done for a Gas DST test.  The Shut-In Pressure Data panel holds the digital data for the Pressure/Temperature vs. Time Data.  The Wellington KGS 1-32 DST tests was done on a Fluid based well so for the last page another well is shown to illustrate the contents of the Gas Rates panel.

Digitize DST 1 Pressure/Temperature vs. Time Plot

This example the user can download DST 1 report & a Portable Network Graphics (PNG) image file from the KGS Server. Download the following files into your PC.

Wellington KGS 1-32: Drill Stem Test 1 Report and Pressure/Temperature vs. Time PNG Image.

| Type | File URLs |
| --- | --- |
| Report | http://www.kgs.ku.edu/Gemini/Tools/documentation/Wellington_KGS_1-32-DST-1-Report.pdf |
| DST 1 | http://www.kgs.ku.edu/Gemini/Tools/documentation/Wellington_KGS_1-32-Miss.png |

The first file is the Portable Document Format (PDF) Document of the DST 1 Report.  The Second file is an image capture of the Pressure/Temperature vs. Time Portable Network Graphics (PNG) Image that will be used in the digitizing process to capture the Shut-In 1 Pressure, Temperature and Time data and the Shut-In 2 Pressure, Temperature and Time data.

Both Shut-In Pressure/Temperature vs. Time data is needed to compute the Initial static reservoir pressure, Pi.

Left Click the on the “Digitizer Plot” image icon button to display the “DST Plot Digitizer” dialog.

The Digitizer was set up as a 7 step process. The following web pages will walk the user through each step to load a DST Pressure vs. Time Plot into the digitizer dialog. To set the data collection areas and to digitize and normalize the shut in pressure temperature and time data for the Horner Plot & Quantitative Analysis dialog.

Note: The DST Pressure vs. Time must be a PNG (Portable Network Graphics), JPEG (Joint Photographic Experts Group) or GIF (Graphics Interchange Format) image file.

(1) Load DST Image File

(2) Set the Axis Direction

(4) Add Pressure/Temperature Summary Data

(5) Digitize Data - Pressure

(5) Digitize Data – Temperature

(5) Digitize Data – Summary

(6) Normalize Data

(7) Transfer Data

To display the “Horner Plot and Quantitative Analysis” dialog click on the “Display Horner Plot” button in the “Shut-In Data” panel.

Horner Plot – Fluid DST

Much of this section is taken from the lecture notes "Modern Concepts in Drill Stem Testing Part 2 Quantitative Analysis"2 by Hugh W. Read, Author & Instructor.

### Conventional Horner Type Pressure Buildup Analysis Conditions Assumed for Analysis Procedure

For a discussion of the theory of pressure buildup interpretation and the derivation of the pertinent equations the reader is referred to the original now classical paper by Horner(1).

90% of all quantitative Drill Stem Test analysis is based on Horner's method of analysis which assumes the following ideal conditions:

- Radial flow -Radial flow is flow into the well bore equally from all directions in the formation. By and large condition is met by most DST's through linear systems such as fractures, narrow channel sands etc. are obvious exception which may be handled separately.

- Homogeneous Reservoir - Homogeneity in a reservoir denotes constant characteristics throughout the length and thickness. Any values calculated from test data then become averages over the length and/or thickness.

- "Steady-State" conditions of flow - "Steady-state" flow conditions imply a rate and pressure drawdown (which is responsible for the flow) which is constant.

- Infinite Reservoir - An infinite reservoir is one in which there are no finite limits or depletion. All DST's in wildcat wells can be considered to test infinite reservoirs, except those testing very small local porosity or fracture developments which in any event deplete rapidly and are not commercial.

- Single Phase Flow - Single phase flow assumes that only one phase is entering the well bore. Thus for example any gas produced with oil is considered as solution gas and any liquids in a gas test are considered as condensation of gases in the pipe or immediate well bore vicinity.

### Method of Construction of Horner plot - Fluid Drill Stem Test

This example is from the Wellington KGS 1-32; DST 1 in the Mississippian from 3664.0 ft to 3690.0 ft dst-fluid.las. The Recovery and Pressure Summary Data will be needed in performing the horner plot and quantitative analysis and is listed as follows,

| Recovery Table. Length (ft); Volume (bbl); Description. 55.0; 0.27; MCW 40%m 60%w. 190.0; 0.934; MCW 25%m 75%w. 555.0; 5.043; MCW 10%m 90%w. ______; _____. 800.0; 6.247; Totals. |  | Pressure Summary Table. Time; Pressure; Temperature; Annotation. (min); (psig); (deg F). 0.0; 1916.58; 106.15; Initial Hydrostatic. 12.0; 68.92; 110.06; Open to Flow 1. 44.0; 239.67; 115.88; Shut-in 1. 109.0; 1047.57; 114.85; End Shut-in 1. 109.0; 257.73; 114.46; Open to Flow 2. 168.0; 414.1; 117.63; Shut-in 2. 348.0; 1049.42; 116.7; End Shut-in 2. 357.0; 1804.33; 114.39; Final Hydrostatic. |
| --- | --- | --- |

(1) The data necessary for this plot is a set of detailed readings of pressures and times during each shut in period plus the total amount of time spent flowing the well.

(2) Arrange the data in tabular form where,

T = Cumulative flowing time (minutes)
ΔT = Times during shut in that each pressure is read (minutes)
P = Pressure read during shut in time (psig)

| Table 3 Tabular Listing of Shut In Data for Horner Plot Calculation |  |  |
| --- | --- | --- |
| Shut In 1 (Initial). Total Flow Time (T) = 32 minutesa. Time (ΔT); (T+ΔT)/ΔT; Pressure (P); Temperature. (min); (psig); (deg F). 0.0; ---; 239.67; 115.88. 4.0; 9.00; 904.76; 115.56. 8.0; 5.00; 950.75; 115.24. 12.0; 3.67; 970.32; 114.92. 16.0; 3.00; 986.51; 114.87. 20.0; 2.60; 1001.44; 114.83. 24.0; 2.33; 1008.28; 114.78. 28.0; 2.14; 1014.37; 114.74. 32.0; 2.00; 1020.47; 114.69. 36.0; 1.89; 1026.49; 114.64. 40.0; 1.80; 1029.39; 114.64. 44.0; 1.73; 1032.30; 114.67. 48.0; 1.67; 1035.21; 114.71. 52.0; 1.62; 1038.12; 114.74. 56.0; 1.57; 1041.03; 114.77. 60.0; 1.53; 1043.93; 114.81. 64.0; 1.50; 1046.84; 114.84. |  | Shut In 2 (Final). Total Flow Time (T) = 59 + 32 = 91 minutesb. Time (ΔT); (T+ΔT)/ΔT; Pressure (P); Temperature. (min); (psig); (deg F). 0.0; ---; 414.1; 117.63. 8.0; 12.38; 905.06; 117.44. 16.0; 6.69; 946.13; 117.25. 24.0; 4.79; 967.73; 117.07. 32.0; 3.84; 985.14; 116.89. 40.0; 3.28; 996.56; 116.76. 48.0; 2.90; 1006.962; 116.63. 56.0; 2.63; 1012.76; 116.50. 64.0; 2.42; 1018.56; 116.46. 72.0; 2.26; 1022.48; 116.46. 80.0; 2.14; 1025.96; 116.46. 88.0; 2.03; 1029.44; 116.46. 96.0; 1.95; 1032.92; 116.46. 104.0; 1.88; 1034.80; 116.46. 112.0; 1.81; 1036.54; 116.47. 120.0; 1.76; 1038.28; 116.506. 128.0; 1.71; 1040.01; 116.537. 136.0; 1.67; 1041.46; 116.55. 144.0; 1.63; 1042.91; 116.58. 152.0; 1.60; 1044.35; 116.619. 160.0; 1.57; 1045.80; 116.63. 168.0; 1.54; 1047.25; 116.66. 176.0; 1.52; 1048.70; 116.69. |
| From the Pressure Summary Table (a) Shut-in 1 Time - Open to Flow 1 Time { 44.0 - 12.0 = 32.0 minutes } (b) Shut-in 2 Time - Open to Flow 2 Time + Total Flow Time from (a) { 168.0 - 109.0 + 32.0 = 91.0 minutes } |  |  |

In the table note that the Shut-In 1 (Initial) period was broken down into 4 minute intervals while only 8 minute increments was considered sufficient for the Shut-In 2 (Final) period.

The Pressure Summary Table gives a summary of the DST Pressure-Temperature-Time Readings at specific points. This DST had a 32.0 minute pre flow and a 59.0 minute main flow. Thus 'T' for the Shut-In 1 (Initial) period = 32.0 minutes since there was a total of 32 minutes flow time prior to Shut-In 1 (Initial). T for the Shut-In 2 (Final) period becomes 91 minutes (32 minutes pre flow + 59 minutes main flow since T is the cumulative flow time prior to shut in.

The quantity (T + ΔT)/ΔT is mathematically the same as 1 + (T/ΔT) thus for the Shut In 2 (Final) period example the (T + ΔT)/ΔT for the first increment = (91+8)/8 = 12.38, which is the same as 1 + (91/8) = 1 + 11.38 = 12.38.

(3) The Horner plot is then constructed by plotting log10[(T+ΔT)/ΔT] vs. P for each point.

(4) In figure above both Shut-In Pressure-Time curves have been plotted. In order to determine the static reservoir pressure the linear portion is extrapolated to the left hand margin where the value of the time function (T + ΔT)/ΔT = 1. Since the logarithm of 1 is zero the log10[(T + ΔT)/ΔT ] = 0 (or infinity) at this point. Thus we are extrapolating the Horner straight line to a 'infinite' shut in time where completely static pressure prevails. In the example here the static pressure ~ 1078.0 psig. Both Shut In intervals extrapolations yield this value. In cases where the Shut-In 2 (Final) extrapolation is substantially less than the Shut-In 1 (Initial) depletion is indicated. (If the depletion factor 100.0 * Shut-In [(Initial)-(Final))/(Initial) ] is greater than 4%, then the indications of depletion can be considered serious or substantial.

(5) The final piece of information which is obtained from the Horner plot is the "slope" (m) of the straight line segment. Numerically this is the difference between the pressure at log10[(T + ΔT)/ΔT ]=0 and at log10[(T + ΔT)/ΔT ]=1, where the slope is simply the change in pressure over one complete log cycle. The slope in this example using the Shut-In (Final) slope m=165.96 psi/cycle. The slope is used in the calculations to derive permeability and other reservoir parameters.

### Transmissibility (K*h/μ), Permeability Thickness (K*h), Permeability (K)

The well known equation describing pressure build-up characteristics of a well is expressed as

Theoretically, the plot of Pressure Po vs. log10[(T+ΔT)/ΔT] will result in a straight line;            Po = Pf - m * log10[(T+ΔT)/ΔT]

where 

Pf is the extrapolation of log10[ (T+ΔT)/ΔT ] to zero or formation pressure and
m is the slope of the line.

m = 162.6 * q * μ * B / (K * h) [psi] / [log cycle] 
q = flow rate [STB] / [day]
μ = viscosity [cp]
B = Formation Volume Factor [RB] / [STB]
h = vertical thickness of continuous porosity [ft]
K = average effective permeability [md]

This states that the slope of the buildup is representative of a given fluid having physical properties μ, B flowing at a rate q through a formation having physical properties K * h.

Note: the flow rate (q) is computed by using the total fluid volume in the recovery table and the total flow time computed from the pressure summary table and converting to days, i.e. q = (Total Fluid Volume [bbls] / Total Flow Time [min] ) * ( 60 [min] / [hr] ) * ( 24 [hr] / [day] ).

Transmissibility can be found my rearranging the slope as follows,

| Transmissibility: | K * h / μ = 162.6 * q * B / m | [md]-[ft]/[cp] |
| --- | --- | --- |
| Permeability Thickness: | K * h = ( Transmissibility ) * μ | [md]-[ft] |
| Permeability: | K = ( Permeability Thickness ) / h | [md] |

### Damage Ratio (DR), Skin factor (S), and related quantities

Damage Ratio (DR) - A numerical value used to predict the flow rate if the damage were removed. In essence the DR becomes a number which when multiplied by the flow rate which existed at the time of the test will yield the flow rate if the damage (or improvement) over the natural condition is removed. This flow rate will be the rate at the same flowing pressure drawdown which existed during the test. From the definition of DR if a zone is neither damaged nor improved then q theoretical is identical to q actual and DR = 1.

where,

| Pi | = Initial static reservoir pressure | [psig] |
| --- | --- | --- |
| Pwf | = Bottom Hole Flowing Pressure (FFP) | [psig] |
| m | = slope from the Horner plot | [psi] / [log cycle] |
| K | = Permeability ( K ) | [md] |
| To | = Flow Time | [min] |
| φ | = Porosity |  |
| μ | = viscosity | [cp] |
| c | = compressibility | [vol]/[vol]/[psi] |
| rw | = well bore radius | [ft] |

If K, φ, μ, c and rw are not known then the Damage Ratio can be estimated by,

Skin factor (S) - A numerical quantity which attempts to describe the amount of damage or improvement. The numerical value of skin is so defined that a positive value for the skin factor indicates a damaged condition and a negative skin factor indicates improvement over the natural condition. The skin factor is zero when neither skin damage nor improvement are present i.e. no alterations to natural condition.

where,

| Pi | = Initial static reservoir pressure | [psig] |
| --- | --- | --- |
| Pwf | = Bottom Hole Flowing Pressure (FFP) | [psig] |
| m | = slope from the Horner plot | [psi] / [log cycle] |
| K | = Permeability ( K ) | [md] |
| To | = Flow Time | [min] |
| φ | = Porosity |  |
| μ | = viscosity | [cp] |
| c | = compressibility | [vol]/[vol]/[psi] |
| rw | = well bore radius | [ft] |

If φ, μ, c and rw are not known then the Skin factor (S), assuming average values of φ=0.15, μ*c=0.00000955 [cp]/[psi] and for simplicity assume rw=0.333 [ft], can be estimated by,

Pressure Drop Across a Damage Zone ΔPskin The additional pressure drop during flow caused by the presence of skin (ΔPskin) may be calculated as follows,

ΔPskin = 0.87 * S * m

where,

| S | = Skin factor |  |
| --- | --- | --- |
| m | = slope from the Horner plot [psi] / | [log cycle] |

Productivity Index (PI) Productivity Index is a measure of a zone's ability to flow at a particular drawdown (reservoir pressure - flowing pressure). It may be formally defined as the flow rate [STB]/[day] per unit [psi] of pressure differential between the well bore at the formation face and the static reservoir pressure in the zone.

| K | = average effective permeability | [md] |
| --- | --- | --- |
| h | = vertical thickness of continuous porosity | [ft] |
| μ | = viscosity | [cp] |
| B | = Formation Volume Factor | [RB] / [STB] |
| re | = drainage radius | [ft] |
| rw | = well bore radius | [ft] |
| S | = Skin factor |  |

Stabilized Production Rate (qs) Once the stable PI has been derived the stable flow rate at any pressure may be calculated simply by multiplying the chosen drawdown by the PI.

qs = PI * ( Pi - Pwf )

where,

| Pi | = Initial static reservoir pressure | [psig] |
| --- | --- | --- |
| Pwf | = Bottom Hole Flowing Pressure (FFP) | [psig] |
| PI | = Productivity Index |  |

Radius of Investigation (RI) The radius of investigation is the radial distance from the well bore which the pressure disturbance from the DST has investigated (in other words "how far out into the zone the DST has looked").

RI = 2.6408 * K*To / (c))

where,

| K | = Permeability ( K ) | [md] |
| --- | --- | --- |
| To | = Flow Time | [min] |
| φ | = Porosity |  |
| μ | = viscosity | [cp] |
| c | = compressibility | [vol]/[vol]/[psi] |

If φ, μ, and c are not known then the Radius of Investigation (RI) is as follows,

For Water: RI = 6.83 * K * Tf)

For Oil: RI = 4.63 * K * Tf)

References: 
(1) Pressure Build-Up in Wells, by D.R. Horner, 3rd World Petroleum Congress, May 28 - June 6, 1951 , The Hague, the Netherlands http://www.onepetro.org/mslib/servlet/onepetropreview?id=WPC-4135&soc=WPC 
(2) Drill Stem Test Interpretation Seminar, Author & Instructor: H. W. Reid 
(3) Recent Developments in the Interpretation and Application of DST Data, by L. F. Maier Journal of Petroleum Technology Volume 14, Number 11, November 1962, pgs 1213-1222 http://www.onepetro.org/mslib/servlet/onepetropreview?id=00000290

Horner Plot – Gas DST

Much of this section is taken from the lecture notes "Modern Concepts in Drill Stem Testing Part 2 Quantitative Analysis"2 by Hugh W. Read, Author & Instructor.

### Conventional Horner Type Pressure Buildup Analysis Conditions Assumed for Analysis Procedure

For a discussion of the theory of pressure buildup interpretation and the derivation of the pertinent equations the reader is referred to the original now classical paper by Horner(1).

90% of all quantitative Drill Stem Test analysis is based on Horner's method of analysis which assumes the following ideal conditions:

- Radial Flow - Radial flow is flow into the well bore equally from all directions in the formation. By and large condition is met by most DST's through linear systems such as fractures; narrow channel sands etc. are obvious exception which may be handled separately.

- Homogeneous Reservoir - Homogeneity in a reservoir denotes constant characteristics throughout the length and thickness. Any values calculated from test data then become averages over the length and/or thickness.

- "Steady-State" Conditions of Flow - "Steady-state" flow conditions imply a rate and pressure drawdown (which is responsible for the flow) which is constant.

- Infinite Reservoir - An infinite reservoir is one in which there are no finite limits or depletion. All DST's in wildcat wells can be considered to test infinite reservoirs, except those testing very small local porosity or fracture developments which in any event deplete rapidly and are not commercial.

- Single Phase Flow - Single phase flow assumes that only one phase is entering the well bore. Thus for example any gas produced with oil is considered as solution gas and any liquids in a gas test are considered as condensation of gases in the pipe or immediate well bore vicinity.

### Method of Construction of Horner plot - Gas Drill Stem Test

This example is from the Xit 1; DST 1 in the Upper Kearny Member from 5690.0 ft to 5745.0 ft dst-gas.las. The Gas Rates and Pressure Summary Data will be needed in performing the plot and quantitative analysis and is listed as follows,

| Gas Rates Table. Flow; Elapsed; Pressure; Gas Rate; Choke. Period; Time; (psig); (Mcf/day); (in). 1; 10.0; 14.0; 577000.0; 1.0. 1; 20.0; 24.0; 817000.0; 1.0. 1; 30.0; 28.0; 907000.0; 1.0. 2; 10.0; 26.0; 865000.0; 1.0. 2; 20.0; 33.0; 1012000.0; 1.0. 2; 30.0; 35.0; 1057000.0; 1.0. 2; 40.0; 36.0; 1076000.0; 1.0. 2; 50.0; 36.0; 1076000.0; 1.0. 2; 60.0; 37.0; 1098000.0; 1.0. |  | Pressure Summary Table. Time; Pressure; Temperature; Annotation. (min); (psig); (deg F). 0.0; 2748.7854; 122.407204; Initial Hydrostatic. 5.5; 155.137894; 123.1147; Open to Flow 1. 36.0; 496.501312; 121.527901; Shut-in 1. 96.0; 853.742798; 126.989899; End Shut-in 1. 97.0; 233.796402; 126.3834; Open to Flow 2. 156.5; 435.617401; 115.143501; Shut-in 2. 280.0; 850.436829; 125.351997; End Shut-in 2. 285.5; 2727.0625; 126.5923; Final Hydrostatic. |
| --- | --- | --- |

See the paper "Recent Developments in the Interpretation and Application of DST Data" by L.F.Maier(3). There were a number of DST papers that seem to use the Pressure squared method. Reid suggests that the linear Horner plot was sufficient. A future version of the DST Tool may allow the user to select between Pressure or Pressure squared in a Horner plot analysis.

(1) The data necessary for this plot is a set of detailed readings of pressures and times during each shut in period plus the total amount of time spent flowing the well.

(2) Arrange the data in tabular form where,

T = Cumulative flowing time (minutes)
ΔT = Times during shut in that each pressure is read (minutes)
P = Pressure read during shut in time (psig)
P2 = Pressure squared (psig2)

| Shut In 1 (Initial). Total Flow Time (T) = 30 minutesa. Time (ΔT); (T+ΔT)/ΔT; Pressure (P); Temperature; Pressure squared (P2). (min); (psig); (deg F); (psig2). 0.0; ---; 496.501312; 121.527901; 246,512.25. 4.0; 8.50; 841.170715; 125.614502; 707,566.97. 8.0; 4.75; 844.496826; 126.780701; 713,180.25. 12.0; 3.50; 846.328003; 126.656998; 716,274.47. 16.0; 2.88; 847.812927; 126.512199; 718,781.80. 20.0; 2.50; 848.991882; 126.430603; 720,784.02. 24.0; 2.25; 849.629028; 126.429497; 721,871.14. 28.0; 1.07; 850.446777; 126.441704; 723,265.20. 32.0; 1.94; 850.998596; 126.487801; 724,201.00. 36.0; 1.83; 851.615723; 126.541397; 725,256.62. 40.0; 1.75; 851.901672; 126.6035; 725,733.61. 44.0; 1.68; 852.340271; 126.679298; 726483.48. 48.0; 1.63; 852.739502; 126.757896; 727,165.51. 52.0; 1.57; 853.130676; 126.837502; 727,830.80. 56.0; 1.54; 853.542114; 126.921898; 728,530.53. 60.0; 1.50; 853.742798; 126.989899; 728,871.99. |  | Shut In 2 (Final). Total Flow Time (T) = 59.25 + 30 = 89.25 minutesb. Time (ΔT); (T+ΔT)/ΔT; Pressure (P); Temperature; Pressure squared (P2). (min); (psig); (deg F); (psig2). 0.0; ---; 435.617401; 115.143501; 189,764.78. 4.0; 23.31; 825.081787; 119.8134; 680,757.01. 8.0; 12.16; 830.118713; 121.163101; 689,099.21. 12.0; 8.44; 833.033508; 121.227501; 693,938.98. 16.0; 6.68; 835.235901; 121.179703; 697,625.86. 20.0; 5.46; 837.006775; 121.144402; 700,585.74. 24.0; 4.72; 838.476685; 121.177597; 703,048.71. 28.0; 4.19; 839.725891; 121.214996; 705,146.47. 32.0; 3.79; 840.914917; 121.298103; 707,129.63. 36.0; 3.48; 841.682373; 121.412399; 708,425.22. 40.0; 3.23; 842.520203; 121.562202; 709,839.95. 44.0; 3.03; 843.182373; 121.730797; 710,952.51. 48.0; 2.86; 844.06543; 121.915604; 712,454.16. 52.0; 2.72; 844.552002; 122.107803; 713,264.70. 56.0; 2.59; 844.938416; 122.306; 713,923.60. 60.0; 2.49; 845.54541; 122.5158; 714,954.80. 64.0; 2.39; 846.011902; 122.712097; 715,732.92. 68.0; 2.31; 846.609009; 122.936401; 716,748.49. 72.0; 2.24; 846.955078; 123.124702; 717,341.24. 76.0; 2.17; 847.331421; 123.330902; 717,968.13. 80.0; 2.12; 847.858093; 123.534203; 718,866.58. 84.0; 2.06; 848.078918; 123.712997; 719,239.69. 88.0; 2.01; 848.460205; 123.904503; 719,884.37. 92.0; 1.97; 848.756104; 124.090897; 720,393.54. 96.0; 1.93; 848.997009; 124.264297; 720,801.00. 100.0; 1.89; 849.177612; 124.4431; 721,106.67. 104.0; 1.86; 849.583923; 124.618301; 721,786.18. 108.0; 1.83; 849.724426; 124.773598; 722,024.08. 112.0; 1.80; 850.115723; 124.930603; 722,704.01. 116.0; 1.77; 850.256226; 125.089798; 722,942.07. 120.0; 1.74; 850.521973; 125.233299; 723,384.27. 124.0; 1.72; 850.436829; 125.351997; 723,231.18. |
| --- | --- | --- |

From the Pressure Summary Table

(a) Shut-in 1 Time - Open to Flow 1 Time {38.5 - 8.5 = 30.0 minutes} 
(b) Shut-in 2 Time - Open to Flow 2 Time + Total Flow Time from (a) {159.0 - 99.75 + 30.0 = 89.25 minutes}

In the table note that the Shut-In 1 & 2 periods were broken down into 4 minute intervals.

The Pressure Summary Table gives a summary of the DST Pressure-Temperature-Time Readings at specific points. This DST had a 30.0 minute pre-flow and a 59.25 minute main flow. Thus 'T' for the Shut-In 1 (Initial) period = 30.0 minutes since there was a total of 30 minutes flow time prior to Shut In 1 (Initial). T for the Shut-In 2 (Final) period becomes 89.25 minutes 30 minutes pre-flow + 59.25 minutes main-flow since T is the cumulative flow time prior to shut in.

The quantity (T + ΔT)/ΔT is mathematically the same as 1 + (T/ΔT) thus for the Shut-In 2 (Final) period example the (T + ΔT)/ΔT for the first increment = (89.25+4)/4 = 23.31, which is the same as 1 + (89.25/4) = 1 + 22.31 = 23.31.

(3) The Horner plot is then constructed by plotting log10[(T+ΔT)/ΔT] vs P2 for each point.

(4) In Figure 2 both Shut-In Pressure-Time curves have been plotted. In order to determine the static reservoir pressure the linear portion is extrapolated to the left hand margin where the value of the time function (T + ΔT)/ΔT = 1. Since the logarithm of 1 is zero the log10[(T + ΔT)/ΔT ] = 0 (or infinity) at this point. Thus we are extrapolating the Horner straight line to a 'infinite' shut in time where completely static pressure prevails. In the example here the static pressure squared ~ 738,000.0 psig2 or 859.0 psig. Both Shut In intervals extrapolations yield this value. In cases where the Shut-In 2 (Final) extrapolation is substantially less than the Shut-In 1 (Initial) depletion is indicated.  If the depletion factor 100.0 * Shut-In [(Initial)-(Final))/(Initial) ] is greater than 4%, then the indications of depletion can be considered serious or substantial.

(5) The final piece of information which is obtained from the Horner plot is the "slope" (m) of the straight line segment. Numerically this is the difference between the pressure at log10[(T + ΔT)/ΔT ]=0 and at log10[(T + ΔT)/ΔT ]=1, where the slope is simply the change in pressure over one complete log cycle. The slope in this example using the Shut In (Final) slope m=0.061 psi2/cycle. The slope is used in the calculations to derive permeability and other reservoir parameters.

### Transmissibility (K*h/μ), Permeability Thickness (K*h), Permeability (K)

The pressure build-up equation applicable to gas wells is expressed by

Po2 = Pf2 – mg * log10[(T+T)/T]

The build-up plot of Po2 vs. log10[(T+ΔT)/ΔT] is constructed and extrapolated to                      log10[ (T+ΔT)/ΔT ] to zero to obtain Pf2 and mg, the slope of the line.

mg = 1632 * qg * μ * Z * Tf / (K * h) [psi2] / [log cycle] 
qg = flow rate [Mcf] / [day]
μ = viscosity [cp]
Z = Gas Deviation Factor ( Compressibility factor)
Tf = Formation Temperature [oR]; oR = oF + 459.67
h = vertical thickness of continuous porosity [ft]
K = average effective permeability [md]

This states that the slope of the buildup is representative of a given fluid having physical properties μ, Z, Tf flowing at a rate qg through a formation having physical properties K * h.

Note: the flow rate (q) is computed by using an average of the final gas rates in the gas rates table.

Transmissibility can be found my rearranging the slope as follows,

| Transmissibility: | K * h / μ = 162.6 * q * B / m | [md]-[ft]/[cp] |
| --- | --- | --- |
| Permeability Thickness: | K * h = ( Transmissibility ) * μ | [md]-[ft] |
| Permeability: | K = ( Permeability Thickness ) / h | [md] |

### Damage Ratio (DR), Skin factor (S), and related quantities

Damage Ratio (DR) - A numerical value used to predict the flow rate if the damage were removed. In essence the DR becomes a number which when multiplied by the flow rate which existed at the time of the test will yield the flow rate if the damage (or improvement) over the natural condition is removed. This flow rate will be the rate at the same flowing pressure drawdown which existed during the test. From the definition of DR if a zone is neither damaged nor improved then q theoretical is identical to q actual and DR = 1.

where,

| Pi | = Initial static reservoir pressure | [psig] |
| --- | --- | --- |
| Pwf | = Bottom Hole Flowing Pressure (FFP) | [psig] |
| mg | = slope from the Horner plot | [psi] / [log cycle] |
| K | = Permeability ( K ) | [md] |
| To | = Flow Time | [min] |
| φ | = Porosity |  |
| μ | = viscosity | [cp] |
| c | = compressibility | [vol]/[vol]/[psi] |
| rw | = well bore radius | [ft] |

If K, φ, μ, c and rw are not known then the Damage Ratio can be estimated by,

Skin factor (S) - A numerical quantity which attempts to describe the amount of damage or improvement. The numerical value of skin is so defined that a positive value for the skin factor indicates a damaged condition and a negative skin factor indicates improvement over the natural condition. The skin factor is zero when neither skin damage nor improvement are present i.e. no alterations to natural condition.

| Pi | = Initial static reservoir pressure | [psig] |
| --- | --- | --- |
| Pwf | = Bottom Hole Flowing Pressure (FFP) | [psig] |
| mg | = slope from the Horner plot | [psi] / [log cycle] |
| K | = Permeability ( K ) | [md] |
| To | = Flow Time | [min] |
| φ | = Porosity |  |
| μ | = viscosity | [cp] |
| c | = compressibility | [vol]/[vol]/[psi] |
| rw | = well bore radius | [ft] |

If φ, μ, c and rw are not known then the Skin factor (S) can be estimated by,

Pressure Drop Across a Damage Zone ΔPskin The additional pressure drop during flow caused by the presence of skin (ΔPskin) may be calculated as follows,

Pskin = Pwf * Pwf2 + 0.87 * S * m)

where,

| S | = Skin factor |  |
| --- | --- | --- |
| Pwf | = Bottom Hole Flowing Pressure (FFP) | [psig] |
| m | = slope from the Horner plot | [psi] / [log cycle] |

Radius of Investigation (RI) The radius of investigation is the radial distance from the well bore which the pressure disturbance from the DST has investigated (in other words "how far out into the zone the DST has looked").

RI = 2.6408 * K*To / (c))

where,

| Pi | = Initial static reservoir pressure | [psig] |
| --- | --- | --- |
| K | = Permeability ( K ) | [md] |
| To | = Flow Time | [min] |
| φ | = Porosity |  |
| μ | = viscosity | [cp] |
| c | = compressibility | [vol]/[vol]/[psi] |

If φ, μ, and c are not known then the Radius of Investigation (RI) is as follows,

For Gas: RI = 2.6408 * K*Tf*Pi )

Effects of Turbulence(2) The skin factor and damage ratio values computed from the preceding equations do not take account of the additional pressure drop which is caused by turbulence in gas flow. The turbulence factor is not easy to determine precisely in DST's, however, an approximate empirical relationship between turbulence and permeability has been determined.

Step 1: The flowing pressure corrected for turbulence (Pwfc) is computed as follows,

|  | Pwf G | = Bottom Hole Flowing Pressure (FFP) = Gas Specific Gravity (air = 1) | [psig] |
| --- | --- | --- | --- |
|  | Tf | = Formation Temperature | [oR]; oR = oF + 459.67 |
|  | Z | = Gas Deviation Factor ( Compressibility factor) |  |
|  | q | = Flow Rate - | [ Mcf ] / [ Day ] |
|  | K | = Permeability | [md] |
|  | h | = thickness | [ft] |
|  | rw | = well bore radius | [ft] |
|  | RI | = Radius of Investigation | [ft] |

Step 2: The value of (Pwfc) is then substituted for (Pwf) in the skin factor (S) and damage ratio (DR) equations to determine the true skin factor (S) and damage ratio (DR) to exclude the effects of turbulence.

Step 3: The pressure drop due to turbulence (ΔPt) may also be of interest and may be calculated simply as,

ΔPt = - Pwf + √( Pwf2 + 0.87 * mg * S )

Absolute Open Flow Capacity (qAOF) A theorectical quantity which describes the maximum flow rate of a gas zone at zero sandface pressure. It is an unattainable rate in practice but from it may be derived the flow rates at any practical flowing pressure.

Houpert equation: Pi2 - Pwf2 = a*q + b*q*q, where

Pwf = 0 by definition

| Pi | = Initial static reservoir pressure | [psig] |
| --- | --- | --- |
| Pwf | = Bottom Hole Flowing Pressure (FFP) | [psig] |
| K | = Permeability ( K ) | [md] |
| h | = vertical thickness of continuous porosity | [ft] |
| μ | = viscosity | [cp] |
| Z | = Gas Deviation Factor ( Compressibility factor) |  |
| Tf | = Formation Temperature | [oR]; oR = oF + 459.67 |
| G | = Gas Specific Gravity (air = 1) |  |
| re | = drainage radius | [ft] |
| rw | = well bore radius | [ft] |

If b = 0 then qAOF = Pi2/ a

References: 
(1) Pressure Build-Up in Wells, by D.R. Horner, 3rd World Petroleum Congress, May 28 - June 6, 1951 , The Hague, the Netherlands http://www.onepetro.org/mslib/servlet/onepetropreview?id=WPC-4135&soc=WPC 
(2) Drill Stem Test Interpretation Seminar, Author & Instructor: H. W. Reid 
(3) Recent Developments in the Interpretation and Application of DST Data, by L. F. Maier Journal of Petroleum Technology Volume 14, Number 11, November 1962, pgs 1213-1222 http://www.onepetro.org/mslib/servlet/onepetropreview?id=00000290

Save DST Test Data in Log ASCII Standard (LAS) version 3.0 File

The LAS (Log ASCII Standard) is rapidly becoming the accepted industry standard for electronic transmission of digital wire-line logs. Earlier digital formats were commonly coded in binary (such as LIS) and so required specialized software to read them. The LAS standard was introduced by the Canadian Well Logging Society (http://www.cwls.org/) in 1989 to standardize the organization of digital log curve information for personal computer users. It did this very successfully and the standard became popular worldwide. Version 1.2 was the first version and was followed in September 1992 by version 2.0 to address some inconsistencies. A more versatile version LAS 3.0 was released in 1999 however at present LAS 2.0 remains the dominant product. LAS 3.0 clarify several of the poorly defined specifications of LAS 2.0 and provide expanded data storage capabilities, but have seen limited implementation.

The GEMINI Tools programs will read either a Log ASCII Standard (LAS) version 2.0 or 3.0 file and version 1.2 but the Well Information Section is backward in data definition and will not be parsed correctly in the GEMINI Tools web apps.

The sections defined for the LAS 2.0 standard are as follows (http://www.cwls.org/wp-content/uploads/2014/09/LAS_20_Update_Jan2014.pdf):

- "~V" (also known as "~VERSION INFORMATION SECTION") is a required section; has formatting requirements; must be the first section; identifies the version number and whether data is in "wrapped" or "un-wrapped" mode.

- "~W" (also known as "~WELL INFORMATION SECTION") is a required section; has formatting requirements; is preferably the second section; contains information on the well name, location, and start and stop values of the data in this file.

- "~C" (also known as ~CURVE INFORMATION SECTION") is a required section; has formatting requirements; contains curve mnemonics and their definitions in the order that they appear in the data section.

- "~P" (also known as ~PARAMETER INFORMATION SECTION") is an optional section; has formatting requirements; contains information on parameters or constants relevant to the wellbore such as mud resistivity, wire line engineer, truck number, elevation data, etc.

- "~O" (also known as "~OTHER") is an optional section; has no formatting requirements; contains other information or comments.

- "~A" (also known as ~ASCII LOG DATA") is a required section; has formatting requirements; is the last section in the file and also referred to as the data section. The index of the data columns is either Depth or Time. The index values always appear in the first column and each column of data must be separated by at least one space (ASCII 32). All values in the ASCII log data section must be floating point or integer (long) values. Other formats such as Text or Exponential values are not supported.

LAS 3.0 (http://www.cwls.org/wp-content/uploads/2014/09/LAS_3_File_Structure.pdf) will be used to save the well data for the GEMINI Tools web apps since it can hold all the well data in one file. You can even think of LAS 2.0 as a subset of LAS 3.0 since the LAS 2.0 is only concerned with the LOG Data. Note this section includes some of the referenced LAS 3 File Structure PDF; see the above URL for the complete LAS 3.0 File structure.

The ~Version and ~Well sections must appear in every LAS 3.0 file as the first and second sections respectively.

Other sections are grouped by data type. Each group consists of two or three sections; a Parameter Data section (optional for all but Log data), a Column Definition section, and a Column Data section, in that order.

For example, core analysis data would have the following three sections:

~Core_Parameter
~Core_Definition
~Core_Data.

At least one group or data type of either the defined LAS 3.0 data types or a user defined type must exist in every LAS 3.0 file.

The Column Definition and the Column Data sections for each data type are matched sets and must both appear in that order. The corresponding Parameter Data section is optional (except for Log data), but if used must appear before the corresponding Column Definition Section.

LAS 3.0 defines six specific well related data types and their root Section Title names. They are:

~Ascii or ~Log
~Core
~Inclinometry
~Drilling
~Tops
~Test

Additional data types can be defined by the user and content rules discussed elsewhere in the document may define other section titles.

Stand alone user defined Parameter Data sections can be included. Care must be taken to use standalone Parameter Data sections only when the data contained does not fit into any of the other defined data types.

When used, the section order of each set of the three sections for each data type must be Parameter, Definition, and then Data.

Blank lines and comment lines can appear within Column Data sections, but can only appear BEFORE the first Column Data line of that section, or after the LAST Column Data line of that section.

The names of each channel can optionally appear above each channel as a comment line immediately before, after or on the section title line of that section if space allows.

Note: Do not use the ~Other section recognized by LAS version 2.0. It is no longer allowed in LAS 3.0. Any data that can be stored in this section must now be stored properly in a user defined Parameter Data or Column Data section.

### The LAS version 3.0 file has the potential to hold all the well data that was collected, i.e. multiple log data files, core data, tops data, DST data, Perforation data, Cuttings Report data, etc.  As an example the Wellington KGS 1-32 has log, core, tops, brine data, geologist cuttings/core descriptions as well as DST Test data created from the DST web app.  You can view the file at the following URL addresses,

### LAS 3.0 Example File:  http://www.kgs.ku.edu/Gemini/Tools/documentation/Wellington_KGS_1-32-DST.las

### LAS 3.0 in WinZip File :  http://www.kgs.ku.edu/Gemini/Tools/documentation/Wellington_KGS_1-32-DST.zip

### Standard LAS 3.0 Data Sections for the Wellington_KGS_1-32-DST.las

- ~Version - Version Section

- ~Well - Well Header Information Section

- ~Log - Log Data Section – there are multiple log data (2) sections.

  - ~Log_Parameter

  - ~ Log_Definition

  - ~ Log_Data | Log_Definition, which must be the last section in the LAS File if the Log data is present.

- ~Tops - Formation Top (Stratigraphic Units) Picks Data Section

  - ~Tops_Parameter

  - ~Tops_Definition

  - ~Tops_Data | Tops_Definition

- ~Core – Measured Core Data Section

  - ~Core_Parameter

  - ~Core_Definition

  - ~Perforation_Data | Perforation_Definition

- ~Test – DST Data Section – there are 4 DST Test Data Sections in this file.

  - ~Test_Parameter

  - ~Test_Definition

  - ~Test_Data | Test_Definition

### GEMINI Tools Defined LAS 3.0 Data Sections for the Wellington_KGS_1-32-DST.las

- ~IQ_Control - Recreate the Profile Plot Data Section

  - ~IQ_Control_Parameter

  - ~IQ_Control_Definition

  - ~IQ_Control_Data | IQ_Control_Definition

- ~IQ_Brine – This data holds the brine data.

  - ~ IQ_Brine _Parameter

  - ~ IQ_Brine _Definition

  - ~ IQ_Brine_Data | IQ_Brine_Definition

- ~IQ_Geo_Report – This data holds the geologist cuttings report/core description.

  - ~ IQ_Geo_Report_Parameter

  - ~ IQ_Geo_Report_Definition

  - ~ IQ_Geo_Report_Data | IQ_Geo_Report_Definition

- ~IQ_Las_Parameter - Selected LAS Curves – This Parameter Section was designed to remember the Log Curves selected by the user so the user does not have to map LAS File Curve Mnemonics to KGS Standard Tool Mnemonics when they run this file with other GEMINI Tools.

Select the Floppy Disk Imageto Open the “LAS File Data Types” Dialog.  This dialog allows the user to make changes or additions to the data before it is saved to a Log ASCII Standard (LAS) version 3.0 File.

The Tabs at the top identifies the data that will be saved to the Log ASCII Standard (LAS) version 3.0 File.  This example saves the well info, log data, core data, brine data, tops data, DST Test data, profile plot control data, and cuttings/core description. The “LAS File Data Types” Dialog allows the user to modify the well header data that is being saved to the LAS version 3.0 file.  Also noted in the two images are identified “required” fields for a valid LAS file.  The GEMINI Tools Web Apps doesn’t care that the LAS file is not valid, just that the data follows the basic rules for the well data sections in the LAS 3.0 file.

Example: Wellington KGS 1-32 - Previous Session

Importing PC Data - Download Well Data to PC – Previous Session

This section loads Drill Stem Test Data saved in a Log ASCII Standard (LAS) version 3.0 file. Download either the ASCII Text Files directly or the Zip files extracting the contents into a directory.  The problem with the ASCII Text Files being downloaded directly from a web page is that the web page will alter the contents so it does not retain the basic structure and add HTML text to the file.  The preferred method if you have Zip or WinZip is to download the zip files to your PC and extract.

Well Data: Wellington KGS 1-32, Sumner County, Kansas

### LAS 3.0 Example File:  http://www.kgs.ku.edu/Gemini/Tools/documentation/Wellington_KGS_1-32-DST.las

### LAS 3.0 in WinZip File :  http://www.kgs.ku.edu/Gemini/Tools/documentation/Wellington_KGS_1-32-DST.zip

This file contains, Log Data, Tops Data, Core Data, Brine Data,  4 Drill Stem Tests (DST) and the Geologist Cuttings/Core Description.  This program will only load the Well Header Information, i.e. the “~Well” Data Section and the “~Parameters” Data Section and the 4 DST Test Data Sections.

A Note, the user can load either a LAS version 2.0 or version 3.0 file with or without DST Test Data and when the user saves the DST Session they will be able to create a LAS version 3.0 file with the original LAS file data plus the DST Test Section Data.

Left Click on the “Prev Session LAS File” icon image button to display the “Select LAS File from your PC” Dialog.  This dialog allows the user to search their PC for the file of interest.

In this example it is the LAS version 3.0 file Wellington-KGS-1-32-DST.las, highlighted below. Select the Open button to display the “LAS File Curve Sections” Dialog.

Notice that some of the check boxes are colored with different colors, which shows that the curves were automatically selected, but also to represent the curve type by color.The Curves are colored by type (data units) as follows,

- Orange -  OHM-M or Resistivity Logs

- Cyan – PU or porosity Logs, Neutron Porosity, Density Porosity, etc.

- Greenish yellow – BARNS/E or Photoelectric Factor Logs

- Green – GM/CC or Bulk Density Log

- Forest Green – USEC/FT or the Acoustic Transit Time Log

- Red – API, PPM or “%” as Radioactive logs, Gamma Ray, Spectral Gamma Ray, etc.

- Blue – MD or Permeability Logs

- Brown – F, FT or IN or Depth

- Middle yellow – FRAC, or other log curve types.

- Dark Violet – UNI or Unknown Linear Curves

- Medium Violet – UNL or Unknown Logrithum Curves

The color coding of the selected curves were added to also help the user visually recognize that a curve was selected or not.

Select the Continue Button to read and parse the DST Test Data selected into the DST Web App.