Gamma Ray Cross Section Java Applet                                               by John R. Victorine

Introduction

The Gamma Ray Cross Section web app has 2 sources for importing well data, 1) the user’s PC or 2) the Kansas Geological Survey (KGS) Server & ORACLE Database.  This program allows the user to import Log and Tops Data.

There will be occasions when the user would like to inspect the contents of a LAS file. So, for example, the user may wish to find the identity of the well, the types of logs, depth range, well or log parameters. Alternatively, the user can suspect that the file is not coded correctly in LAS format if the Import LAS function fails, and wishes to examine the file to troubleshoot the problem. The LAS file is an ASCII file and can be read by any text editor, i.e. Notepad, WordPad, TextPad, etc. The CSV (Comma Separated Values) files are also ASCII and can be edited in the same way or with Microsoft Excel.

To access Gamma Ray Cross Section go to http://www.kgs.ku.edu/stratigraphic/GR_CROSS_SECTION/.  At the top of the web page there is a menu "Main Page|Description|Applet|Help|Copyright & Disclaimer|".  Select the "Applet" menu option a "Warning - Security" Dialog will appear. The program has to be able to read and write to the user’s PC and access the Kansas Geological Survey (KGS) Database and File Server, ORACLE requires this dialog. The program does not save your files to KGS, but allows you to access the KGS for well information that may be missing in your Kansas logs. The program does not use Cookies or any hidden software it only reads the LAS and CSV files for the Gamma Ray Cross Section Session. The blue shield on the warning dialog is a symbol that the Java web app is created by a trusted source, which is the University of Kansas.  Select the "Run" Button, which will show the Cross Section Panel,

# Contents

Cross Section Panel ………………………………………………..…………………………………………………………………………….. 4

Quick Cross Section Work Flow ……………………………..…………………………………………………………………………….. 5

Loading Well Data   …………………………..………………………………………………………………………………………………….. 6 Data Source Panel	………………………………………………………………………………………………………………………... 6 Data Loaded Panel ………………………………………………………………………………………………………………………... 7

Importing KGS (Database & Server) Data …….…………………………………………………………………………………….… 8 Importing Well Data ……………………………………………………………………………………………………………………... 8   * LAS File Data …………………………………………………..………….…………………………………………………………….  11 - Map Curves & Change Curve Selections …………………………..…….……………………….………………………... 13 * Tops Picks ………………………………………………………………………………….….………………………………………….. 18 - MKD Source Example ………………………………………………………………………………………………………………… 19

Importing PC Data – Download Well Data from PC …….……………………………………………………………………… 24 Log ASCII Standard (LAS) version 2.0 File …………………………………………………………………………………….. 24 - Map Curves & Change Curve Selections …………………..………………………………………………………….……. 26   Tops CSV (Comma Separated Values) File …………………………………………………………………………….……… 36      - Tops CSV (Comma Separated Values) File Structure ………………………………………………………………….. 37

Examples: Importing Well Data into Cross Section ………………………………………………….………………….……… 40 Import Well Data from the Kansas Geological Survey’s Database and File Server ………………………… 40 - Newby 2-28R (15-189-22225) ……………………………………………………………………………………………………. 41 - Lightcap 1-28 ( 15-189-22306) …………………………………………………………………………………………………… 45 - Newby 3-28 (15-189-21240 ……………………………………………………………………………………………………….. 49 Importing Well Data from User’s PC …………………………………………………………………………………...………. 55 - Wellington KGS 1-32 ………………………………………………………………………………………………………………….. 56 -- Import Log ASCII Standard (LAS) version 2.0 File ………………………………………………………………………. 56 -- Import Tops Comma Separated Values (CSV) ASCII Delimited File …………..……………………………….. 58  - Wellington KGS 1-28 ………………………………………………………………………………………………………………….. 66 -- Importing Log ASCII Standard (LAS) version 3.0 File …………………………………………………………………. 66 - Meridith 3 (15-191-21556) ………………………………………………………………………………………………………… 69 Importing Log ASCII Standard (LAS) version 3.0 File …………………………………………………………………….. 69 Import the XML (Extensible Markup Language) file from User’s PC …………………………………………….. 74

Exporting Gamma Ray Cross Section Well Data Information …………..……..………………………………….……… 79

Cross Section Panel – Well Location Image Map …………………………………………………………….……….….……… 81

Cross Section Panel – Change the Order of the Selected Wells ……………….………………………………….……… 82

Cross Section Plot Control Dialog …………………………………………………………….………………………………….……… 94

Adding & Modifying Tops …………………………………….………...………………………………………….………………………. 87  Enter Horizon Data - Stratigraphic Units Panel ………………………………………………………………………….…. 87      - Add Shawnee Group to the Stratigraphic Units List ……………………..………………………………….……..…. 89     - Add Unknown Bed (SG-A Bed) to the Stratigraphic Units List …………………………………………………….. 92

Cross Section Panel

The Cross Section Panel manages the basic well data for the cross section plot.  The user can import multiple well data (log, tops and core/cuttings descriptions) or measured sections to plot in a multiple well profile plot.  This program does not necessarily need the X-Y-Z position of the well to plot, but the “Map” button does if the user wishes to plot the location of the well in 2 dimensions and the “Cross Section” button does if the user wishes to plot the wells by elevation.

Figure: Description of the Cross Section Panel, the main entry point for entering multiple well data.

The cross section requires the user to manage the memory in building the cross section plot, which is displayed on the main panel; the panel has the Memory Usage as well as the Total Available Memory Usage Status Bar Fields. For each cross section the user must decide how many wells and their respective depth ranges will fit in one plot. The memory size is dependent on the user's own PC. This applet will allow from 1 to 100 wells on one cross section plot. This web app can support 100 wells because it requires only one log curve, Gamma Ray, unlike the Cross Section web app that plots all available log curves which requires a lot of extra memory to hold the data.

Quick Cross Section Work Flow

- Select the “New Well” button to load data for each well into the cross section panel.

  - Maximum of 100 wells allowed for plotting a cross section.

  - Minimum of 1 well can be plotted in the cross section plot.

  - For each well loaded,

    - Verify that the well has a latitude, longitude and elevation.

    - If any latitude, longitude and elevation is missing then,

      - Highlight the well/measured section in the list.

      - Select the “Modify” button to enter the missing data, either manually or by using the “Search KGS Database for Well Information” button to search the Kansas Geological Survey (KGS) database for existing well header information.

- For more 2 wells select the “Map” button to plot the wells on latitude versus longitude map plot to verify the order is correct.

  - If a well is out of order then,

    - Highlight the well in question.

    - Select the “Up” or “Down” buttons to change the order of the well, verify with the latitude versus longitude map plot.

- Select the “Cross Section” Button to create a Cross Section Plot.

Loading Well Data

Click the "New Well” Button, which will show the "Load Data" Dialog. The dialog below displays an example of the Wellington KGS 1-32 well data loaded from the PC Data icon buttons with the data in the tables above.  The icon buttons in the Data Source Panel assists the user in loading well data into the Gamma Ray Cross Section Applet.

Data Source Panel

The Data Source Panel provides two methods of importing data into the Gamma Ray Cross Section Web App.  The Kansas Geological Survey (KGS) Database & File Server and the user’s PC.  A number of icon buttons are provided to assist the user in importing the specific data type of interest. When the user selects the icon button a search dialog is provided specific to the data type.  The CSV (Comma Separated Values) icon buttons under the “PC ASCII Delimited Data Files” Panel are expecting a general type of data presentation.  Although the order of the specific data columns is not important, the “Mnemonics” of the data column is.  Each data type in GEMINI Tools web apps have a data mnemonic list that will be presented later as each icon search dialog is presented. The CSV Search Dialog will use the first two lines of the CSV file to automatically match the file column data mnemonics with the web app curve mnemonics, but if the program does not recognize the file data mnemonic then it will leave it blank and expect the user to match the file data mnemonic to the web app curve mnemonics, this will be explained later.

Data Loaded Panel

The Data Loaded Panel provides a visual feedback that the data type was loaded, by the file names of the files that were accessed to load the data and by the data type that is loaded.  The data type is important in that it shows where the data came from.  The KGS data has the ORACLE Database from which the Tops Data, and Core Data, etc. are retrieved from a XML (Extensible Markup Language) data stream that is constructed using the ORACLE PL/SQL for each data type.  The user’s PC will load the data from CSV (Comma Delimited Values) file.  The LAS File can be downloaded automatically from the KGS Server in the program or from the user’s PC.  This program allows the user to import up to 3 Log ASCII Standard (LAS) version 2.0 or 3.0 files.  The LAS version 3.0 file can hold all the well data, but if the user wishes to add log curves from a LAS version 2.0 file it is advised that the LAS version 3.0 file be loaded last.  In most cases the user is importing multiple LAS version 2.0 files, which can be saved into a single LAS version 3.0 file.

Importing KGS (Database & Server) Data

KGS (Database & Server) - Importing Well Data

The Kansas Geological Survey (KGS) has a good collection of well data stored in the ORACLE Database and File Server as Files Log ASCII Standard (LAS) version 3.0 Files.  In this example the user will download the well data available from the KGS, Log data (LAS version 2.0 File), Tops Data, Measured Core Data, and Perforation Data.  The ORACLE Database is accessed by making Stored Procedure PL/SQL calls to the ORACLE Database from which an Extensible Markup Language (XML) data stream is created containing the well data that is passed back to the web app making the request.

Left Click on the “Well Data” Icon Button in the Data Source Panel of the Load Data Dialog.

This will display the “Search for Data on KGS Server” Dialog, see above image.  This dialog allows the user to search the KGS database for well data.  In this example, the well of interest will be the Newby 2-28R, this well contains all the well data that can be retrieved from the KGS Database, i.e. Log Data (LAS version 3.0 File), Tops Data, Core Data, and Perforations.

As the Summary image suggests there are 3 methods for searching for the well data within this dialog,

- By API-Number – KGS has a specific format for the API-Number, i.e.SS-CCC-99999 where SS is the state code for Kansas 15, CCC is the county code for Newby 2-28R it is 189 for Stevens County and the 5-Digit Well Number for Newby 2-28R it is 22225.

- By Partial Lease Name – The stored procedure used to retrieve the well header information allows the user to enter a partial phrase, in this example Newby.  The program places a ‘%’ in front and back of the phrase and sends the request to the Database, i.e. “%Newby%”.

- By Township Range Section – This search is by location in Kansas, this search also allows the user to enter just the Township and Range to search for wells, e.g. to look for the Newby 2-28R, enter Township as 31 set the S (South) Radio button and Range as 37 set the W (West) Radio button.

The user only needs to enter the above data and select the “Search” Button to display the list of Wells in the Kansas Database that match the search criteria.  In the image below the Lease Name “Newby” was entered to search for all wells in Kansas with the Phrase Newby in it. The user searches through the list until they find the well of interest.  In this example it is the Newby 2-28R, which is highlighted.

Notice that the LTCI represents the type of data that the well contains.  It is a visual aid that lets the user see what is available before trying to download the data.  If you require a LAS file you would want to see an L in that column.  The LTCI labels stand for the following,

- L – Log ASCII Standard (LAS) version 2.0 Files

- T – Tops Data ( Stratigraphic Unit Horizons )

- C – Measured Core Data

- I – Core Joint Photographic Experts Group (JPEG) Image Files

This dialog allows the user to now download each of the data types that are available.

Load KGS Well Data – LAS File Data

The “Search for Data on KGS Server” Dialog allows the user to download data from the KGS Database & Server to the web app.  The “LAS File Data” Button will display the “Select LAS File” Dialog with a list of LAS version 2.0 Files that are available.

In this example there are three LAS files available, with a table suggesting the log data type in the file.  In the beginning of the GEMINI Project (2000-2003) a precursor to the GEMINI Tools the KGS populated the Tool Types from every log that was in the KGS Server at that time.  Unfortunately KGS has not maintain that table for wells uploaded after 2003 so the LAS File may have “No” for all the log types, which is not accurate.  The user will need to open or download the file or search for the Well on the KGS Master List of Oil and Gas Wells in Kansas Web Page (http://www.kgs.ku.edu/Magellan/Qualified/index.html) to see what is in the File Header before deciding to download data from this program.  For this example the first log has most of the data necessary except the Spectral Gamma Ray Logs.  The Table above identifies the following log types,

- OHM-M – Resistivity Logs

- Neutron – Neutron Porosity Log

- Density – Bulk Density and/or Density Porosity Log

- Sonic – Acoustic Transit Time and/or Sonic Porosity Log

- GR – Gamma Ray (API units) Log

- PE – Photoelectric Factor Log

- THOR – Thorium Concentration

- URAN – Uranium Concentration

- POTA – Potassium Concentration)

In this example the first log contains the data needed, highlight the first log and click on the “Select” Button to display the “LAS File Curve Sections” Dialog. The “LAS File Curve Sections” Dialog allows the user to map unknown LAS Curve Mnemonics to the KGS “Standard” Curve Mnemonics so they will be plotted in the 3D Cross Plot.  This program reads the “LAS Tool Curve Mnemonics map to KGS Standard Mnemonics” XML File (http://www.kgs.ku.edu/software/gemini/data/las_standard_tools.xml ), which will automatically maps the Curve Mnemonics from the LAS file to one of 31 KGS “Standard“ Curve Mnemonics.

As you can see this log has all the log types of interest, Gamma Ray API, Resistivity, Neutron/Density,  Photoelectric Factor, Sonic and Permeability.  If a curve Mnemonic is not recognized the program will place a “?” in front of the Mnemonic, e.g. “?(MPERM16)” for the “. : Curve # 51” Log Curve.  If the user is satisfied with the automatic curve selections, which are checked and color coded, they only need to select the “Continue” Button at the bottom of the Dialog to import the file. The next section will take the user through a series of examples in changing the curve selections and mapping unknown curve mnemonics.

Notice that some of the check boxes are colored with different colors, which shows that the curves were automatically selected, but also to represent the curve type by color.The Curves are colored by type (data units) as follows,

- Orange -  OHM-M or Resistivity Logs

- Cyan – PU or porosity Logs, Neutron Porosity, Density Porosity, etc.

- Greenish yellow – BARNS/E or Photoelectric Factor Logs

- Green – GM/CC or Bulk Density Log

- Forest Green – USEC/FT or the Acoustic Transit Time Log

- Red – API, PPM or “%” as Radioactive logs, Gamma Ray, Spectral Gamma Ray, etc.

- Blue – MD or Permeability Logs

- Brown – F, FT or IN or Depth

- Middle yellow – FRAC, or other log curve types.

- Dark Violet – UNI or Unknown Linear Curves

- Medium Violet – UNL or Unknown Logrithum Curves

The color coding of the selected curves were added to also help the user visually recognize that a curve was selected or not.

Map Curves & Change Curve Selections

Some logs will have curve mnemonics that are not recognized as one of the KGS “Standard” Curve Mnemonics.  The user will need to map the log curve to one of the KGS standard curves if they want to display the curve.  Slide the scroll bar down to the Permeability Curves MPERM and ?(MPERM16).

Click on the “?(MPERM16)” Button to display the “Select KGS Standard Tools” Dialog.  This dialog provides a list of the KGS “Standard” Curve Mnemonics, from which the user can map an unrecognized log curve to one of the KGS standard curve mnemonics.  The KGS “Standard” Curve Mnemonics List was created as a way to standardize the alpha bit soup of Log Mnemonics.  Each logging company has their own curve mnemonics to represent similar tools.  The Gamma Ray Cross Section program is a later version of code from the GEMINI Project Cross Section Module, which needed to standardize the log curves so the curves could be automatically read and assigned a plot track.  The “LAS Tool Curve Mnemonics map to KGS Standard Mnemonics” XML File was created to map the log curves from logs that were part of the KGS LAS File Collection which is not a complete list of possible curve mnemonics.

To map the unknown curve mnemonic “?(MPERM16)” you first notice that it similar to the MPERM curve above, which is a permeability curve.  In this example both curves will be plotted together, but the Permeability Plot Track can only plot one curve.  The web app has the ability to allow the user to plot up to 4 unknown logrithum curves and 4 unknown linear curves.  The permeability is usually plotted as logrithmic.  Click on the “?(MPERM16)” Button to display the “Select KGS Standard Tools” Dialog.  Slide the scroll bar down to the “LOG_1” Mnemonic – Semilog Track Curve 1 and highlight that curve.  Click on the “Select” Button to map the “?(MPERM16) to the Semilog Track Curve 1.

The ?(MPERM16) Curve has been changed to MPERM16 removing the ?() around the Curve Mnemonic.  Also select the check box next to it, which changes to a dark violet.

In order to plot both Permeability Curves on the same track, the MPERM must be assigned to the unknown Log Curves with MPERM16.  Click on the MPERM Mnemonic Button to display the

“Select KGS Standard Tools” Dialog.  Slide the scroll bar down to the “LOG_2” Mnemonic – Semilog Track Curve 2 and highlight that curve.  Click on the “Select” Button to map the “MPERM to the Semilog Track Curve 2.

Click the check box next to the MPERM Mnemonic Button you will notice that it has the same color as MPERM16, which now groups the two curves together.  Select the “Continue” Button to load the LAS File into the web app.

Load KGS Well Data – Top Picks

The “Search for Data on KGS Server” Dialog allows the user to download data from the KGS Database & Server to the web app.  The “Top Picks” Button will display the “Move/Merge KGS Data” Dialog with available top picks grouped by the sources creating the tops.

This dialog allows the user to add all or some the tops from each of the sources.  Both tables are set up so the user can use the “Ctrl” Key with the left click of mouse to select multiple tops, i.e.

Notice that the only some of the tops are selected.  You can then select the Add Button to move only the selected tops to the “User’s Stratigraphic Units” Table.

MKD Source Example:

Once the list of tops are in the “User’s Stratigraphic Units” Table the user can edit the list by removing any duplicate or invalid tops.  Notice that the “Council Grove” Top occurs 2 times in the list.  Highlight the one of the “Council Grove” tops.

Now select the “Remove” Button.

The MKD-07/2006 has only one top, so this dialog allows the user to add that top to the “User’s Stratigraphic Units” Table.  The MKD does not have this top and this is an extra top missing from the MKD data set.

Now with the data set complete select the “Load Data” Button to import the Tops data into the web app.

As the user accepted each data type the “Data Source Filenames:” Panel in the Load Data Dialog changes. The LAS File that was downloaded from the KGS Server to the Gamma Ray Cross Section Web App is added to the “Log ASCII Standard (LAS) Files” panel will show the filename downloaded.  The Log Data, Perforations and Tops Data, have been downloaded from KGS.

Select the “Add Well Data” Button to transfer the Newby 2-28R Well Data to the Cross Section Data Structure.

Importing PC Data - Download Well Data to PC

Importing PC Data – Log ASCII Standard (LAS) version 2.0 File

Most of the web apps will use the same input dialogs to import Log ASCII Standard (LAS) version 2.0 or 3.0 files.  The Load Data Dialog is basically the same for most of the Web Apps, except they only load a subset of the total data types.  In this example a LAS version 2.0 file is being imported into the web app.

Left Click on the “LAS File” Icon Button in the Data Source Panel of the Load Data Dialog.  This will display the “Select LAS File from your PC” Dialog.  This dialog allows the user to search their PC for the file of interest.  In this example it is the LAS version 2.0 file Wellington-KGS-1-32.las, highlighted below. Select the Open button to display the “LAS File Curve Sections” Dialog.

The “LAS File Curve Sections” Dialog allows the user to map unknown LAS Curve Mnemonics to the KGS “Standard” Curve Mnemonics so they will be plotted in the 3D Cross Plot.  This program reads the “LAS Tool Curve Mnemonics map to KGS Standard Mnemonics” XML File (http://www.kgs.ku.edu/software/gemini/data/las_standard_tools.xml ), which will automatically maps the Curve Mnemonics from the LAS file to one of 31 KGS “Standard“ Curve Mnemonics.  If a curve Mnemonic is not recognized the program will place a “?” in front of the Mnemonic, e.g. “?(BSZ)” for the “.in : Bit Size” Log Curve.  If the user is satisfied with the automatic curve selections, which are checked and color coded, they only need to select the “Continue” Button at the bottom of the Dialog to import the file. The next section will take the user through a series of examples in changing the curve selections and mapping unknown curve mnemonics.

Notice that some of the check boxes are colored with different colors, which shows that the curves were automatically selected, but also to represent the curve type by color.The Curves are colored by type (data units) as follows,

- Orange -  OHM-M or Resistivity Logs

- Cyan – PU or porosity Logs, Neutron Porosity, Density Porosity, etc.

- Greenish yellow – BARNS/E or Photoelectric Factor Logs

- Green – GM/CC or Bulk Density Log

- Forest Green – USEC/FT or the Acoustic Transit Time Log

- Red – API, PPM or “%” as Radioactive logs, Gamma Ray, Spectral Gamma Ray, etc.

- Blue – MD or Permeability Logs

- Brown – F, FT or IN or Depth

- Middle yellow – FRAC, or other log curve types.

- Dark Violet – UNI or Unknown Linear Curves

- Medium Violet – UNL or Unknown Logrithum Curves

The color coding of the selected curves were added to also help the user visually recognize that a curve was selected or not.

Map Curves & Change Curve Selections

Some logs will have curve mnemonics that are not recognized as one of the KGS “Standard” Curve Mnemonics.  The user will need to map the log curve to one of the KGS standard curves if they want to display the curve.  The first example is to map the Acoustic Transit Time (DT),which is labeled as “.uspf : WAVE DTC” log curve in the LAS File. Also notice that the button label “?(DTC)” is not recognized by the Gamma Ray Cross Section web app.

Click on the “?(DTC)” Button to display the “Select KGS Standard Tools” Dialog.  This dialog provides a list of the KGS “Standard” Curve Mnemonics, from which the user can map an unrecognized log curve to one of the KGS standard curve mnemonics.  The KGS “Standard” Curve Mnemonics List was created as a way to standardize the alpha bit soup of Log Mnemonics.  Each logging company has their own curve mnemonics to represent similar tools.  The Gamma Ray Cross Section program is a later version of code from the GEMINI Project Cross Section Module, which needed to standardize the log curves so the curves could be automatically read and assigned a plot track.  The “LAS Tool Curve Mnemonics map to KGS Standard Mnemonics” XML File was created to map the log curves from logs that were part of the KGS LAS File Collection which is not a complete list of possible curve mnemonics.

To map the unknown curve mnemonic “?(DTC)” you first notice that the unit is “uspf” (micro seconds per foot) a unit of time.  Also the Acoustic Transit Time Curve Mnemonic is similar to the KGS “Standard” Curve Mnemonic “DT”.  By selecting the “?(DTC)” Button you will display the “Select KGS Standard Tools” Dialog.

Highlight the “DT” Mnemonic Row and click on the “Select” Button to map the “?(DTC)” Curve Mnemonic to “DT” KGS Standard Curve Mnemonic.

The “?(DTC). .uspf : WAVE DTC” has changed to “DT.usec/ft : Acoustic transit time”. We want to change the selected “ITTT” Log Curve to “DT” Log Curve.  The reason is that “ITTT” is the wrong curve type for the Acoustic Transit Time.  The program found the curve mnemonic as similar to the “DT” Standard Curve Mnemonic, but this curve will not plot correctly in the 3D Cross Plot.  Just click on the green check box in front of the “ITTT” Mnemonic Button to deselect the curve and then click on the check box in front of the “DT” Mnemonic Button to select it.  Also notice that the ~Log_Definition Text Area was modified to show the change.

Moving the scroll bar down to the porosity curves, Neutron Porosity, and Density Porosity.

The LAS File Read will select the first curve that it recognizes and selects and color codes the curve.  In this case the Neutron porosity mnemonic selected is “NPHS”, which is a valid curve, but the “NPHI” curve is desired so like the Acoustic Transit Time, you can deselect the “NPHS” and then select the “NPHI” Curve.  Also the “DPHS” Density Porosity Curve can be deselected since the “RHOB” Bulk Density Curve has been selected.  The reason for deselecting the Density Porosity Curve, if the Bulk Density Curve is present, is to force the Gamma Ray Cross Section program to recompute the Density Porosity using a Limestone Matrix.  If the Neutron Porosity, Bulk Density, Gamma Ray with/without a Photoelectric Factor Logs are present then the program will automatically compute a Lithology Composition Plot, but the Density Porosity has to be computed with a Limestone Matrix or the Lithology Composition Plot will not be computed corectly.

The above dialog represents the changes made for the neutron/density porosity logs. The last curves to be modified are the Array Induction Logs.  Haliburton uses a different curve mnemonic for these logs.  Move the scroll bar up to find the Array Induction Logs, RT90, RT60, etc.

Like the Acoustic Transit Time the “?(RT90).ohmm: 90in Resistivity 2ft Res” through “?(RT10).ohmm : 10in Resistivity 2ft Res” are not recognized.  These curves can be map to the “AHT90 Array Induction Resistivity-90” to “AHT10 Array Induction Resistivity-10” KGS Curves Respectively.  Click on the “?(RT90)” Mnemonic Button to display the “Select KGS Standard Tools” Dialog.

Highlight the AHT90 and click on the “Select” Button.

The “(?AHT90).ohmm : 90in Resistivity 2ft Res” has changed to “AHT90.OHM-M  : Array Induction Resistivity-90” and the orange check box is selected.  The rest of the Array Induction Log Curves each are mapped to the respective KGS Mnemonic Curve as follows,

(?RT90).ohmm : 90in Resistivity 2ft Res to AHT90.OHM-M  : Array Induction Resistivity-90 (?RT60).ohmm : 60in Resistivity 2ft Res to AHT60.OHM-M  : Array Induction Resistivity-60 (?RT30).ohmm : 30in Resistivity 2ft Res to AHT30.OHM-M  : Array Induction Resistivity-30 (?RT20).ohmm : 20in Resistivity 2ft Res to AHT20.OHM-M  : Array Induction Resistivity-20 (?RT10).ohmm : 10in Resistivity 2ft Res to AHT10.OHM-M  : Array Induction Resistivity-10

Select the Continue Button to read and parse the LAS log curves selected into the Gamma Ray Cross Section Web App. Notice that the “Data Source Filenames:” Panel lists the LAS version 2.0 File that was just read in as well as the type of data, i.e. Log Data from LAS Data Type.

Importing PC Data – Tops CSV (Comma Separated Values) File.

Most of the web apps will use the same input dialogs to import tops CSV (Comma Separated Values) file.  The Load Data Dialog is basically the same for most of the Web Apps except they only load a subset of the total data types.  In this example a Tops CSV file is being imported into the web app.

Left Click on the “Tops Data” Icon Button in the Data Source Panel of the Load Data Dialog.  This will display the “Select Formation Tops Comma Delimited File from your PC” Dialog.  This dialog allows the user to search their PC for the file of interest.  In this example it is the Tops CSV file Wellington-KGS-1-32_Tops.csv, highlighted below. Select the Open button to display the “Map File Column Number to Region Column” Dialog.

The “Map File Column Number to Region Column” Dialog allows the user to map the file columns number to the web app tops data structure.  In this example the file has the well information in line one of the Tops CSV File and line two of the Tops CSV File has the file data columns.  In this case the chosen file columns match the Tops Mnemonics for the tops data structure.  The File Column Number is automatically assigned to the Region Column Names. The user only needs to select the “Load Data” Button to parse the Tops Data into the web app.

Tops CSV (Comma Separated Values) File Structure.

The Wellington KGS 1-32 Tops CSV example has two introduction lines, the first line is the well header information and the second line is the actual column labels for the tops data, illustrated below,

Figure: Partial Contents of the Wellington-KGS-1-32_Tops.csv File.

The “Map File Column Number to Region Column” Dialog allows the user to map the data in the Tops CSV File to the web app data structure variables.  The program first reads the first and second line of the CSV File looking for the data column headers.    The lines are each parsed to single out the data column headers and to match those headers to the tops data structure.  The program then assigns the column number to the Region Column Name starting at column 1,2,3, … if the file column name used matches the expected region column name.  The Column Names matrix used to parse the file column variables are listed below,

| Depth Top | Top | Start |
| --- | --- | --- |
| Depth Base | Base | End |
| Stratigraphic Unit Rank [ SYSTEM, GROUP, etc. ] | Rank |  |
| Stratigraphic Name | Name |  |
| Alternate Name | Alt Name |  |
| Era |  |  |
| System | Sys |  |
| Subsystem | subsys |  |
| Series | Ser |  |
| Subseries { Pennsylvanian & Mississippian Series } | Subseries | Subser |
| Stage | Stg |  |
| Group | Grp |  |
| Subgroup | subgrp |  |
| Formation | Form |  |
| Start Age (Ma) | Start Age |  |
| End Age (Ma) | End Age |  |

The Wellington KGS 1-32 Tops CSV File example above line 2 has only the Top, Tops Name, Rank, System, Subsystem, Series and Source as the column name variables.  The program was able to map each of the column headers to the tops data structure, except Source, i.e.

| Column | File Column Label | Tops Data Name |
| --- | --- | --- |
| 1 | Top | Depth Top |
| 2 | Name | Stratigraphic Name |
| 3 | Rank | Stratigraphic Unit Rank |
| 4 | System | System |
| 5 | Subsystem | Subsystem |
| 6 | Series | Series |
| 7 | Source |  |

When the user selects the “Load Data” Button on the “Map File Column Number to Region Column” Dialog the data is parsed into the Gamma Ray Cross Section Program, where the Tops CSV file name is entered into the “PC ASCII Files:” Panel as well as the data type source.

Examples: Importing Well Data into Cross Section

The cross section was created to allow the user to plot multiple wells and plot versus depth.  The user can datum the logs by tops, elevation or log depth (default).  Like other GEMINI Tools web apps this program can load from two sources the user’s PC and the Kansas Geological Survey (KGS) database and file server.  This section will give 2 examples that will cover each of the import processes,

- Import well data from Kansas Geological Survey Database and File Server, which will search for 3 wells that are part of the same Township, Range and Section.

- Import well data from the user’s PC, this example will allow the user to import 3 wells from the user’s PC to load Log ASCII Standard (LAS) version 2.0 and 3.0 files as well as Comma Separated Values (CSV) tops and Geologist Cuttings/Core Description ASCII delimited file.

- Import the XML (Extensible Markup Language) file with a list of 56 wells with the location of Log ASCII Standard (LAS) version 2.0 file and tops for each well.  This file contains the location of LAS files on the KGS Server, the Gamma Ray Cross Section web app will automatically retrieve the LAS from the KGS Server and load the Gamma Ray log data.

The “New Well” button on the Cross Section Panel allows the user to import well/measured section data into the cross section web app. The user can import from data from each of the well sources or

Import Well Data from the Kansas Geological Survey’s Database and File Server

This example will focus on wells that are part of the same Township, Range and Section, i.e. Township 31 South, Range 37 West and Section 28.  The three wells for this example are as follows,

- Newby 2-28R (15-189-22225)

- Lightcap 1-28 (15-189-22306)

- Newby 3-28 (15-189-21240)

The user must load the well data for each well separately by clicking the “New Well” button for each well loading the Log & Tops Data with the “Load Data” dialog.  Clicking the “Add Well Data” button at the bottom of the “Load Data” dialog will transfer the well data to the Cross Section Panel.  The user can search for well data in the KGS Database and File Server using the “Search for Data on KGS Server” by API-Number, Partial Lease Name or by Township-Range Section.  To create this cross section, the Township, Range and Section (TRS) of Newby 2-28R was used to find other wells that appear within the same TRS location. The details of loading well data from the KGS Database and File Server can be found by going to the KGS (Database & Server) - Importing Well Data Section (page 7) of this document for loading the Newby 2-28R well data example.

- Newby 2-28R (15-189-22225)

Click on the “New Well” button on the Cross Section Panel to open the “Load Data” dialog. The user then clicks on the “Well Data” icon button to open the “Search for Data on KGS Server” dialog.

Click on the “LAS File Data” button to load the “Select LAS File” dialog.

Highlight the first LAS File, notice that with this well the KGS Database has defined the log curve types that are part of the LAS version 2.0 files, i.e. it has Resistivity Logs (OHM-M), Neutron Porosity (Neutron), Bulk Density (Density), Acoustic Transit Time (Sonic), Gamma Ray (GR), and Photoelectric Factor (PE) Logs. Now click on the “Select” Button to display the “LAS File Curve Sections” dialog, which lists all the data sections in the LAS File.  This example only has Log Data so only one panel will be displayed with a “~Log_Definition” tab.

Click on the “Continue” button to read and parse the LAS file into the cross section web app. The user will notice that on the “Load Data” dialog the LAS file name “1022012442.las” is entered into the “Log ASCII Standard (LAS) Files” panel. And the Log Data and Perforations data Type are marked with a “YES” under the KGS Column in the Data Type Panel.

The perforations data will automatically load if the data is present in the Perforations Database Table for the selected well.  The well header summary data is also automatically downloaded to the “Load Data” panel also.

Click on the “Top Picks” button to load the “Move/Merge KGS Data” dialog, which will display in the “KGS Stratigraphic Units” panel a list of Tops Sources, i.e., “HUG ELOG-EM”, “MKD” and “MKD-07/2006”.  Load the both MKD Top sources by clicking on the “MKD” tops source and then in the “Add to User’s Stratigraphic Units List” radio buttons, select the “Add to List” radio button. Click on the “Add All” button to copy the list of tops to the “User’s Stratigraphic Units” table. Then Click on the “MKD-07/2006” tops source radio button and then click on the “Add All” button to copy the top to the “User’s Stratigraphic Units” table.

The MKD-07/2006 has only one top pick, but the MKD in the source name implies that it is add on to the MKD top picks.

Click on the “Load Data” button to import the tops data in the “User’s Stratigraphic Units” table to the “Load Data” panel. Notice that in the “Data Type” Panel the Tops Data has a “YES” in the KGS Column.

Now select the “Close” button in the “Search for Data on KGS Server” dialog since all the data that can be imported for the Newby 2-28R has been imported. Now click on the “Add Well Data” button in the “Load Data” dialog to transfer the Newby 2-28R well data to the Cross Section Panel.

Notice that in the LTG column identifies the data type that was imported from the KGS database and file server, i.e. L=Log Data and T=Tops Data. Notice that the “Cross Section” Plot button, Modify Well Header Information Data “Modify” button and the “Remove”, “Remove All”, “Up” and “Down” buttons are now enabled, because there is a well in the “Cross Section Well/Location List” table.  The “Map” button will not enable until there is at least two wells in the table.

- Lightcap 1-28 (15-189-22306)

Click on the “New Well” button on the Cross Section Panel to open the “Load Data” dialog. The user then clicks on the “Well Data” icon button to open the “Search for Data on KGS Server” dialog.

Click on the “LAS File Data” button to load the “Select LAS File” dialog.

Highlight the LAS File, notice that with this well the KGS Database has defined the log curve types that are part of the LAS version 2.0 files, i.e. it has Resistivity Logs (OHM-M), Neutron Porosity (Neutron), Bulk Density (Density), Acoustic Transit Time (Sonic), Gamma Ray (GR), and Photoelectric Factor (PE) Logs. Now click on the “Select” Button to display the “LAS File Curve Sections” dialog, which lists all the data sections in the LAS File.  This example only has Log Data so only one panel will be displayed with a “~Log_Definition” tab.

Click on the “Continue” button to read and parse the LAS file into the cross section web app. The user will notice that on the “Load Data” dialog the LAS file name “1027928289.las” is entered into the “Log ASCII Standard (LAS) Files” panel. And the Log Data Type is marked with a “YES” under the KGS Column in the Data Type Panel.

The perforations data will automatically load if the data is present in the Perforations Database Table for the selected well.  The well header summary data is also automatically downloaded to the “Load Data” panel also.

Click on the “Top Picks” button to load the “Move/Merge KGS Data” dialog, which will display in the “KGS Stratigraphic Units” panel a list of Tops Sources, i.e., “MKD” and “SEALS”.  Load the both Top sources by clicking on the “MKD” tops source and then in the “Add to User’s Stratigraphic Units List” radio buttons, select the “Add to List” radio button. Click on the “Add All” button to copy the list of tops to the “User’s Stratigraphic Units” table. Then Click on the “SEALS” tops source radio button and then click on the “Add All” button to copy all the tops to the “User’s Stratigraphic Units” table.

The “SEALS” source has tops that are picked below the “MKD” Source tops picks and should be included to import all the possible top picks to the Lightcap 1-28 Well.

Click on the “Load Data” button to import the tops data in the “User’s Stratigraphic Units” table to the “Load Data” panel. Notice that in the “Data Type” Panel the Tops Data has a “YES” in the KGS Column.

Now select the “Close” button in the “Search for Data on KGS Server” dialog since all the data that can be imported for the Lightcap 1-28 has been imported. Now click on the “Add Well Data” button in the “Load Data” dialog to transfer the Lightcap 1-28 well data to the Cross Section Panel.

Notice that in the LTG column identifies the data type that was imported from the KGS database and file server, i.e. L=Log Data and T=Tops Data. At this point notice that the “Map” button is now enabled, which allows the user to display a map of the two wells plotted to give a sense of the location.

- Newby 3-28 (15-189-21240)

Click on the “New Well” button on the Cross Section Panel to open the “Load Data” dialog. The user then clicks on the “Well Data” icon button to open the “Search for Data on KGS Server” dialog.

Click on the “LAS File Data” button to load the “Select LAS File” dialog.

Highlight the LAS File, notice that with this well the KGS Database has defined the log curve types that are part of the LAS version 2.0 files, i.e. it has Resistivity Logs (OHM-M), Neutron Porosity (Neutron), Bulk Density (Density), Acoustic Transit Time (Sonic), and Gamma Ray (GR) Logs. Now click on the “Select” Button to display the “LAS File Curve Sections” dialog, which lists all the data sections in the LAS File.  This example only has Log Data so only one panel will be displayed with a “~Log_Definition” tab.

Click on the “Continue” button to read and parse the LAS file into the cross section web app. The user will notice that on the “Load Data” dialog the LAS file name “1011640904.las” is entered into the “Log ASCII Standard (LAS) Files” panel. And the Log Data Type is marked with a “YES” under the KGS Column in the Data Type Panel.

The perforations data will automatically load if the data is present in the Perforations Database Table for the selected well.  The well header summary data is also automatically downloaded to the “Load Data” panel also.

Click on the “Top Picks” button to load the “Move/Merge KGS Data” dialog, which will display in the “KGS Stratigraphic Units” panel a list of Tops Sources, i.e., “ACO-1”, “ELOG-EM”, “HUG ELOG-EM” and “MKD”.  Clicking on each source name radio button will show that only the “HUG ELOG-EM” tops source will have the necessary tops. Load by clicking on the “HUG ELOG_EM” tops source and then in the “Add to User’s Stratigraphic Units List” radio buttons, select the “Add to List” radio button. Click on the “Add All” button to copy the list of tops to the “User’s Stratigraphic Units” table.

Click on the “Load Data” button to import the tops data in the “User’s Stratigraphic Units” table to the “Load Data” panel. Notice that in the “Data Type” Panel the Tops Data has a “YES” in the KGS Column.

Now select the “Close” button in the “Search for Data on KGS Server” dialog since all the data that can be imported for the Newby 3-28 has been imported. Now click on the “Add Well Data” button in the “Load Data” dialog to transfer the Newby 3-28 well data to the Cross Section Panel.

Now click on the “Map” button to display the map plot to view the selected wells. The well locations are plotted on latitude versus longitude map grid to help the user visualize the location of the well in relative 2-dimensional space and to also verify that the order of the wells follow a logical presentation.  If the wells were out of order with respect to each other the user will see the blue cross section lines crossing.  The user can correct any errors in the well order by clicking on the “Up” or “Down” buttons on the Cross Section Panel.  As the user will see that the order of the wells are correct and no further action needs to be taken.  The user can click on the “Create PDF of Plot” button to create a Portable Network Graphic (PNG) image of the cross section map.  The user will see a search PC dialog that will allow the user to search their PC directory structure to save the PNG image.  Once the image is saved a HTML web page will display with a “Create a PDF Document of this PNG Image” URL Link at the top of the page above the cross section map image.  Click on the link to launch the Create PDF (Portable Document Format) from Image Applet that will allow the user to convert the saved PNG image to a PDF document.

The user now can create a cross section plot of the wells that have been selected.  Click on the “Cross Section” Plot button to display the cross section of the three wells. The image below illustrates the cross section of the three wells that have been imported into the Cross Section Web App.  The Cross Section Plot Control plots all three wells in a depth profile.   The plot below represents the cross section of 3 Kansas wells with the same Township Range and Sections, i.e. Township 31 South (S) – Range 37 West (W) and Section 28.  The plot is plotted by elevation depth from 700’ down to 0’ at a 100 feet / inch scale.

Figure: Cross Section of three Kansas wells in the same Township-Range-Section Location, i.e. Township 31S-Range 37W-Section 28 plotted by Elevation from 700 to 0 feet at 100 feet/inch scale.

Importing Well Data from User’s PC

Download either the ASCII Text Files directly or the Zip files extracting the contents into a directory.  The problem with the ASCII Text Files being downloaded directly from a web page is that the web page will alter the contents so it does not retain the basic structure and add HTML text to the file.  The preferred method if you have Zip or WinZip is to download the zip files to your PC and extract.

Well Data: Wellington KGS 1-32, Sumner County, Kansas

| Type | ASCII Text Files |
| --- | --- |
| LAS 2.0 | http://www.kgs.ku.edu/Gemini/Tools/documentation/Wellington-KGS-1-32.las |
| Tops | http://www.kgs.ku.edu/Gemini/Tools/documentation/Wellington-KGS-1-32_Tops.csv |

| Type | Zip Files |
| --- | --- |
| LAS 2.0 | http://www.kgs.ku.edu/Gemini/Tools/documentation/Wellington-KGS-1-32.zip |
| Tops | http://www.kgs.ku.edu/Gemini/Tools/documentation/Wellington-KGS-1-32_Tops.zip |

Well Data: Wellington KGS 1-28, Sumner County, Kansas

| Type | ASCII Text Files |
| --- | --- |
| LAS 3.0 | http://www.kgs.ku.edu/Gemini/Tools/documentation/Wellington_KGS_1-28_LAS3.las |

| Type | Zip Files |
| --- | --- |
| LAS 3.0 | http://www.kgs.ku.edu/Gemini/Tools/documentation/Wellington_KGS_1-28_LAS3.zip |

Well Data: Meridith 3, Township 31 South, Range 1 West, Section 28, Sumner County, Kansas

| Type | ASCII Text Files |
| --- | --- |
| LAS 3.0 | http://www.kgs.ku.edu/Gemini/Tools/documentation/Meridith_3-T31S-R1W-sec28.las |

| Type | Zip Files |
| --- | --- |
| LAS 3.0 | http://www.kgs.ku.edu/Gemini/Tools/documentation/Meridith_3-T31S-R1W-sec28.zip |

This example will focus on well data that are on the user’s PC.  The three wells for this example are as follows,

- Wellington KGS 1-32 (15-191-22591)

- Wellington KGS 1-28 (15-191-22590)

- Meridith 3 (15-191-21556)

The user must load the well data for each well separately by clicking the “New Well” button for each well loading the well data with the “Load Data” dialog.  Clicking the “Add Well Data” button at the bottom of the “Load Data” dialog will transfer the well data to the Cross Section Panel.  The details of loading well data from the User’s PC can be found by going to the Importing PC Data - Download Well Data to PC Section (page 27) of this document for loading the Wellington KGS 1-32 well data example.  The Wellington KGS 1-32 has three separate files the Log ASCII Standard (LAS) version 2.0 file, Tops Comma Separated Values (CSV) file and the Geologist Cuttings/Core Description ASCII Delimited file. The other two wells are Log ASCII Standard (LAS) version 3.0 Files with the same data types loaded in the individual files, i.e. the LAS 3.0 file will hold not only the log data, but also the tops, measured core data, geologist cuttings/core description, etc.

- Wellington KGS 1-32

  - Import Log ASCII Standard (LAS) version 2.0 File

Click on the “New Well” button on the Cross Section Panel to open the “Load Data” dialog. The user then clicks on the “LAS version 2.0 & 3.0” icon button to open the “Select LAS File from your PC” dialog.

Highlight “Wellington-KGS-1-32” LAS File and click on the “Open” button to open the “LAS File Curve Sections” dialog, which lists all the data sections in the LAS File.  This example only has Log Data so only one panel will be displayed with a “~Log_Definition” tab.

Click on the “Continue” button to read and parse the LAS file into the cross section web app. The user will notice that on the “Load Data” dialog the LAS file name “Wellington-KGS-1-32.las” is entered into the “Log ASCII Standard (LAS) Files” panel. And the Log Data Type is marked with a “YES” under the LAS Column in the Data Type Panel.

  - Import Tops Comma Separated Values (CSV) ASCII Delimited File

The user now clicks on the “Tops CSV” icon button to open the “Select Formation Tops Comma Delimited File from your PC” dialog.

Highlight “Wellington-KGS-1-32_Tops” CSV File and click on the “Open” button to open the “Map File Column Number to Region Column” dialog, which will parse the Tops CSV file first two lines and map the column number (starting at 1, 2, 3 …) of the data header to the Tops data structure variables.  As the user sees the “File Column Number” column are matched to the Tops data structure variables.

The Wellington KGS 1-32 Tops CSV example has two introduction lines, the first line is the well header information and the second line is the actual column labels for the tops data, illustrated below,

Figure: Partial Contents of the Wellington-KGS-1-32_Tops.csv File.

The “Map File Column Number to Region Column” Dialog allows the user to map the data in the Tops CSV File to the web app data structure variables.  The program first reads the first and second line of the CSV File looking for the data column headers.    The lines are each parsed to single out the data column headers and to match those headers to the tops data structure.  The program then assigns the column number to the Region Column Name starting at column 1,2,3, … if the file column name used matches the expected region column name.

The Wellington KGS 1-32 Tops CSV File example above line 2 has only the Top, Tops Name, Rank, System, Subsystem, Series and Source as the column name variables.  The program was able to map each of the column headers to the tops data structure, except Source, i.e.

| Column | File Column Label | Tops Data Name |
| --- | --- | --- |
| 1 | Top | Depth Top |
| 2 | Name | Stratigraphic Name |
| 3 | Rank | Stratigraphic Unit Rank |
| 4 | System | System |
| 5 | Subsystem | Subsystem |
| 6 | Series | Series |
| 7 | Source |  |

When the user selects the “Load Data” button on the “Map File Column Number to Region Column” Dialog the data is parsed into the Cross Section Program, where the Tops CSV file name is entered into the “PC ASCII Files:” Panel as well as the data type source.

Now click on the “Add Well Data” button in the “Load Data” dialog to transfer the Wellington KGS 1-32 well data to the Cross Section Panel.

Notice that in the LTG column identifies the data type that was imported from the KGS database and file server, i.e. L=Log Data and T=Tops Data. Notice that the “Cross Section” Plot button, Modify Well Header Information Data “Modify” button and the “Remove”, “Remove All”, “Up” and “Down” buttons are now enabled, because there is a well in the “Cross Section Well/Location List” table.  The “Map” button will not enable until there is at least two wells in the table.

Notice also that the Status, Latitude, Longitude and Elevation columns are missing data.  Highlight the Wellington KGS #1-32 row and click on the “Modify” button to display the “Edit Header Information” dialog displaying the well header information data. The user can manually enter the fields if they have access to the data or if this is a Kansas Well search the KGS Database for the well header information by clicking on the “Search KGS Database for Well Header Information” button at the top of the “Edit Header Information” dialog, which will display the “Search for Data on KGS Server” dialog.  This dialog allows the user to search by API-Number, Partial Lease Name or by Township Range Section.  Since the API-Number is known, enter the API-Number in the “Enter API-Number” text field and select the “Search” button to search the KGS Database for the well data.

The Header Information Dialog displays the contents of the header information data structure.  The user can edit the fields and select the “Ok” Button to transfer the information back to the Profile Program and any summary information will be updated in the Profile Control and Plot.

As this example illustrates there are missing fields in the header information data.  The user can select the “Search KGS Database for Well Header Information” Button, which will display a “Search for Data on KGS Server” Dialog that will allow the user to build a query that will download all wells that match the query.

This will display the “Search for Data on KGS Server” Dialog, see image below.  This dialog allows the user to search the KGS database for well header data.  In this example, the well of interest will be the Wellington KGS 1-32.

As the Summary image suggests there are 3 methods for searching for the well header information within this dialog,

- By API-Number – KGS has a specific format for the API-Number, i.e.SS-CCC-99999 where SS is the state code for Kansas 15, CCC is the county code for Wellington KGS 1-32 it is 191 for Sumner County and the 5-Digit Well Number for the Wellington KGS 1-32 is 22591.

- By Partial Lease Name – The stored procedure used to retrieve the well header information allows the user to enter a partial phrase, in this example Wellington.  The program places a ‘%’ in front and back of the phrase and sends the request to the Database, i.e. “%Wellington%”.

- By Township Range Section – This search is by location in Kansas, this search also allows the user to enter just the Township and Range to search for wells, e.g. to look for the Wellington KGS 1-32, enter Township as 31 set the S (South) Radio button and Range as 1 set the E (East) Radio button.

The user only needs to enter the above data and select the “Search” Button to display the list of Wells in the Kansas Database that match the search criteria.  In the image below the Lease Name “Wellington” is entered to search for all wells in Kansas with the Phrase Wellington in it. The user searches through the list until they find the well of interest.  In this example it is the Wellington KGS 1-32, which is highlighted.

The user clicks on the “Select” button to transfer the header information to the Edit Header Information Dialog.

The “Show KGS Well Header Data” radio button will become enabled if the KGS well header information has been downloaded.  The user can move between the initial header information by selecting the “Show Initial Header Data” radio button the KGS well header information by selecting the “Show KGS Well Header Data” radio button.

There a number of buttons on the panel that will allow the user to change or compute data in the header information.  The “Kansas TRS to Latitude, Longitude & Elevation” Button computes the latitude, longitude and elevation from the township, range and section by making an Oracle PL/SQL Stored Procedure, e.g.

http://chasm.kgs.ku.edu/ords/iqstrat.TRS2LL_pkg.getXML?iTownship=31&sTownship=S&iRange=1&sRange=E&iSection=32

The call will return a XML with the latitude, longitude and ground level elevation.

<?xml version="1.0"?>

<!DOCTYPE headers [

<!ELEMENT headers (data*)>

<!ATTLIST headers records CDATA #IMPLIED>

<!ELEMENT data (info*,

other?,

loc?,

xy?,

z?,

comments?,

misc?,

cnt?)>

<!ELEMENT info  EMPTY>

<!ATTLIST info  kid   CDATA #IMPLIED

well_kid   CDATA #IMPLIED

key   CDATA #IMPLIED

type  CDATA #IMPLIED

api   CDATA #IMPLIED

name   CDATA #IMPLIED

status CDATA #IMPLIED

error  CDATA #IMPLIED>

<!ELEMENT other EMPTY>

<!ATTLIST other operator  CDATA #IMPLIED

oper_kid  CDATA #IMPLIED

field     CDATA #IMPLIED

field_kid CDATA #IMPLIED>

<!ELEMENT loc EMPTY>

<!ATTLIST loc state     CDATA #IMPLIED

state_cd  CDATA #IMPLIED

county    CDATA #IMPLIED

county_cd CDATA #IMPLIED

loc       CDATA #IMPLIED

town      CDATA #IMPLIED

town_dir  CDATA #IMPLIED

range     CDATA #IMPLIED

range_dir CDATA #IMPLIED

section   CDATA #IMPLIED>

<!ELEMENT xy  EMPTY>

<!ATTLIST xy  latitude  CDATA #IMPLIED

longitude CDATA #IMPLIED

zone      CDATA #IMPLIED

utm_x     CDATA #IMPLIED

utm_y     CDATA #IMPLIED>

<!ELEMENT z   EMPTY>

<!ATTLIST z   depth     CDATA #IMPLIED

gl        CDATA #IMPLIED

kb        CDATA #IMPLIED

df        CDATA #IMPLIED>

<!ELEMENT comments (#PCDATA)>

<!ELEMENT misc EMPTY>

<!ATTLIST misc user      CDATA #IMPLIED

access    CDATA #IMPLIED

source    CDATA #IMPLIED

date      CDATA #IMPLIED>

<!ELEMENT cnt EMPTY>

<!ATTLIST cnt  las       CDATA #IMPLIED

tops      CDATA #IMPLIED

core      CDATA #IMPLIED

images    CDATA #IMPLIED>]>

<headers records="1">

<data>

<loc town="31" town_dir="S" range="1" range_dir="E" section="32" />

<xy latitude="37.311703" longitude="-97.339619" />

<z gl="1277" />

</data>

</headers>

The “UTM” Button will compute the UTM XY coordinates from the latitude & longitude. The analysis uses A Working Manual by John P. Snyder, U.S. Geological Survey Professional Paper 1395, USG Printing Office, Washington, DC, 1987 (http://pubs.er.usgs.gov/djvu/PP/PP_1395.pdf ).

Select the “Ok” button to update the Wellington KGS 1-32 Header Information in the Cross Section Panel.

- Wellington KGS 1-28

  - Import Log ASCII Standard (LAS) version 3.0 File

Click on the “New Well” button on the Cross Section Panel to open the “Load Data” dialog. The user then clicks on the “LAS version 2.0 & 3.0” icon button to open the “Select LAS File from your PC” dialog.

Highlight “Wellington-KGS-1-28” LAS File and click on the “Open” button to open the “LAS File Curve Sections” dialog, which lists all the data sections in the LAS File.  This example contains Log Data, Tops Data and Geologist Description Data so there will be multiple tabbed panels displayed i.e.

- “~Log_Definition” tab – holds the LAS File Log Data Section

- “~Tops_Definition” tab – holds the Top Picks Data Section

- “~IQ_Control_Definition” tab – holds the Profile Plot Control Data Section, which allows the user to automatically re-plot the profile plot to pre-selected values, i.e. the selected Log Curves that were not originally defined in the LAS version 2.0 file.

- “~IQ_Geo_Report_Definition” tab – holds the Geologist Cuttings/Core Description Data Section.

Notice the last two panels have “~IQ_” before the description. These data sections are “user” defined sections, outlined in the Canadian Well Logging Society LAS 3 File Structure Document, http://www.cwls.org/wp-content/uploads/2014/09/LAS_3_File_Structure.pdf .

Click on the “Continue” button to read and parse the LAS file into the cross section web app. The user will notice that on the “Load Data” dialog the LAS file name “Wellington_KGS_1-28_LAS3.las” is entered into the “Log ASCII Standard (LAS) Files” panel. And the Log, Tops and Geologist Report Data Types are marked with a “YES” under the 3.0 Column in the Data Type Panel.

Now click on the “Add Well Data” button in the “Load Data” dialog to transfer the Wellington KGS 1-28 well data to the Cross Section Panel.

Notice that in the LTG column identifies the data type that was imported from the KGS database and file server, i.e. L=Log Data, T=Tops Data and G=Geologist Report Data. At this point notice that the “Map” button is now enabled, which allows the user to display a map of the two wells plotted to give a sense of the location.

- Meridith 3 (15-191-21556)

  - Import Log ASCII Standard (LAS) version 3.0 File

Click on the “New Well” button on the Cross Section Panel to open the “Load Data” dialog. The user then clicks on the “LAS version 2.0 & 3.0” icon button to open the “Select LAS File from your PC” dialog.

Highlight “Meridith_3-T31S-R1W-sec28” LAS File and click on the “Open” button to open the “LAS File Curve Sections” dialog, which lists all the data sections in the LAS File.  This example contains Log Data, Tops Data and Geologist Description Data so there will be multiple tabbed panels displayed i.e.

- “~Log_Definition” tab – holds the LAS File Log Data Section

- “~Tops_Definition” tab – holds the Top Picks Data Section

- “~IQ_Control_Definition” tab – holds the Profile Plot Control Data Section, which allows the user to automatically re-plot the profile plot to pre-selected values, i.e. the selected Log Curves that were not originally defined in the LAS version 2.0 file.

- “~IQ_Geo_Report_Definition” tab – holds the Geologist Cuttings/Core Description Data Section.

Notice the last two panels have “~IQ_” before the description. These data sections are “user” defined sections, outlined in the Canadian Well Logging Society LAS 3 File Structure Document, http://www.cwls.org/wp-content/uploads/2014/09/LAS_3_File_Structure.pdf .

Click on the “Continue” button to read and parse the LAS file into the cross section web app. The user will notice that on the “Load Data” dialog the LAS file name “Meridith_3-T31S-R1W-sec28.las” is entered into the “Log ASCII Standard (LAS) Files” panel. And the Log, Tops and Geologist Report Data Types are marked with a “YES” under the 3.0 Column in the Data Type Panel.

Now click on the “Add Well Data” button in the “Load Data” dialog to transfer the Meridith 3 well data to the Cross Section Panel.

Now click on the “Map” button to display the map plot to view the selected wells. The well locations are plotted on latitude versus longitude map grid to help the user visualize the location of the well in relative 2-dimensional space and to also verify that the order of the wells follow a logical presentation.  If the wells were out of order with respect to each other the user will see the blue cross section lines crossing.  The user can correct any errors in the well order by clicking on the “Up” or “Down” buttons on the Cross Section Panel.  As the user will see that the order of the wells are correct and no further action needs to be taken.  The user can click on the “Create PDF of Plot” button to create a Portable Network Graphic (PNG) image of the cross section map.  The user will see a search PC dialog that will allow the user to search their PC directory structure to save the PNG image.  Once the image is saved a HTML web page will display with a “Create a PDF Document of this PNG Image” URL Link at the top of the page above the cross section map image.  Click on the link to launch the Create PDF (Portable Document Format) from Image Applet that will allow the user to convert the saved PNG image to a PDF document.

The user now can create a cross section plot of the wells that have been selected.  Click on the “Cross Section” Plot button to display the cross section of the three wells. The image below illustrates the cross section of the three wells that have been imported into the Cross Section Web App.  The Cross Section Plot Control plots all three wells in a depth profile.   The plot below represents the cross section of 3 wells imported from the user’s PC.  The plot is plotted by elevation depth from -2200’ down to -3000’ at a 100 feet / inch scale.

Figure: Cross Section of three wells imported from the user's PC, plotted by elevation depth from -2200 to -3000 feet at 100 feet/inch scale.

Import the XML (Extensible Markup Language) file from User’s PC with a list of 56 wells

Download either the ASCII Text File directly or the Zip files extracting the contents into a directory.  The problem with the ASCII Text Files being downloaded directly from a web page is that the web page will alter the contents so it does not retain the basic structure and add HTML text to the file.  The preferred method if you have Zip or WinZip is to download the zip files to your PC and extract.

List of 56 Well Data LAS File Location & Tops along Township 11 South from Ranges 21-40 West.

| Type | ASCII Text Files |
| --- | --- |
| LAS 3.0 | http://www.kgs.ku.edu/Gemini/Tools/documentation/gr_x_section_11S-R21-40W-Final.xml |
| Type | Zip Files |
| LAS 3.0 | http://www.kgs.ku.edu/Gemini/Tools/documentation/gr_x_section_11S-R21-40W-Final.zip |

This example will focus on loading a cross section that was created by selecting wells data from the KGS Server and saved to a XML File.

Click on the “File” menu in the top left corner of the Cross Section panel and then select the “Open an Existing Gamma Ray Cross Section” menu option to open the “Select LAS File from your PC” dialog.

Highlight “gr_x_section_11S-R21-40W-Final” XML File and click on the “Open” button to automatically load the 56 Wells into the Gamma Ray Cross Section Panel.  You will notice a pump jack dialog to show that the data is being loaded. You will also notice that the Load XML Status Bar Field will start to show the status of wells loaded.

The Load XML Status Bar divides the number of wells loaded by the total number of wells in the list and multiplies by 100.  When all the wells are loaded the “Cross Section Well/Location List” Table will show the wells.

Now click on the “Map” button to display the map plot to view the selected wells. The well locations are plotted on latitude versus longitude map grid to help the user visualize the location of the well in relative 2-dimensional space and to also verify that the order of the wells follow a logical presentation.  If the wells were out of order with respect to each other the user will see the blue cross section lines crossing.  The user can correct any errors in the well order by clicking on the “Up” or “Down” buttons on the Cross Section Panel.  As the user will see that the order of the wells are correct and no further action needs to be taken.  The user can click on the “Create PDF of Plot” button to create a Portable Network Graphic (PNG) image of the cross section map.  The user will see a search PC dialog that will allow the user to search their PC directory structure to save the PNG image.  Once the image is saved a HTML web page will display with a “Create a PDF Document of this PNG Image” URL Link at the top of the page above the cross section map image.  Click on the link to launch the Create PDF (Portable Document Format) from Image Applet that will allow the user to convert the saved PNG image to a PDF document.

The user now can create a cross section plot of the wells that have been selected.  Click on the “Cross Section” Plot button to display the cross section of the 56 wells. The image below illustrates the cross section of the 56 wells that have been imported into the Cross Section Web App.  The Cross Section Plot Control plots all 56 wells in a depth profile.   The plot is plotted by elevation depth from -2200’ down to -3000’ at a 100 feet / inch scale.

The blue and green colors represent clean formation, yellow, orange and red are shaly and shale formations.  The user also has the option of turning off the colors and plotting the data by gray scale. This cross section illustrates the sub surface geology over about 60 Miles in western Kansas.

The light grays are clean formation where the darker grays are the shaly and shale formations.

Exporting Gamma Ray Cross Section Well Data Information

The user can save the gamma ray cross section to a XML (Extensible Markup Language) file, which will retain the Well Header Information, the URL directory path & the LAS filename and the Tops Data for each well.  Since it takes time to load a large data set into the program the save capability allows the user to save their work.  The 56 wells used to create the previous section of this document took about 6 hours to locate and evaluate each well for the type of data and quality of the tops data.  It would be a shame not to be able to save that data to display at a later time or to add to it to extend it further across Kansas.

Select the “File” menu and then select the “Save Gamma Ray Cross Section Information” menu option in the Cross Section Panel.  This will open the “Select a Different Directory Path” dialog.

Select the “Continue” button to save the gamma ray cross section information.  The user can click on the XML File to see the contents, which will open up in a browser.

It doesn’t matter how many wells were originally selected this XML file has everything it needs to retrieve the well data, i.e. LAS File and Well Header and Tops information for each well and rebuild the gamma ray cross section.  As long as the URL Directory Path is not changed either in the Kansas Geological Survey (KGS) Server if the well data was retrieved from the KGS or from the user’s PC the Gamma Ray Cross Section Web App will find the files and reload the gamma ray cross section.

Cross Section Panel – Well Location Image Map

Cross Section Panel – Change the Order of the Selected Wells

Cross Section Plot Control Dialog

The “Cross Section Plot Control” dialog allows the user to change the presentation of the Cross Section Plot, by depth range, by depth scale, Paint Gamma Ray Colorlith by Color or by Grays, Datum By: (Log Depth, Horizon and Elevation) , or add, modify or delete tops data through Tops Data Entry dialog.

Stratigraphic Units Plot Track – The user can left click the mouse on the well gamma ray colorlith plot track to display the “Enter Horizon Data” Dialog with the “Stratigraphic Units” Data Entry Panel displayed.  This panel assists the user in adding, modifying or deleting tops from the profile plot.  The user will need to keep track of which well is being modified. This dialog has two buttons to set the Stratigraphic Units for a top, i.e.

- ICS (International Commission on Stratigraphy) Chart Button displays the accepted stratigraphic units.

- 1968 Kansas Chart Button displays the Accepted Kansas stratigraphic units.

Adding Tops

### Enter Horizon Data - Stratigraphic Units Panel

The Stratigraphic Units Panel allows the user to Add, Modify or Remove Stratigraphic Units. There are two files that are used to assist in mapping Stratigraphic Units to a specific top name.

- The 2010 International Commission on Stratigraphy Stratigraphic Units and RGB color for the Stratigraphic Units XML File (http://www.kgs.ku.edu/software/gemini/data/ISC.xml) are used to display the tops in the Stratigraphics Plot Track by Age (RGB Color).

- The Kansas Geological Survey (KGS) Stratigraphic Succession in Kansas, edited by D.E. Zeller, December 1968, updated 2012 (http://www.kgs.ku.edu/software/gemini/data/kansas.xml) will help map the Kansas Top Names to Stratigraphic Units, System, Sub-System, Series, etc. and to map the top depth of one top pick to the base depth of another top pick.

The KGS Stratigraphic Succession of Kansas edited by D. E. Zeller, Dec. 1968, updated 2012 (http://www.kgs.ku.edu/PRS/Ozark/TYPE_LOG/Stratigraphic/index.html).

Figure: The 2010 International Commission on Stratigraphy Stratigraphic Units and RGB colors for the Stratigraphic Units.

### Add Shawnee Group to the Stratigraphic Units List

### This first example is to add a Shawnee Group, which is part of the KGS Stratigraphic Succession in Kansas. First place the mouse in the Stratigraphic Units Plot Track and left click with the mouse to display the “Enter Horizon Data” Dialog with the “Stratigraphic Units” Panel.

### The user can manually enter the tops with as much information as they wish to display.  If the stratigraphic unit is part of the Kansas Stratigraphic Units List all necessary fields can be loaded by using the “1968 Kansas Chart” button to display the list of Kansas Stratigraphic Units.

### Add Unknown Bed (SG-A Bed) to the Stratigraphic Units List

This second example is to add an unknown bed (SG-A Bed) to the Stratigraphic Units List, which is not part of the KGS Stratigraphic Succession in Kansas. This example will enter the basic information to the Depth Range text fields, the Stratigraphic Name text filed and setting the Rank as a BED.  The Stratigraphic Units will be provided by the International Commission on Stratigraphy Dialog by selecting the “ICS Chart” Button.  The “International Stratigraphic Chart” Dialog displays the stratigraphic units as a series of colored boxes, each stratigraphic unit is has the recommended RGB (Red-Green-Blue) Color defined by the International Commission on Stratigraphy.  The user only needs to click the colored box to select all the stratigraphic data associated with the selected stratigraphic unit and transfer the data back to the “Stratigraphic Units” Panel in the “Enter Horizon Data” Dialog.