PfEFFER-java Applet                                                                            by John R. Victorine

Introduction

The PfEFFER-java web app has 2 sources for importing well data, 1) the user’s PC or 2) the Kansas Geological Survey (KGS) Server & ORACLE Database.  This program allows the user to import Log, Tops, and Core Data.

There will be occasions when the user would like to inspect the contents of a LAS file. So, for example, the user may wish to find the identity of the well, the types of logs, depth range, well or log parameters. Alternatively, the user can suspect that the file is not coded correctly in LAS format if the Import LAS function fails, and wishes to examine the file to troubleshoot the problem. The LAS file is an ASCII file and can be read by any text editor, i.e. Notepad, WordPad, TextPad, etc. The CSV (Comma Separated Values) files are also ASCII and can be edited in the same way or with Microsoft Excel.

To access PfEFFER-java go to http://www.kgs.ku.edu/software/PfEFFER-java/.  At the top of the web page there is a menu "Main Page|Description|Applet|Help|Copyright & Disclaimer|".  Select the "Applet" menu option a "Warning - Security" Dialog will appear. The program has to be able to read and write to the user’s PC and access the Kansas Geological Survey (KGS) Database and File Server, ORACLE requires this dialog. The program does not save your files to KGS, but allows you to access the KGS for well information that may be missing in your Kansas logs. The program does not use Cookies or any hidden software it only reads the LAS and CSV files for the PfEFFER-java Session and writes a LAS 3.0 File to your PC to save your PfEFFER-java Session and the well data imported. The blue shield on the warning dialog is a symbol that the Java web app is created by a trusted source, which is the University of Kansas.  Select the "Run" Button, which will show the PfEFFER-java "Enter" Panel illustrated below,

# Contents

Loading Well Data   …………………………..………………………………………………………………………………………………….. 4 Data Source Panel	………………………………………………………………………………………………………………………... 4 Data Loaded Panel ………………………………………………………………………………………………………………………... 5

Importing KGS (Database & Server) Data …….…………………………………………………………………………………….… 6 Importing Well Data ……………………………………………………………………………………………………………………... 6   * LAS File Data …………………………………………………..………………………………………………………………………….  9 - Map Curves & Change Curve Selections …………………………..…….……………………….………………………... 11 * Tops Picks ………………………………………………………………………………….….………………………………………….. 16 - MKD Source Example ………………………………………………………………………………………………………………… 17 * Core Data …………………………………………………………………………………………………………………………………. 20

Importing PC Data – Download Well Data from PC …….……………………………………………………………………… 22 Log ASCII Standard (LAS) version 2.0 File …………………………………………………………………………………….. 23 - Map Curves & Change Curve Selections …………………..………………………………………………………….……. 25     Tops CSV (Comma Separated Values) File …………………………………………………………………………….……… 35      - Tops CSV (Comma Separated Values) File Structure ………………………………………………………………….. 36     Core CSV (Comma Separated Values) File ……………………………………………………………………………………. 39      - Core Data CSV (Comma Separated Values) File Structure ………………………………………………………….. 41

PfEFFER Dialog ………………..…………………………………….…….……………………………………………………………………… 45 PfEFFER Panel Organization …………………….………………………………………………………………………………….. 46 – Tool Bar ……..………………………………………….………………..………………………………………………………….……. 47 – Selected Values Table ……………..…………….………………..………………………………………………………….……. 47     – Archie & Other Parameters Panel ………….………………..………………………………………………………….……. 47 – Home Area …………………….………..…………….………………..………………………………………………..……….……. 50

Adding Flow Units ………………….……………………….………………..………………………………………………………….……. 51     By User Defined Depth Ranges …………………………………………………………………………………………….……… 52      By Zonation a Depth-Constrained Cluster Analysis ……….…………………………………………………………….. 56     By Tops ……………………………………………………………………………………………………………………………………….. 60

Cut Offs and Colors ………………………………….………………………………………………………………………………….……… 62

Modifying the Log Curve Selections ….…….………………………………………………………………………………….……… 64  Calculating RT ……………………………………………………………………………………………………………………………… 64 Calculating VSH ………………………………………………………………………………………………………….………………… 65 Calculating PHI …………………………………………………………………………………………………………………………..… 67 Calculating RXO …………………………………………………………………………………………………………………………… 70

Selecting a Water Saturation Model ……….………………………………………………………………………………….……… 71 Selecting Simandoux Menu Option ……………………………………………………………………………………………… 73 Selecting Dual Water Menu Option …………………………………………………………………………………………….. 74

PfEFFER Plot Options ……….…………………………………………………………………………………………………………………. 75 Profile Plot Control Dialog & Profile Plot ………………………………………………..……………………………………. 75 - Data Entry Dialogs …………………………………………………………………………………………………………………….. 78 Pickett & Hingle Plots ……………………………………………………………………………………………………………..…… 81 * Modify Plot Titles ……………………………………………………………………………………………………………………… 82 * Modify Plot Axes ………………………………………………………………………………………………………………………. 83 * Connect Data Points …………………………………………………………………………………………………………………. 84 * Modify Archie Equation Parameters …………………………………………………………………………………………. 86 * Filter By Data ……………………………………………………………………………………………………………………………. 88 * Modify Plot Sw, Bvw, K Lines ………………………………………………………………………………………………….…. 91 - Water Saturation (Sw) Plot Lines …………………………………………………………………………………………….…. 91 - Bulk Volume Water (Bvw) Plot Lines …………………………………………………………………………………………. 94 - Permeability (K) Plot Lines ……………………………………………………………………………………………………….… 95 * Modify Plot Data Colors ………………………………………………………………………………………………………….… 96 - Initial Data Point Color - Black ……………………………………………………………………………………………….….. 97 - Default Button: RHOmaa-Umaa-GR Data Limits - Use  Default Limits ……………………………….….……. 97 - Filter Button: RHOmaa-Umaa-GR Data Limits - Set using Filter Data Limits ……….……………….……… 98 - Depth Button: Evenly distribute 40 colors over depth range …………………………………………….……….. 98 * Modifying the Log Curve Selection ………………………………………………………………………………….………… 99 - Calculating RT ………………………………………………………………………………………………………………….………… 99 - Calculating VSH …………………………………………………………………………………………………….……….………… 100 - Calculating PHI …………………………………………………………………………………………………………….………..… 102 Moveable Hydrocarbon Plot ……………………………………………………………………………………………………… 105

PfEFFER Save Data Options ………………………………………………………………………………………….…………………... 112 Save well data as Log ASCII Standard (LAS) version 3.0 File ……………………………………………………….. 112 Save Cottonwood Flow Unit as a Comma Separated Values (CSV) File ………………………………………. 119

Loading Well Data

Click the "PfEFFER-java Enter” Icon Button, which will show the "Load Data" Dialog. The dialog below displays an example of the Newby 2-28R well data loaded from the KGS Data icon button with the data in the tables above.  The icon buttons in the Data Source Panel assists the user in loading well data into the PfEFFER-java Applet.

Data Source Panel

The Data Source Panel provides two methods of importing data into the PfEFFER-java Web App.  The Kansas Geological Survey (KGS) Database & File Server and the user’s PC.  A number of icon buttons are provided to assist the user in importing the specific data type of interest. When the user selects the icon button a search dialog is provided specific to the data type.  The CSV (Comma Separated Values) icon buttons under the “PC ASCII Delimited Data Files” Panel are expecting a general type of data presentation.  Although the order of the specific data columns is not important, the “Mnemonics” of the data column is.  Each data type in GEMINI Tools web apps have a data mnemonic list that will be presented later as each icon search dialog is presented. The CSV Search Dialog will use the first two lines of the CSV file to automatically match the file column data mnemonics with the web app curve mnemonics, but if the program does not recognize the file data mnemonic then it will leave it blank and expect the user to match the file data mnemonic to the web app curve mnemonics, this will be explained later.

Data Loaded Panel

The Data Loaded Panel provides a visual feedback that the data type was loaded, by the file names of the files that were accessed to load the data and by the data type that is loaded.  The data type is important in that it shows where the data came from.  The KGS data has the ORACLE Database from which the Tops Data, Core Data and Brine Data, etc. are retrieved from a XML (Extensible Markup Language) data stream that is constructed using the ORACLE PL/SQL for each data type.  The user’s PC will load the data from CSV (Comma Delimited Values) files or a delimited file for the Geologist Report.  The LAS File can be downloaded automatically from the KGS Server in the program or from the user’s PC.  This program allows the user to import up to 3 Log ASCII Standard (LAS) version 2.0 or 3.0 files.  The LAS version 3.0 file can hold all the well data, but if the user wishes to add log curves from a LAS version 2.0 file it is advised that the LAS version 3.0 file be loaded last.  In most cases the user is importing multiple LAS version 2.0 files, which can be saved into a single LAS version 3.0 file.

Importing KGS (Database & Server) Data

KGS (Database & Server) - Importing Well Data

The Kansas Geological Survey (KGS) has a good collection of well data stored in the ORACLE Database and File Server as Files Log ASCII Standard (LAS) version 3.0 Files and Core Image Files as Joint Photographic Experts Group (JPEG) images as well as Tagged Image File Format (TIFF) images.  In this example the user will download the well data available from the KGS, Log data (LAS version 2.0 File), Tops Data, Measured Core Data, Perforation Data and Core JPEG Image Files.  The ORACLE Database is accessed by making Stored Procedure PL/SQL calls to the ORACLE Database from which an Extensible Markup Language (XML) data stream is created containing the well data that is passed back to the web app making the request.

Left Click on the “Well Data” Icon Button in the Data Source Panel of the Load Data Dialog.

This will display the “Search for Data on KGS Server” Dialog, see above image.  This dialog allows the user to search the KGS database for well data.  In this example, the well of interest will be the Newby 2-28R, this well contains all the well data that can be retrieved from the KGS Database, i.e. Log Data (LAS version 3.0 File), Tops Data, Core Data, Perforation Data and Core JPEG Image Files.

As the Summary image suggests there are 3 methods for searching for the well data within this dialog,

- By API-Number – KGS has a specific format for the API-Number, i.e.SS-CCC-99999 where SS is the state code for Kansas 15, CCC is the county code for Newby 2-28R it is 189 for Stevens County and the 5-Digit Well Number for Newby 2-28R it is 22225.

- By Partial Lease Name – The stored procedure used to retrieve the well header information allows the user to enter a partial phrase, in this example Newby.  The program places a ‘%’ in front and back of the phrase and sends the request to the Database, i.e. “%Newby%”.

- By Township Range Section – This search is by location in Kansas, this search also allows the user to enter just the Township and Range to search for wells, e.g. to look for the Newby 2-28R, enter Township as 31 set the S (South) Radio button and Range as 37 set the W (West) Radio button.

The user only needs to enter the above data and select the “Search” Button to display the list of Wells in the Kansas Database that match the search criteria.  In the image below the Lease Name “Newby” was entered to search for all wells in Kansas with the Phrase Newby in it. The user searches through the list until they find the well of interest.  In this example it is the Newby 2-28R, which is highlighted.

Notice that the LTCI represents the type of data that the well contains.  It is a visual aid that lets the user see what is available before trying to download the data.  If you require a LAS file you would want to see an L in that column.  The LTCI labels stand for the following,

- L – Log ASCII Standard (LAS) version 2.0 Files

- T – Tops Data ( Stratigraphic Unit Horizons )

- C – Measured Core Data

- I – Core Joint Photographic Experts Group (JPEG) Image Files

This dialog allows the user to now download each of the data types that are available.

Load KGS Well Data – LAS File Data

The “Search for Data on KGS Server” Dialog allows the user to download data from the KGS Database & Server to the web app.  The “LAS File Data” Button will display the “Select LAS File” Dialog with a list of LAS version 2.0 Files that are available.

In this example there are three LAS files available, with a table suggesting the log data type in the file.  In the beginning of the GEMINI Project (2000-2003) a precursor to the GEMINI Tools the KGS populated the Tool Types from every log that was in the KGS Server at that time.  Unfortunately KGS has not maintain that table for wells uploaded after 2003 so the LAS File may have “No” for all the log types, which is not accurate.  The user will need to open or download the file or search for the Well on the KGS Master List of Oil and Gas Wells in Kansas Web Page (http://www.kgs.ku.edu/Magellan/Qualified/index.html) to see what is in the File Header before deciding to download data from this program.  For this example the first log has most of the data necessary except the Spectral Gamma Ray Logs.  The Table above identifies the following log types,

- OHM-M – Resistivity Logs

- Neutron – Neutron Porosity Log

- Density – Bulk Density and/or Density Porosity Log

- Sonic – Acoustic Transit Time and/or Sonic Porosity Log

- GR – Gamma Ray (API units) Log

- PE – Photoelectric Factor Log

- THOR – Thorium Concentration

- URAN – Uranium Concentration

- POTA – Potassium Concentration)

In this example the first log contains the data needed, highlight the first log and click on the “Select” Button to display the “LAS File Curve Sections” Dialog. The “LAS File Curve Sections” Dialog allows the user to map unknown LAS Curve Mnemonics to the KGS “Standard” Curve Mnemonics so they will be plotted in the Profile Plot.  This program reads the “LAS Tool Curve Mnemonics map to KGS Standard Mnemonics” XML File (http://www.kgs.ku.edu/software/gemini/data/las_standard_tools.xml ), which will automatically maps the Curve Mnemonics from the LAS file to one of 31 KGS “Standard“ Curve Mnemonics.

As you can see this log has all the log types of interest, Gamma Ray API, Resistivity, Neutron/Density,  Photoelectric Factor, Sonic and Permeability.  If a curve Mnemonic is not recognized the program will place a “?” in front of the Mnemonic, e.g. “?(MPERM16)” for the “. : Curve # 51” Log Curve.  If the user is satisfied with the automatic curve selections, which are checked and color coded, they only need to select the “Continue” Button at the bottom of the Dialog to import the file. The next section will take the user through a series of examples in changing the curve selections and mapping unknown curve mnemonics.

Notice that some of the check boxes are colored with different colors, which shows that the curves were automatically selected, but also to represent the curve type by color.The Curves are colored by type (data units) as follows,

- Orange -  OHM-M or Resistivity Logs

- Cyan – PU or porosity Logs, Neutron Porosity, Density Porosity, etc.

- Greenish yellow – BARNS/E or Photoelectric Factor Logs

- Green – GM/CC or Bulk Density Log

- Forest Green – USEC/FT or the Acoustic Transit Time Log

- Red – API, PPM or “%” as Radioactive logs, Gamma Ray, Spectral Gamma Ray, etc.

- Blue – MD or Permeability Logs

- Brown – F, FT or IN or Depth

- Middle yellow – FRAC, or other log curve types.

- Dark Violet – UNI or Unknown Linear Curves

- Medium Violet – UNL or Unknown Logrithum Curves

The color coding of the selected curves were added to also help the user visually recognize that a curve was selected or not.

Map Curves & Change Curve Selections

Some logs will have curve mnemonics that are not recognized as one of the KGS “Standard” Curve Mnemonics.  The user will need to map the log curve to one of the KGS standard curves if they want to display the curve.  Slide the scroll bar down to the Permeability Curves MPERM and ?(MPERM16).

Click on the “?(MPERM16)” Button to display the “Select KGS Standard Tools” Dialog.  This dialog provides a list of the KGS “Standard” Curve Mnemonics, from which the user can map an unrecognized log curve to one of the KGS standard curve mnemonics.  The KGS “Standard” Curve Mnemonics List was created as a way to standardize the alpha bit soup of Log Mnemonics.  Each logging company has their own curve mnemonics to represent similar tools.  The Profile program is a later version of code from the GEMINI Project LAS File Viewer Module, which needed to standardize the log curves so the curves could be automatically read and assigned a plot track.  The “LAS Tool Curve Mnemonics map to KGS Standard Mnemonics” XML File was created to map the log curves from logs that were part of the KGS LAS File Collection which is not a complete list of possible curve mnemonics.

To map the unknown curve mnemonic “?(MPERM16)” you first notice that it similar to the MPERM curve above, which is a permeability curve.  In this example both curves will be plotted together, but the Permeability Plot Track can only plot one curve.  The web app has the ability to allow the user to plot up to 4 unknown logrithum curves and 4 unknown linear curves.  The permeability is usually plotted as logrithmic.  Click on the “?(MPERM16)” Button to display the “Select KGS Standard Tools” Dialog.  Slide the scroll bar down to the “LOG_1” Mnemonic – Semilog Track Curve 1 and highlight that curve.  Click on the “Select” Button to map the “?(MPERM16) to the Semilog Track Curve 1.

The ?(MPERM16) Curve has been changed to MPERM16 removing the ?() around the Curve Mnemonic.  Also select the check box next to it, which changes to a dark violet.

In order to plot both Permeability Curves on the same track, the MPERM must be assigned to the unknown Log Curves with MPERM16.  Click on the MPERM Mnemonic Button to display the

“Select KGS Standard Tools” Dialog.  Slide the scroll bar down to the “LOG_2” Mnemonic – Semilog Track Curve 2 and highlight that curve.  Click on the “Select” Button to map the “MPERM to the Semilog Track Curve 2.

Click the check box next to the MPERM Mnemonic Button you will notice that it has the same color as MPERM16, which now groups the two curves together.  Select the “Continue” Button to load the LAS File into the web app.

Load KGS Well Data – Top Picks

The “Search for Data on KGS Server” Dialog allows the user to download data from the KGS Database & Server to the web app.  The “Top Picks” Button will display the “Move/Merge KGS Data” Dialog with available top picks grouped by the sources creating the tops.

This dialog allows the user to add all or some the tops from each of the sources.  Both tables are set up so the user can use the “Ctrl” Key with the left click of mouse to select multiple tops, i.e.

Notice that the only some of the tops are selected.  You can then select the Add Button to move only the selected tops to the “User’s Stratigraphic Units” Table.

MKD Source Example:

Once the list of tops are in the “User’s Stratigraphic Units” Table the user can edit the list by removing any duplicate or invalid tops.  Notice that the “Council Grove” Top occurs 2 times in the list.  Highlight the one of the “Council Grove” tops.

Now select the “Remove” Button.

The MKD-07/2006 has only one top, so this dialog allows the user to add that top to the “User’s Stratigraphic Units” Table.  The MKD does not have this top and this is an extra top missing from the MKD data set.

Now with the data set complete select the “Load Data” Button to import the Tops data into the web app.

Load KGS Well Data – Core Data

The “Search for Data on KGS Server” Dialog allows the user to download data from the KGS Database & Server to the web app.  The “Core Data” Button will automatically load any measured core data that is in the KGS Database and import directly into the web app.

As the user accepted each data type the “Data Source Filenames:” Panel in the Load Data Dialog changes. The LAS File that was downloaded from the KGS Server to the Profile Web App is added to the “Log ASCII Standard (LAS) Files” panel will show the filename downloaded.  The Log Data, Perforations, Tops Data, Measured Core Data, Core Images and the Geologist Report Data Type have been downloaded from KGS.

Importing PC Data - Download Well Data to PC

Download either the ASCII Text Files directly or the Zip files extracting the contents into a directory.  The problem with the ASCII Text Files being downloaded directly from a web page is that the web page will alter the contents so it does not retain the basic structure and add HTML text to the file.  The preferred method if you have Zip or WinZip is to download the zip files to your PC and extract.

Well Data: Wellington KGS 1-32, Sumner County, Kansas

| Type | ASCII Text Files |
| --- | --- |
| LAS 2.0 | http://www.kgs.ku.edu/Gemini/Tools/documentation/Wellington-KGS-1-32.las |
| Tops | http://www.kgs.ku.edu/Gemini/Tools/documentation/Wellington-KGS-1-32_Tops.csv |
| Core | http://www.kgs.ku.edu/Gemini/Tools/documentation/Wellington-KGS-1-32_Core_Data.csv |

| Type | Zip Files |
| --- | --- |
| LAS 2.0 | http://www.kgs.ku.edu/Gemini/Tools/documentation/Wellington-KGS-1-32.zip |
| Tops | http://www.kgs.ku.edu/Gemini/Tools/documentation/Wellington-KGS-1-32_Tops.zip |
| Core | http://www.kgs.ku.edu/Gemini/Tools/documentation/Wellington-KGS-1-32_Core_Data.zip |

Importing PC Data – Log ASCII Standard (LAS) version 2.0 File

Most of the web apps will use the same input dialogs to import Log ASCII Standard (LAS) version 2.0 or 3.0 files.  The Load Data Dialog is basically the same for most of the Web Apps, except they only load a subset of the total data types.  In this example a LAS version 2.0 file is being imported into the web app.

Left Click on the “LAS File” Icon Button in the Data Source Panel of the Load Data Dialog.  This will display the “Select LAS File from your PC” Dialog.  This dialog allows the user to search their PC for the file of interest.  In this example it is the LAS version 2.0 file Wellington-KGS-1-32.las, highlighted below. Select the Open button to display the “LAS File Curve Sections” Dialog.

The “LAS File Curve Sections” Dialog allows the user to map unknown LAS Curve Mnemonics to the KGS “Standard” Curve Mnemonics so they will be plotted in the Profile Plot.  This program reads the “LAS Tool Curve Mnemonics map to KGS Standard Mnemonics” XML File (http://www.kgs.ku.edu/software/gemini/data/las_standard_tools.xml ), which will automatically maps the Curve Mnemonics from the LAS file to one of 31 KGS “Standard“ Curve Mnemonics.  If a curve Mnemonic is not recognized the program will place a “?” in front of the Mnemonic, e.g. “?(BSZ)” for the “.in : Bit Size” Log Curve.  If the user is satisfied with the automatic curve selections, which are checked and color coded, they only need to select the “Continue” Button at the bottom of the Dialog to import the file. The next section will take the user through a series of examples in changing the curve selections and mapping unknown curve mnemonics.

Notice that some of the check boxes are colored with different colors, which shows that the curves were automatically selected, but also to represent the curve type by color.The Curves are colored by type (data units) as follows,

- Orange -  OHM-M or Resistivity Logs

- Cyan – PU or porosity Logs, Neutron Porosity, Density Porosity, etc.

- Greenish yellow – BARNS/E or Photoelectric Factor Logs

- Green – GM/CC or Bulk Density Log

- Forest Green – USEC/FT or the Acoustic Transit Time Log

- Red – API, PPM or “%” as Radioactive logs, Gamma Ray, Spectral Gamma Ray, etc.

- Blue – MD or Permeability Logs

- Brown – F, FT or IN or Depth

- Middle yellow – FRAC, or other log curve types.

- Dark Violet – UNI or Unknown Linear Curves

- Medium Violet – UNL or Unknown Logrithum Curves

The color coding of the selected curves were added to also help the user visually recognize that a curve was selected or not.

Map Curves & Change Curve Selections

Some logs will have curve mnemonics that are not recognized as one of the KGS “Standard” Curve Mnemonics.  The user will need to map the log curve to one of the KGS standard curves if they want to display the curve.  The first example is to map the Acoustic Transit Time (DT),which is labeled as “.uspf : WAVE DTC” log curve in the LAS File. Also notice that the button label “?(DTC)” is not recognized by the PROFILE web app.

Click on the “?(DTC)” Button to display the “Select KGS Standard Tools” Dialog.  This dialog provides a list of the KGS “Standard” Curve Mnemonics, from which the user can map an unrecognized log curve to one of the KGS standard curve mnemonics.  The KGS “Standard” Curve Mnemonics List was created as a way to standardize the alpha bit soup of Log Mnemonics.  Each logging company has their own curve mnemonics to represent similar tools.  The Profile program is a later version of code from the GEMINI Project LAS File Viewer Module, which needed to standardize the log curves so the curves could be automatically read and assigned a plot track.  The “LAS Tool Curve Mnemonics map to KGS Standard Mnemonics” XML File was created to map the log curves from logs that were part of the KGS LAS File Collection which is not a complete list of possible curve mnemonics.

To map the unknown curve mnemonic “?(DTC)” you first notice that the unit is “uspf” (micro seconds per foot) a unit of time.  Also the Acoustic Transit Time Curve Mnemonic is similar to the KGS “Standard” Curve Mnemonic “DT”.  By selecting the “?(DTC)” Button you will display the “Select KGS Standard Tools” Dialog.

Highlight the “DT” Mnemonic Row and click on the “Select” Button to map the “?(DTC)” Curve Mnemonic to “DT” KGS Standard Curve Mnemonic.

The “?(DTC). .uspf : WAVE DTC” has changed to “DT.usec/ft : Acoustic transit time”. We want to change the selected “ITTT” Log Curve to “DT” Log Curve.  The reason is that “ITTT” is the wrong curve type for the Acoustic Transit Time.  The program found the curve mnemonic as similar to the “DT” Standard Curve Mnemonic, but this curve will not plot correctly in the Profile Plot.  Just click on the green check box in front of the “ITTT” Mnemonic Button to deselect the curve and then click on the check box in front of the “DT” Mnemonic Button to select it.  Also notice that the ~Log_Definition Text Area was modified to show the change.

Moving the scroll bar down to the porosity curves, Neutron Porosity, and Density Porosity.

The LAS File Read will select the first curve that it recognizes and selects and color codes the curve.  In this case the Neutron porosity mnemonic selected is “NPHS”, which is a valid curve, but the “NPHI” curve is desired so like the Acoustic Transit Time, you can deselect the “NPHS” and then select the “NPHI” Curve.  Also the “DPHS” Density Porosity Curve can be deselected since the “RHOB” Bulk Density Curve has been selected.  The reason for deselecting the Density Porosity Curve, if the Bulk Density Curve is present, is to force the PROFILE program to recompute the Density Porosity using a Limestone Matrix.  If the Neutron Porosity, Bulk Density, Gamma Ray with/without a Photoelectric Factor Logs are present then the program will automatically compute a Lithology Composition Plot, but the Density Porosity has to be computed with a Limestone Matrix or the Lithology Composition Plot will not be computed corectly.

The above dialog represents the changes made for the neutron/density porosity logs. The last curves to be modified are the Array Induction Logs.  Haliburton uses a different curve mnemonic for these logs.  Move the scroll bar up to find the Array Induction Logs, RT90, RT60, etc.

Like the Acoustic Transit Time the “?(RT90).ohmm: 90in Resistivity 2ft Res” through “?(RT10).ohmm : 10in Resistivity 2ft Res” are not recognized.  These curves can be map to the “AHT90 Array Induction Resistivity-90” to “AHT10 Array Induction Resistivity-10” KGS Curves Respectively.  Click on the “?(RT90)” Mnemonic Button to display the “Select KGS Standard Tools” Dialog.

Highlight the AHT90 and click on the “Select” Button.

The “(?AHT90).ohmm : 90in Resistivity 2ft Res” has changed to “AHT90.OHM-M  : Array Induction Resistivity-90” and the orange check box is selected.  The rest of the Array Induction Log Curves each are mapped to the respective KGS Mnemonic Curve as follows,

(?RT90).ohmm : 90in Resistivity 2ft Res to AHT90.OHM-M  : Array Induction Resistivity-90 (?RT60).ohmm : 60in Resistivity 2ft Res to AHT60.OHM-M  : Array Induction Resistivity-60 (?RT30).ohmm : 30in Resistivity 2ft Res to AHT30.OHM-M  : Array Induction Resistivity-30 (?RT20).ohmm : 20in Resistivity 2ft Res to AHT20.OHM-M  : Array Induction Resistivity-20 (?RT10).ohmm : 10in Resistivity 2ft Res to AHT10.OHM-M  : Array Induction Resistivity-10

Select the Continue Button to read and parse the LAS log curves selected into the PROFILE Web App. Notice that the “Data Source Filenames:” Panel lists the LAS version 2.0 File that was just read in as well as the type of data, i.e. Log Data from LAS Data Type.

Importing PC Data – Tops CSV (Comma Separated Values) File.

Most of the web apps will use the same input dialogs to import tops CSV (Comma Separated Values) file.  The Load Data Dialog is basically the same for most of the Web Apps except they only load a subset of the total data types.  In this example a Tops CSV file is being imported into the web app.

Left Click on the “Tops Data” Icon Button in the Data Source Panel of the Load Data Dialog.  This will display the “Select Formation Tops Comma Delimited File from your PC” Dialog.  This dialog allows the user to search their PC for the file of interest.  In this example it is the Tops CSV file Wellington-KGS-1-32_Tops.csv, highlighted below. Select the Open button to display the “Map File Column Number to Region Column” Dialog.

The “Map File Column Number to Region Column” Dialog allows the user to map the file columns number to the web app tops data structure.  In this example the file has the well information in line one of the Tops CSV File and line two of the Tops CSV File has the file data columns.  In this case the chosen file columns match the Tops Mnemonics for the tops data structure.  The File Column Number is automatically assigned to the Region Column Names. The user only needs to select the “Load Data” Button to parse the Tops Data into the web app.

Tops CSV (Comma Separated Values) File Structure.

The Wellington KGS 1-32 Tops CSV example has two introduction lines, the first line is the well header information and the second line is the actual column labels for the tops data, illustrated below,

Figure: Partial Contents of the Wellington-KGS-1-32_Tops.csv File.

The “Map File Column Number to Region Column” Dialog allows the user to map the data in the Tops CSV File to the web app data structure variables.  The program first reads the first and second line of the CSV File looking for the data column headers.    The lines are each parsed to single out the data column headers and to match those headers to the tops data structure.  The program then assigns the column number to the Region Column Name starting at column 1,2,3, … if the file column name used matches the expected region column name.  The Column Names matrix used to parse the file column variables are listed below,

| Depth Top | Top | Start |
| --- | --- | --- |
| Depth Base | Base | End |
| Stratigraphic Unit Rank [ SYSTEM, GROUP, etc. ] | Rank |  |
| Stratigraphic Name | Name |  |
| Alternate Name | Alt Name |  |
| Era |  |  |
| System | Sys |  |
| Subsystem | subsys |  |
| Series | Ser |  |
| Subseries { Pennsylvanian & Mississippian Series } | Subseries | Subser |
| Stage | Stg |  |
| Group | Grp |  |
| Subgroup | subgrp |  |
| Formation | Form |  |
| Start Age (Ma) | Start Age |  |
| End Age (Ma) | End Age |  |

The Wellington KGS 1-32 Tops CSV File example above line 2 has only the Top, Tops Name, Rank, System, Subsystem, Series and Source as the column name variables.  The program was able to map each of the column headers to the tops data structure, except Source, i.e.

| Column | File Column Label | Tops Data Name |
| --- | --- | --- |
| 1 | Top | Depth Top |
| 2 | Name | Stratigraphic Name |
| 3 | Rank | Stratigraphic Unit Rank |
| 4 | System | System |
| 5 | Subsystem | Subsystem |
| 6 | Series | Series |
| 7 | Source |  |

When the user selects the “Load Data” Button on the “Map File Column Number to Region Column” Dialog the data is parsed into the Profile Program, where the Tops CSV file name is entered into the “PC ASCII Files:” Panel as well as the data type source.

Importing PC Data – Core CSV (Comma Separated Values) File.

Most of the web apps will use the same input dialogs to import Core Data CSV (Comma Separated Values) file.  The Load Data Dialog is basically the same for most of the Web Apps, except they only load a subset of the total data types.  In this example a Core Data CSV file is being imported into the web app.

Left Click on the “Rock Data” Icon Button in the Data Source Panel of the Load Data Dialog.  This will display the “Select Rock Data Comma Delimited File from your PC” Dialog.  This dialog allows the user to search their PC for the file of interest.  In this example it is the Core CSV file Wellington-KGS-1-32_Core_Data.csv, highlighted below. Select the Open button to display the “Map File Column Number to Rock Column” Dialog.

The “Map File Column Number to Rock Data Column” Dialog allows the user to map the file columns number to the web app tops data structure.  In this example the file has the well information in line one of the Core Data CSV File and line two of the Core Data CSV File has the file data columns.  In this case the chosen file columns match the Core Mnemonics for the core data structure.  The File Column Number is automatically assigned to the Rock Data Column Names. The user only needs to select the “Load Data” Button to parse the Core Data into the web app.

Core Data CSV (Comma Separated Values) File Structure.

The Wellington KGS 1-32 Core Data CSV example has two introduction lines, the first line is the well header information and the second line is the actual column labels for the core data, illustrated below,

Figure: Partial Contents of the Wellington-KGS-1-32_Core_Data.csv File.

The “Map File Column Number to Rock Data Column” Dialog allows the user to map the data in the Core Data CSV File to the web app data structure variables.  The program first reads the first and second line of the CSV File looking for the data column headers.    The lines are each parsed to single out the data column headers and to match those headers to the core data structure.  The program then assigns the column number to the Rock Data Column Name starting at column 1,2,3, … if the file column name used matches the expected rock data column name.  The Column Names matrix used to parse the file column variables are listed below,

| Mnem | Description | Mnem | Description |
| --- | --- | --- | --- |
| Depth Data |  | Density Data |  |
| TOP | Depth Top | GMCC | Grain Density (gm/cc) |
| BASE | Depth Bottom | RHOD | Density of Rock Dry (gm/cc) |
| CORR | Depth Correction | RHOW | Density of Rock Wet (gm/cc) |
| Stratigraphic & Environment Descriptions |  | Permeability Data |  |
| STU | Stratigraphic Unit | KMAX | Whole Core Permeability Maximum |
| STN | Stratigraphic Name | K90 | Whole Core Permeability 90 deg |
| ENV | Depositional Environment | KVRT | Whole Core Permeability Vertical |
| LITHO | Lithofacies | KPLG | Core Plug Permeability Routine |
| Porosity Data |  | KKL | Core Plug Permeability KL Routine |
| PCORE | Whole Core Porosity | KINSI | Core Plug Permeability Insitu |
| PPLUG | Core Plug Porosity Routine | KKLIN | Core Plug Permeability KL Insitu |
| P800 | Core Plug Porosity 800 PSI | KPVRT | Core Plug Permeability Vertical |
| PINSI | Core Plug Porosity Insitu | Archie Constants |  |
| PEFF | Effective Rock Porosity | MAMB | Archie Cementation Ambient |
| Saturation Data |  | MINSI | Archie Cementation Insitu |
| SOIL | Oil Saturation | NAMB | Archie Saturation Ambient |
| SW | Water Saturation | NINSI | Archie Saturation Insitu |
| Mnem | Description | Mnem | Description |
| Radioactive Data |  | Computed Data Types |  |
| GR | Gamma Ray | COMPUTED | Th/U Thorium/Uranium Ratio |
| CGR | Gamma Ray Minus Uranium | COMPUTED | Th/K Thorium/Potassium Ratio |
| PGR | Pseudo Gamma Ray | COMPUTED | Gamma Ray |
| THOR | Thorium Concentration | COMPUTED | Grain Density (gm/cc) |
| URAN | Uranium Concentration | COMPUTED | Porosity |
| Unknown Linear Data |  | Unknown Log Data |  |
| LIN_1 | Linear Track Curve 1 | LOG_1 | Semilog Track Curve 1 |
| LIN_2 | Linear Track Curve 2 | LOG_2 | Semilog Track Curve 2 |
| LIN_3 | Linear Track Curve 3 | LOG_3 | Semilog Track Curve 3 |
| LIN_4 | Linear Track Curve 4 | LOG_4 | Semilog Track Curve 4 |

The Wellington KGS 1-32 Core Data CSV File example above line 2 has only the Top, Base KMAX, K90, KVRT, GMCC, PCORE, SW and SOIL as the column name variables.  The program was able to map each of the column headers to the core data structure, i.e.

| Column | File Column Label | Core Data Name |
| --- | --- | --- |
| 1 | NO |  |
| 2 | TOP | Depth Top |
| 3 | - |  |
| 4 | BASE | Depth Bottom |
| 5 | KMAX | Whole Core Permeability Maximum |
| 6 | K90 | Whole Core Permeability 90 deg |
| 7 | KVRT | Whole Core Permeability Vertical |
| 8 | GMCC | Grain Density (gm/cc) |
| 9 | PCORE | Whole Core Porosity |
| 10 | SW | Water Saturation |
| 11 | SOIL | Oil Saturation |

When the user selects the “Load Data” Button on the “Map File Column Number to Rock Data Column” Dialog the data is parsed into the Profile Program, where the Core Data CSV file name is entered into the “PC ASCII Files:” Panel as well as the data type source.

There must be at least one Resistivity and one Porosity curve to run this program.  Notice that the Resistivity and the Porosity Log Curves are green in the “Log Curves / Files” table as a flag to the user that the necessary curves are present.  The user can now enter “Continue” Button to build the PfEFFER Spreadsheets.

PfEFFER Dialog

The “Load Data” Dialog is the entry to the PfEFFER Spreadsheets.  The user searches the KGS Database for well data and/or from the User’s PC.  The Image below suggests that the well data came from the KGS Database. Once the well data is loaded the “Continue” Button at the bottom of this dialog becomes enabled. Click on the “Continue” Button load the PfEFFER Dialog.

Only the “Headers” Tab is displayed until the user creates flow units, which will appear next to the “Headers” Tab. The header panel allows the user to edit well header information.

PfEFFER Panel Organization

A single “Excel Type” workbook is used to organize all the well log information for a particular well. We will refer to this workbook as a well workbook (even when the well is a dry hole). Each well workbook contains unit worksheets. A unit is a stratigraphic subdivision as defined by a user. So, a unit may be a formal geologic formation, a reservoir interval, or a reservoir subdivision. Logging data may be downloaded for multiple units from LAS File(s).

Tool Bar

Selected Values Table

Archie & Other Parameters Panel

| Variable | Description |
| --- | --- |
| Flow Unit | Name of flow unit |
| Depth Start | Minimum depth of flow unit interval |
| End Start | Maximum depth of flow unit interval |

| Archie Equation |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |

| Archie Equation Parameters |  |
| --- | --- |
| Variable | Description |
| A | Archie constant |
| M | Cementation exponent |
| N | Saturation exponent |
| Rw | Formation water resistivity at formation temperature |
| Rsh | Shale resistivity used in shaly sand computations |
| PHIsh | Shale porosity used in shaly sand computations |

| Archie Equation for Moveable Hydrocarbon |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |

| Moveable Hydrocarbon Parameters |  |
| --- | --- |
| Variable | Description |
| Rmf | Mud Filtrate Resistivity @ Temperature |
| Total Depth | Total Depth of Well |
| Surface Temp | Surface temperature [Default same as Mud Filtrate Temperature (Rmft)] |
| Bottom Temp | Bottom hole temperature |

| Cut-Offs |  |
| --- | --- |
| A zone is considered pay if PHI < Phi Cut; SW < Sw Cut; VSH < Vsh Cut; and BVW < Bvw Cut. If all these criteria are satisfied of oil (THK*PHI*(1-SW)). Otherwise the value of PAY is set to zero. The default values for these cut-offs are 0,1,1 and 1 (for PHI, SW, VSH and BVW, respectively). |  |
| Variable | Description |
| PHI Cut | Porosity Cut Off is not pay if PHI > Porosity cut off. |
| Sw Cut | Water Saturation Cut Off is not pay if SW < Water Saturation cut off. |
| Vsh Cut | V-Shale Cut Off is not pay if VSH < V-Shale cut off. |
| Bvw Cut | Bulk Volume Water Cut Off is not pay if BVW < Bulk Volume Water cut off. |

| Cumulative Values (Computed) |  |
| --- | --- |
| Variable | Description |
| CTHK | Cumulative unit thickness summed from THK Column |
| FTOIL | Cumulative feet (or meters) of oil (or gas) in PAY Column |
| PAYFEET | Cumulative feet (or meters) of section considered to be pay |
| AVPHI | Average Porosity (Φ), computed as a simple average of the porosities in PHI Column |
| AVSW | Average Water Saturation (Sw), computed as a simple average of the water saturations in SW Column |

| Wyllie-Rose Equation |  |
| --- | --- |
|  | P, Q and R: parameters of the Wyllie Rose equation used to estimate permeability (k) from porosity (Φ) and irreducible water. |

Home Area

| Home Area Columns |  |  |
| --- | --- | --- |
| Mnemonic | Name | Description |
| RT | True resistivity | Choice of resistivity log to be used is at the discretion of the user. |
| PHI | "True porosity" | PHI must contain fractional (as opposed to percentage) data and is generally equated with "true porosity". At the user’s discretion, PHI can be derived from a single porosity log (sonic, density, or neutron) stored in the log columns or computed as a function of several porosity logs. |
| RWA | Reconnaissance (apparent) water resisitivity | Apparent water resistivity for each zone, based on the Archie parameters and the zone porosity (PHI) and resistivity (RT) values. |
| RO | Resistivity of Water Saturated Zone | The resistivity of each zone if it was completely saturated with formation water whose resistivity matched the value recorded in cell Archie parameter RW and a rock with Archie parameters A and M. |
| MA | Apparent Archie cementation exponent | Apparent Archie cementation exponent is calculated under the assumption that the zone is completely water-saturated and that the Archie parameter value RW is the actual formation water resistivity. This computation is a companion technique to the RWA method, but in this case provides a means to estimate the cementation exponent in water zones where formation water resistivity is known, but the Archie equation parameters are poorly known. The values of MA also represent the slopes of an individual water line that result when the zone is located on the Pickett plot and linked with the formation water resistivity intercept. |
| SW | Proportional water saturation | The calculated values of proportional water saturation, using the zone porosity (PHI) and resistivity (RT) value in conjunction with the Archie equation values of a, m, n and Rw |
| BVW | Fractional bulk volume water | Calculation of the fractional bulk volume water in each zone as the product of the fractional porosity and water saturation. As described earlier in this manual, the BVW is often known as the "Buckles number" in reservoir zones at "irreducible" water saturation. The value is controlled primarily by pore size, where large pores cause low Buckles numbers, and small pores are matched by high Buckles numbers. |
| VSH | Fractional proportion of shale | The formula for the VSH column may be set automatically using the Vsh button on the PfEFFER toolbar |
| PAY | Incremental thickness of oil (or gas) | Incremental thickness of oil (or gas) in each zone, given by THK*PHI*(1-SW), assuming that the zone satisfies a number of criteria, described below. If a particular zone does not satisfy those criteria, then the PAY entry for that zone is 0, meaning that the zone does not contribute to the net thickness of oil computed in FTOIL |
| 1st PHI | Primary porosity | The primary porosity is determined from the interpolation of the observed bulk transit time between the matrix and fluid transit time extremes, as shown by the formula above. The total porosity column is set equal to column PHI, the porosity in the home area. |
| 2nd PHI | Secondary porosity | Secondary porosity is the total porosity (column PHI) minus the primary porosity determined from the sonic log. |
| PHIr | "Electrically connected porosity" | PHIr (connected porosity) is estimated from resistivity log as the square root of Reconnaissance (apparent) water resisitivity (RWA) divided by the Resistivity of Water Saturation Zone (RO). |
| RXO | Bulk resistivity of flushed zone | Choice of resistivity log to be used is at the discretion of the user. |
| SXO | Saturation of total moveable fluid (assumes filtrate has displaced everything moveable) | The calculated values of saturation of total moveable fluid, using the zone porosity (PHI) and bulk resistivity of flushed zone (Rxo) value in conjunction with the Archie equation values of a, m, n and Rmf |
| BVF | Bulk volume fluid | Calculation of the fractional bulk volume water in each zone as the product of the fractional porosity and saturation of total moveable fluid. Computed as the product of Sxo and PHI. |

Adding Flow Units

There are three methods for adding flow units to the PfEFFER Dialog, which can be accessed by the tool bar at the top of the PfEFFER Dialog as follows,

By User Defined Depth Ranges

Click on the icon image to display the "Add/Remove Flow Units" Dialog.

This dialog allows the user to add flow units to the PfEFFER "Spreadsheet" Dialog. The user can only add or remove the flow unit to or from the spreadsheet. If the user wishes to change the name or the depth range they only need to enter the values in the Archie and Other Parameters Panel on the PfEFFER "Spreadsheet" Dialog. At the top of Archie and Other Parameters Panel is the "Flow Unit:" text fields and the Depth Range text fields. The user can enter the changes directly and the spreadsheet will automatically reflect the depth range in the home area. The tab at the bottom will stay the same, but the name of the flow unit will change. When the user saves the session and returns the tab will reflect the new flow unit name.

Now enter the 1st flow unit, Morrill for flow name at depths 2975.5' to 2978.5'.  The “Add” button will only be enabled when all text fields are filled. To create the flow unit, select the “Add” button, which will add the flow unit to the “Well Flow Unit Calculations” Table.

The RVP2 column in the “Well Flow Unit Calculations” Table identifies the curves that were computed, i.e. R-True Resistivity, V-Shale Fraction, P-True Porosity, and 2-Second Porosity.  Notice that the Morrill Flow Unit has been added to the table. Also notice that a PfEFFER “Spreadsheet” was added in the PfEFFER Dialog with a Morrill tab to the right of the “Headers” tab with the home area columns are automatically filled as follows,

Rt – True Resistivity Hierarchy:			Rxo – Bulk Resistivity Hierarchy:

- ILD - Deep Induction Resistivity			1. SFLU – Spherically Focused Resistivity

- AHT90 - Array Induction Resistivity - 90	2. AHT10 – Array Induction Resistivity – 10

- LL - Deep Laterolog Resistivity			3. LL8 - Shallow Laterolog Resistivity

- RDEP - Deep Resistivity				4. RSHAL - Shallow Resistivity

- First Resistivity found				5. MSFL – Micro Spherically Focused Res.

t – True Porosity Hierarchy: 			Vsh – Fractional Shale Hierarchy:

- PHI Average value of 				1. GR - Gamma Ray

  - NPHI - Neutron Porosity

  - DPHI - Density Porosity

- NPHI - Neutron Porosity				2. CGR - Gamma Ray Minus Uranium

- DPHI - Density Porosity				3. SP - Spontaneous Potential

- SPHI - Sonic Porosity

Now enter the 2nd flow unit, Cottonwood for flow name at depths 2986.0' to 3003.0’.

To create the flow unit, select the “Add” button, which will add the flow unit to the “Well Flow Unit Calculations” Table.

Notice that a PfEFFER “Spreadsheet” was added in the PfEFFER Dialog with a Cottonwood tab to the right of the “Morrill” tab with the home area columns is automatically filled.

By Zonation a Depth-Constrained Cluster Analysis

The Zonation icon button launches a depth-constrained multivariate cluster analysis which segments the entire depth interval based on user-specified set of logs. A hierarchical cluster is used to produce subintervals that are as homogeneous as possible and distinct as possible from each other, in terms of their log characteristics. The software employs Ward's method which, at each step of the clustering process, joins the two groups (subintervals) whose merger produces the least possible increase in the total within-groups sum-of-squares. The sum-of-squares for a single group, k, is given by

where ||xi - xk||2 is the squared distance between the vector of the log values for data point i, xi, and the vector mean for group k, xk. The within-groups sum-of-squares increases equal the total sum-of-squares, T, given by the sum of the squared distances from all the data points to the global mean. At any step in the process, the relative amount of variation "explained" by the grouping can be measured by the value R2 = W/T. R2 increases with every merger and equals 100% when all the data points are joined in a single group.

The depth-constrained cluster analysis implemented in PfEFFER only allows vertically adjacent groups (subintervals) to be joined, greatly reducing the amount of computation relative to a more general cluster analysis, which would allow the possibility of joining any possible pair of groups at each step. The depth- constrained cluster analysis always produces a sequence of group memberships, represented as integers, beginning with 1 at the top of the interval and proceeding sequentially downward.

The "Zonation By Depth-Constrained Cluster Analysis" Dialog will display. The depth range panel initially displays the depth range of the log data. The user must set the Starting Depth and Ending Depth within the selected Log Curves or the log curves will be dropped when you select the "Compute" Button.

The stopping criterion can be specified either in terms of the number of groups (clustering will stop when the number of groups has be reduced to the number specified) or in terms of R2 (clustering will stop when the R2 equals or exceeds the specified value).

In the example above the Gamma Ray log curve and the Neutron Porosity log curve are selected. The Starting Depth will be from 2970 to the Ending Depth 3050. NOTE: The depth range must be within the depth range of all curves selected, i.e., Gamma Ray log curve is set from 2460.0 to 3160.5 and Neutron porosity log curve is set from 2342.0 to 3141.5. Select the "R-squared equals or exceeds" radio button and 25.0% will be used to compute the clusters. Now select the "Compute" Button. The Control & Plot - Cluster Analysis - Group Means" Dialogs will display

To Transfer the groups to the PfEFFER Spreadsheet select the "Save Groups" button on the "Control - Cluster Analysis - Group Means" Dialog and the created flow units will be transferred to the PfEFFER Dialog and automatically loaded, i.e.

Rt – True Resistivity Hierarchy:			Rxo – Bulk Resistivity Hierarchy:

- ILD - Deep Induction Resistivity			1. SFLU – Spherically Focused Resistivity

- AHT90 - Array Induction Resistivity - 90	2. AHT10 – Array Induction Resistivity – 10

- LL - Deep Laterolog Resistivity			3. LL8 - Shallow Laterolog Resistivity

- RDEP - Deep Resistivity				4. RSHAL - Shallow Resistivity

- First Resistivity found				5. MSFL – Micro Spherically Focused Res.

t – True Porosity Hierarchy: 			Vsh – Fractional Shale Hierarchy:

- PHI Average value of 				1. GR - Gamma Ray

  - NPHI - Neutron Porosity

  - DPHI - Density Porosity

- NPHI - Neutron Porosity				2. CGR - Gamma Ray Minus Uranium

- DPHI - Density Porosity				3. SP - Spontaneous Potential

- SPHI - Sonic Porosity

Group 1 through Group 5 tabs are added with the home area for each automatically loaded with default log curve selections.  The SXO and BVF columns are red because the values in those columns are not valid with respect to the Archie Default Parameters values.

References:

Pfeffer-Pro (Petrofacies Evaluation of Formation for Engineering Reservoirs), Kansas Geological Survey, Release Date February 1998. Zonation by Depth-Constrained Cluster Analysis pages 141 to 144.

Numerical zonation of log suites and logfacies recognition by multivariate clustering by Gill, D., Shomrony, A., and Fligelman, H., 1993, AAPG Bulletin, v. 17, no. 10, pp. 1781-1791.

By Tops

Click on the  icon button image to display the “Choose Flow Units From Tops Picks” Dialog, which will display with a list of tops check boxes with the tops Top and Base depths.  This dialog will only display if the user imported tops when the well data was loaded.

This dialog allows the users to build flow units from the top picks. The flow unit is selected by selecting the checkbox next to the tops name. The user can only pick a flow unit if Base Depth is greater than the Top Base, e.g., at depth 2832.0 feet Council Grove is disabled because the Top and Base Depth is equal to each other. The user can change the base depth to enable the checkbox so Council Grove can be selected. Scroll down to the bottom of the table. The dialog can be resized to the width of the user PC’s or move the scroll bar to the right down to access the tops.

In this example Morrill Limestone, Cottonwood Limestone and Neva Limestone checkboxes are selected. Select the "Save as Flow Units" Button to transfer the selected tops to the PfEFFER dialog, which will automatically create a spread sheet and load the log curve data for each flow unit selected, i.e.

Rt – True Resistivity Hierarchy:			Rxo – Bulk Resistivity Hierarchy:

- ILD - Deep Induction Resistivity			1. SFLU – Spherically Focused Resistivity

- AHT90 - Array Induction Resistivity - 90	2. AHT10 – Array Induction Resistivity – 10

- LL - Deep Laterolog Resistivity			3. LL8 - Shallow Laterolog Resistivity

- RDEP - Deep Resistivity				4. RSHAL - Shallow Resistivity

- First Resistivity found				5. MSFL – Micro Spherically Focused Res.

t – True Porosity Hierarchy: 			Vsh – Fractional Shale Hierarchy:

- PHI Average value of 				1. GR - Gamma Ray

  - NPHI - Neutron Porosity

  - DPHI - Density Porosity

- NPHI - Neutron Porosity				2. CGR - Gamma Ray Minus Uranium

- DPHI - Density Porosity				3. SP - Spontaneous Potential

- SPHI - Sonic Porosity

Morrill Limestone, Cottonwood Limestone (selected), and Neva Limestone tabs are added with the home area for each automatically loaded with default log curve selections.  The SXO and BVF columns are red because the values in those columns are not valid with respect to the Archie Default Parameters values.

Cut-Offs and Colors

The Cut-Offs Panel of the PfEFFER worksheet contain cut-off values which determine what is and is not considered pay. The labels for the cut-offs, are PHI Cut:, Sw Cut:, Vsh Cut: and Bvw Cut. A zone is considered pay if

PHI > PHI Cut 
SW < Sw Cut 
VSH < Vsh Cut 
BVW < Bvw Cut

If all these criteria are satisfied, then the PAY values for that zone is equal to the increment thickness of oil (THK * PHI * (1 - SW)). Otherwise the value of pay is set to zero. The default values for these cut-offs are 0, 1, 1, and 1 (for PHI, SW, VSH and BVW respectively). These values would imply that everything is considered pay, expect for zones with physically unreasonable values for one or more parameters (such as SW>1). The user may change the cut-offs simply by editing the cut-off values. The values in the PAY column will respond dynamically to these changes, as will the two summary values FTOIL and PAYFEET in the Cumulative Values (Computed).

Clicking on the Cut Off Colors Checkbox will activate color flagging of pay zones

When the Cut Off Colors Checkbox is checked, cells in the PAY column corresponding to pay zones will be colored yellow. In addition, cells in the PHI column with values less than PHI Cut will be colored light gray, cells with SW > Sw Cut will be cyan, cells with VSH > Vsh Cut will be light gray, and cells with BVW > Bvw Cut will be magenta. These colors will respond dynamically to changes in the cut-offs or to changes in the PHI, SW, VSH and BVW values themselves.

Modifying the Log Curve Selections

PfEFFER Toolbar has a set of buttons designed to ease the transfer of data from the log input columns right into the home area columns RT, VSH and PHI. The VSH button is placed before the PHI button because the VSH column may be employed in the computation of PHI and so the computation of VSH would logically come first.

Calculating RT

When the flow unit is created the spread sheet is automatically loaded using a hierarchy of resistivity curves.  If Deep Induction Resistivity (ILD) Log Curve is present it is automatically selected as the default True Resistivity (Rt).  If ILD is not present it looks for the next resistivity log curve by the following hierarchy,

Rt – True Resistivity Hierarchy:

- ILD - Deep Induction Resistivity

- AHT90 - Array Induction Resistivity - 90

- LL - Deep Laterolog Resistivity

- RDEP - Deep Resistivity

- First Resistivity found

If the user wishes to change the curve they only need to select the RT button on the Tool Bar, which will launch the “Rt Log Curve Selection” dialog, with a list of possible resistivity log curves loaded from the LAS file.

With this example there are 5 resistivity log curves available, highlight the Micro Normal Resistivity (MNOR) log curve and click on the “Select” Button to load the MNOR log data into the RT Column of the  Home Area and recomputed all the other columns.

Calculating VSH

When the flow unit is created the spread sheet is automatically loaded using a hierarchy of log curves that will illustrate change in Shale levels.  If Gamma Ray (GR) Log Curve is present it is automatically selected as the default V-Shale (VSH).  If GR is not present it looks for the next shale level log curve by the following hierarchy,

Vsh – Fractional Shale Hierarchy:

- GR - Gamma Ray

- CGR - Gamma Ray Minus Uranium

- SP - Spontaneous Potential

The VSH button is used to establish the formula for the VSH (shale proportion) column, based on any log curve the user has to decide the clean formation value and the shale value of the log curve selected.  Using the Gamma Ray as an example, the VSH will be computed as,

Vsh = (GR - GRc) / (GRsh - GRc)

Where GR is the observed gamma ray value and GRc and GRsh are the specified gamma ray values for clean formation and for shale formation respectively. The default values for GRc and GRsh for this formation are 10.428 API and 150.0 API respectively.  If the user wishes to change the curve they only need to select the VSH button on the Tool Bar, which will launch the “Vsh Log Selection” dialog, with a list of possible log curves loaded from the LAS file.

As you can see, this dialog is showing all the log curves downloaded from the LAS file.  The user can select any curve, but the user has to determine what is the clean formation level and shale formation level. Highlight the Spontaneous Potential (SP) and click on the “Select” button to display the “Vsh Data Limits: Spontaneous Potential” Dialog.

The Dialog will show the minimum and maximum value, but the user will have to enter the Clean Formation value and the Shale Value, e.g. set the Clean Formation Value to 80 and the Shale Value to 150. Now click on the “Ok” button to load the SP log data into the VSH Column of the Home Area and recomputed all the other columns depending on Vsh.

Calculating PHI

When the flow unit is created the spread sheet is automatically loaded using a hierarchy of Porosity log curves.  If Average Porosity can be calculated, both Neutron and Density Porosity curves are available) then the PHI is automatically is calculated as the default.  If both NPHI and DPHI are not both present it looks for the next porosity log curve by the following hierarchy,

t – True Porosity Hierarchy:

- PHI Average value of

  - NPHI - Neutron Porosity

  - DPHI - Density Porosity

- NPHI - Neutron Porosity

- DPHI - Density Porosity

- SPHI - Sonic Porosity

If the user wishes to change the curve they only need to select the PHI button on the Tool Bar, which will launch the “Phi Calculation - Log Selection” dialog, with a set of radio buttons of possible porosity curves to select.  If the Sonic log curve is not present then the Sonic radio button would be disabled, but as you can see in the image below that all possible porosity log curves are present.  The default button selected is None, but you must have a porosity curve so for this example the Density (gm/cc) radio button will be selected.

The four options correspond to the following formulas, with and without correction for shale volume,

|  | Without correction | With correction |
| --- | --- | --- |
| Density | (ρma - ρb) / (ρma - ρf) | [ρma - ρb - Vsh(ρma - ρsh)] / [ρma - ρf] |
| Neutron | Φn | Φn - Vsh * Φn sh |
| Density/Neutron | √[(Φn2 + Φd2) / 2] | [Φn - (Φn sh / Φd sh) * Φd] / [1 - Φn sh/Φd sh] |
| Sonic | (Δtma - Δtb) / (Δtma - Δtf) | [Δtma - Δtb - Vsh * (Δtma - Δtsh)] / [Δtma - Δtf] |

In each case you will be requested to specify columns containing the appropriate logs and also to specify the constants in the selected equation (such as ρma and ρf). You will also be asked whether you want to correct for Vsh, as described below. Indicate your choice of logs by selecting the appropriate radio button on the Log Selection dialog box and then clicking OK. Here we have chosen to compute the porosity from a density log.

You are now presented with the Parameter Values dialog box ...

which lets you specify the parameters in the selected equation, in this case matrix (or grain) density and fluid density (ρma and ρf). Since the Cottonwood is a limestone interval, we have entered 2.71 for the grain density above. The default value for fluid density is 1.0, which we will leave unchanged.

We could choose to include a Vsh correction by clicking on the “Correct for Vsh?” check box. Doing so will activate the Shale Density (RHOsh) edit box, allowing us to enter the needed value. The shale correction is based on the values in the VSH column. (If you select the Vsh correction prior to filling the VSH column, the formula for PHI will still be set up correctly, but the resulting porosity values will be null. The porosity values will be set to their proper numeric values once values are entered for VSH.) For now we will choose not to use the Vsh correction. The Parameter Values dialog box is similar for the other three porosity computation options, although it contains only the Vsh Correction box for the neutron porosity and density/neutron porosity options. After clicking OK on the Parameter Values dialog box, the program will set up the formula for the PHI column ...

Note that the Pickett plot computations expect porosity as a fraction, not in percent. Using the density and sonic options to compute porosity will yield fractional values automatically. PfEFFER-java automatically converts porosity logs that are in percent to fractional values when the LAS file is imported into the program.

Calculating RXO

When the flow unit is created the spread sheet is automatically loaded using a hierarchy of resistivity curves.  If Shallow Laterolog Resistivity (LL8) Log Curve is present it is automatically selected as the default Bulk resistivity of flushed zone (Rxo).  If LL8 is not present it looks for the next resistivity log curve by the following hierarchy,

Rt – True Resistivity Hierarchy:

- SFLU – Spherically Focused Resistivity

- AHT10 - Array Induction Resistivity - 10

- LL8 - Shallow Laterolog Resistivity

- RSHAL - Shallow Resistivity

- MSFL – Micro Spherically Focused Resistivity

If the user wishes to change the curve they only need to select the RXO button on the Tool Bar, which will launch the “Rt Log Curve Selection” dialog, with a list of possible resistivity log curves loaded from the LAS file.

With this example there are 5 resistivity log curves available, highlight the Micro Inverse Resistivity (MINV) log curve and click on the “Select” Button to load the MINV log data into the RXO Column of the  Home Area and recomputed all the Moveable Oil columns, SXO and BVF.

Selecting a Water Saturation Model

The Sw Model menu lets you choose between the Archie water saturation model (the default) and two shaly sand models, the Simandoux model and the Dual-water model. These options will be demonstrated using the Cottonwood Limestone worksheet. PfEFFER determines which model is currently in effect by checking the contents of "Water Model Used:" text field, which reads Archie whenever a new worksheet is created. The Archie model assumes that column RT contains true resistivity log data, the measured bulk resistivity. You switch to one of the shaly sand models by selecting the appropriate entry from the Sw Model menu in the Tool Bar at the top of the “spreadsheet”.

The values of the parameters displayed in the dialog box are those found in the appropriate cells on the current worksheet, Archie parameters for A, m, n, and Rw, Archie parameter Rsh (shale resistivity), and Archie parameter Phish (shale porosity), Rsh is required for the Simandoux  model and both Rsh & PHIsh are required for the Dual-water model. Changing the value of a parameter on the dialog box changes the corresponding value on the worksheet. If one of the cells on the worksheet is initially empty, then the corresponding text box on the dialog box will initially be empty as well. You must supply all the relevant parameters for a given model before calculations can proceed.

The Iteration settings are fixed at Maximum iterations of 100 and Maximum change of 0.0001 for the shaly sand models, which both use an iterative approach to solve the relevant equations:

Both shaly sand models can be written in the form

a*Swn + b*Sw + c = 0

For the Simandoux model,

a = Co = φm/(A*Rw) 
b = Csh = Vsh/Rsh 
c = -Ct = -1/Rt

and for the Dual-water model,

a = 1 
b = -W = -[Sb*(1-A*Rw/Rb)] 
c = -Ish = -A*Rw/(Rt* φtm)

and note that "a" here is distinct from the Archie parameter, denoted "A" above. This polynomial equation can be solved iteratively using Newton’s method, yielding the following iterative equation for Sw:

Sw(k+1) = Swk - (a*Swkn + b*Swk + c)/(n*a*Swkn-1+b)

where the subscript k is the iteration index. This iteration can be implemented by PfEFFER-Math  using a self-referential formula for each cell containing Sw. The code initializes the Sw values using the quadratic equation, i.e., assuming that n = 2. It then sets the formula for each Sw cell to the right- hand side of the equation above, where the Swk value is in fact a reference to the cell itself.

Selecting Simandoux Menu Option

The Rsh value in the “Archie Equation Parameters” Panel must not be 0.0 and for this example it is set to 5.0.  The user can select the icon image button to display the Profile plot and then turn on the LAS - Induction Resistivity Logs radio button and observe that the Deep Induction Resistivity shale value is about 5.

The user now selects the “Sw Model” Menu and then selects the “Simandoux” Menu Option and the program will automatically compute the resistivity and insert the new resistivity values in the RT column and recomputed the other columns in the Home Area.

Selecting Dual Water Menu Option

The Rsh value in the “Archie Equation Parameters” Panel must not be 0.0 and for this example it is set to 5.0 and the PHIsh value in the “Archie Equation Parameters” Panel must not be 0.0 and for this example it is set to 0.1.  The user can select the icon image button to display the Profile plot and then turn on the LAS - Induction Resistivity Logs radio button and observe that the Deep Induction Resistivity shale value is about 5. In the Shale region the Neutron porosity is about 0.2 PU and the Density porosity is about 0.02 PU and the average is about 0.1005 rounding down to 0.1.

The user now selects the “Sw Model” Menu and then selects the “Dual water” Menu Option and the program will automatically compute the resistivity and insert the new resistivity values in the RT column and recomputed the other columns in the Home Area.

Reference

Pfeffer-Pro (Petrofacies Evaluation of Formation for Engineering Reservoirs), Kansas Geological Survey, Release Date February 1998. Selecting a Water Saturation Model, pages 101 to 105.

Profile Plot Control Dialog & Profile Plot

To display the profile plot of the log & pfeffer data click on the icon image button.  The “Profile Plot Control” dialog allows the user to change the presentation of the Profile Plot, by depth range, by depth scale, by data type, by log type, modify the track curve limits.

When the user first launches the Profile Plot the depth range of the panel that is displayed will automatically set the depth range of the profile plot except when the “Headers” tab is displayed, which will display the whole log. The user has two profile plot presentations, log data and pfeffer data. The pfeffer profile plot displays the contents of the “spreadsheet” with the selected flow units highlighted in a pale yellow across the whole pfeffer profile plot.

Figure: Newby 2-28R - LAS Icon Button

Figure: Newby 2-28R - PfEFFER Icon Button

Data Entry Dialogs

The “Change Plot Limits” Button on the Profile Control Dialog allows the user to change the limits of the log curves.

Notice that the color boxes next to the curve limits of the curves are colored with different colors, which shows the curve type by unit.The Curves are colored by type (data units) as follows,

- Orange -  OHM-M or Resistivity Logs

- Cyan – PU or porosity Logs, Neutron Porosity, Density Porosity, etc.

- Greenish yellow – BARNS/E or Photoelectric Factor Logs

- Green – GM/CC or Bulk Density Log

- Forest Green – USEC/FT or the Acoustic Transit Time Log

- Red – API, PPM or “%” as Radioactive logs, Gamma Ray, Spectral Gamma Ray, etc.

- Blue – MD or Permeability Logs

- Brown – F, FT or IN or Depth

- Middle yellow – FRAC, or other log curve types.

- Dark Violet – UNI or Unknown Linear Curves

- Medium Violet – UNL or Unknown Logrithum Curves

The color coding is the same as the “LAS File Curve Sections” Dialog that helps the user distinguish the type of curves available at a glance.

The user can change as many curves as they wish, understanding that each plot curves are grouped together, i.e. Porosity.  The Resistivity curves are grouped by Plot Track so that if you change the Micro Normal Resistivity (MNOR) and Micro Inverse Resistivity (MINV) the Induction Resistivity Curves will not automatically change unless the user wishes.

The above changes above are entered, i.e.

- The Photoelectric factor (PE) curve is changed from “0.0 to 20.0” to 0.0 to 10.0”

- The Neutron Porosity (NPHI) curve is changed from “-0.1 to 0.3” to “0.0 to 0.5” which also modifies the Density Porosity (DPHI) and the Sonic Porosity (SPHI) to the same limits.

As noted in the image the Profile plot is automatically modified as the user makes changes to each text field.  View the “Litho-Density” Plot track (see image below) reflects the changes made to the plot curves.

Pickett & Hingle Plots

To display the Pickett & Hingle Plots click on the  icon image button.  The Pickett & Hingle “Plot Control” dialog allows the user to change the presentation of the Pickett & Hingle Plots.

Tool Bar

Modify Plot Titles

Modify Plot Axes

Connect Data Points

Filter Data before points are connected

Filter Data after points are connected

Modify Archie Equation Parameters

### The slope of the Water Saturation (Sw), Bulk Volume Water (Bvw), and Permeability (K) Plot lines are affected by the Archie Parameters. Two examples are provided below, which show two distinct slopes and different interpretation for the data. The Hingle & Pickett Plots do not automatically determine the correct Archie Parameters, but requires that the user has some understanding of the plot data and geology.

## Default 100% Water Saturation (Sw) Plot Line

## Modified 100% Water Saturation (Sw) Plot Line

Filter By Data

The “Filter By Data” Table is used to filter the data in the Hingle & Pickett Plots. The Minimum and Maximum data values table to the right is the default values. The log data modifies the default minimum and maximum values for the Depth, Apparent Grain Density, Apparent Acoustic Time, and Apparent Photoelectric values.

| Default Table Values |  |  |  |  | Fluid Values |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Mnemonic | Description | Minimum | Maximum |  | Fresh | Saline | Oil Mud | Units |
| Depth | Depth Range |  |  |  |  |  |  | feet (ft) |
| Rt | Total Resistivity | 0.1 | 1000.0 |  |  |  |  | ohm-m |
| Vsh | V-Shale | 0.0 | 1.0 |  |  |  |  |  |
| PHIt | Total Porosity (Φ) | 0.0 | 1.0 |  |  |  |  | PU |
| GR | Gamma Ray | 0.0 | 150.0 |  |  |  |  | API |
| RHOmaa | Apparent Grain Density (ρmaa) | 2.5 | 3.2 |  | 1.0 | 1.1 | 0.985 | gm/cc |
| DTmaa | Apparent Acoustic Time (Δtmaa) | 35.0 | 65.0 |  | 189.0 | 185.0 | 204.5 | usec/ft |
| Umaa | Apparent Photoelectric (Umaa) | 2.0 | 15.0 |  | 0.398 | 1.36 | 0.136 | usec/ft |

Change Depth Range

Change Apparent Grain Density

Modify Plot Sw, Bvw, K Lines

Water Saturation (Sw) Plot Lines

Adding the Water Saturation (Sw) Plot Lines

Bulk Volume Water (Bvw) Plot Lines

Adding the Bulk Volume Water (Bvw) Plot Lines

Permeability (K) Plot Lines

Adding the Permeability (K) Plot Lines

Modify Plot Data Colors

Initial Data Point Color – Black

Default Button: RHOmaa-Umaa-GR Data Limits – Use Default Limits

Filter Button: RHOmaa-Umaa-GR Data Limits – Set using Filter Data Limits

Depth Button: Evenly distribute 40 colors over depth range

Modifying the Log Curve Selections

PfEFFER Toolbar has a set of buttons designed to ease the transfer of data from the log input columns right into the home area columns RT, VSH and PHI. The VSH button is placed before the PHI button because the VSH column may be employed in the computation of PHI and so the computation of VSH would logically come first.

Calculating RT

When the flow unit is created the spread sheet is automatically loaded using a hierarchy of resistivity curves.  If Deep Induction Resistivity (ILD) Log Curve is present it is automatically selected as the default True Resistivity (Rt).  If ILD is not present it looks for the next resistivity log curve by the following hierarchy,

Rt – True Resistivity Hierarchy:

- ILD - Deep Induction Resistivity

- AHT90 - Array Induction Resistivity - 90

- LL - Deep Laterolog Resistivity

- RDEP - Deep Resistivity

- First Resistivity found

If the user wishes to change the curve they only need to select the RT button on the Tool Bar, which will launch the “Rt Log Curve Selection” dialog, with a list of possible resistivity log curves loaded from the LAS file.

With this example there are 5 resistivity log curves available, highlight the Micro Normal Resistivity (MNOR) log curve and click on the “Select” Button to the resistivity log curve data selected will be downloaded to the Pickett and Hingle Plots. Notice also that the Total Resistivity Axis title is modified on the Control dialog as well as the both plots.

Calculating VSH

When the flow unit is created the spread sheet is automatically loaded using a hierarchy of log curves that will illustrate change in Shale levels.  If Gamma Ray (GR) Log Curve is present it is automatically selected as the default V-Shale (VSH).  If GR is not present it looks for the next shale level log curve by the following hierarchy,

Vsh – Fractional Shale Hierarchy:

- GR - Gamma Ray

- CGR - Gamma Ray Minus Uranium

- SP - Spontaneous Potential

The VSH button is used to establish the formula for the VSH (shale proportion) column, based on any log curve the user has to decide the clean formation value and the shale value of the log curve selected.  Using the Gamma Ray as an example, the VSH will be computed as,

Vsh = (GR - GRc) / (GRsh - GRc)

Where GR is the observed gamma ray value and GRc and GRsh are the specified gamma ray values for clean formation and for shale formation respectively. The default values for GRc and GRsh for this formation are 10.428 API and 150.0 API respectively.  If the user wishes to change the curve they only need to select the VSH button on the Tool Bar, which will launch the “Vsh Log Selection” dialog, with a list of possible log curves loaded from the LAS file.

As you can see, this dialog is showing all the log curves downloaded from the LAS file.  The user can select any curve, but the user has to determine what is the clean formation level and shale formation level. Highlight the Spontaneous Potential (SP) and click on the “Select” button to display the “Vsh Data Limits: Spontaneous Potential” Dialog.

The Dialog will show the minimum and maximum value, but the user will have to enter the Clean Formation value and the Shale Value, e.g. set the Clean Formation Value to 80 and the Shale Value to 150. Now click on the “Ok” button to load the SP log data into the Pickett & Hingle Plots.

Calculating PHI

When the flow unit is created the spread sheet is automatically loaded using a hierarchy of Porosity log curves.  If Average Porosity can be calculated, both Neutron and Density Porosity curves are available) then the PHI is automatically is calculated as the default.  If both NPHI and DPHI are not both present it looks for the next porosity log curve by the following hierarchy,

t – True Porosity Hierarchy:

- PHI Average value of

  - NPHI - Neutron Porosity

  - DPHI - Density Porosity

- NPHI - Neutron Porosity

- DPHI - Density Porosity

- SPHI - Sonic Porosity

If the user wishes to change the curve they only need to select the PHI button on the Tool Bar, which will launch the “Phi Calculation - Log Selection” dialog, with a set of radio buttons of possible porosity curves to select.  If the Sonic log curve is not present then the Sonic radio button would be disabled, but as you can see in the image below that all possible porosity log curves are present.  The default button selected is None, but you must have a porosity curve so for this example the Density (gm/cc) radio button will be selected.

The four options correspond to the following formulas, with and without correction for shale volume,

|  | Without correction | With correction |
| --- | --- | --- |
| Density | (ρma - ρb) / (ρma - ρf) | [ρma - ρb - Vsh(ρma - ρsh)] / [ρma - ρf] |
| Neutron | Φn | Φn - Vsh * Φn sh |
| Density/Neutron | √[(Φn2 + Φd2) / 2] | [Φn - (Φn sh / Φd sh) * Φd] / [1 - Φn sh/Φd sh] |
| Sonic | (Δtma - Δtb) / (Δtma - Δtf) | [Δtma - Δtb - Vsh * (Δtma - Δtsh)] / [Δtma - Δtf] |

In each case you will be requested to specify columns containing the appropriate logs and also to specify the constants in the selected equation (such as ρma and ρf). You will also be asked whether you want to correct for Vsh, as described below. Indicate your choice of logs by selecting the appropriate radio button on the Log Selection dialog box and then clicking OK. Here we have chosen to compute the porosity from a density log.

You are now presented with the Parameter Values dialog box ...

which lets you specify the parameters in the selected equation, in this case matrix (or grain) density and fluid density (ρma and ρf). Since the Cottonwood is a limestone interval, we have entered 2.71 for the grain density above. The default value for fluid density is 1.0, which we will leave unchanged.

We could choose to include a Vsh correction by clicking on the “Correct for Vsh?” check box. Doing so will activate the Shale Density (RHOsh) edit box, allowing us to enter the needed value. The shale correction is based on the values in the VSH column. (If you select the Vsh correction prior to filling the VSH column, the formula for PHI will still be set up correctly, but the resulting porosity values will be null. The porosity values will be set to their proper numeric values once values are entered for VSH.) For now we will choose not to use the Vsh correction. The Parameter Values dialog box is similar for the other three porosity computation options, although it contains only the Vsh Correction box for the neutron porosity and density/neutron porosity options. After clicking OK on the Parameter Values dialog box, the program will set up the formula for the PHI column and transfer the Total Porosity Data to the Hingle & Pickett Plots. Notice also that the Total Porosity Axis title is modified on the Control dialog as well as the both plots. The Porosity Constants will also be modified.

Note that the Pickett plot computations expect porosity as a fraction, not in percent. Using the density and sonic options to compute porosity will yield fractional values automatically. PfEFFER-java automatically converts porosity logs that are in percent to fractional values when the LAS file is imported into the program.

Moveable Hydrocarbon Plot

A moveable hydrocarbon plot (referred to hereafter simply as a moveable oil plot) can be created using the  image icon button provided that a shallow resistivity log is available on the unit worksheet. The Sxo computations are based on the assumption that the zone near the well is permeated with mud filtrate. The Archie equation is used to compute Sxo, the filtrate saturation of this flushed zone based on the mud filtrate hydrocarbon saturations. We will illustrate the Sxo computation using the Cottonwood Limestone worksheet.

The Moveable Hydrocarbon Panel displays the mud filtrate Values as well as the total depth and temperature values.

In order to modify these values you must go to the Headers Tab at the bottom of the worksheet and scroll to the bottom page to the Resistivity measurements in the Parameters section. Any changes to the mud filtrate values will be automatically modified in the Moveable Hydrocarbon Panel for all flow units.

The Moveable Hydrocarbon Panel displays the Mud Filtrate Resistivity @ Temperature along with the Total Depth @ Bottom Temperature degrees Fahrenheit. The Surface Temperature is assumes the mud filtrate temperature unless it is specifically available in the log.

To modify these values you must select the Headers Tab at the bottom of the worksheet and scroll to the bottom of the page where the Resistivity measurements are provided. If the Resistivity or Temperatures are not present then the following values are used by default,

| Variable | Value | Units | Description |
| --- | --- | --- | --- |
| Rmf | 0.55 | ohm-m | Mud Filtrate Resistivity |
| Rmft | 58.0 | oF | Mud Filtrate Temperature |
| ST | 58.0 | oF | Surface Temperature |
| BHT | 120.0 | oF | Bottom Hole Temperature |
| TD |  | Feet | Total Depth of Hole |

The TD is determined automatically from the LAS File when it is imported into PfEFFER-java.

If you wish to enter of modify go to the "Value" Column in the Headers Column next to the variable you wish to change, the "Description" Column will describe the variable. Changing any of the variables above will automatically be reflected in every flow panel. To change the Temperature units you only need to change one of the units for the temperature to reflect all the temperature variables assuming that all were measured with the same units. The only accepted values are DEG-F for Fahrenheit and DEG-C for Celsius.

If the values of RMF, RMFT and TMAX are not displayed in the “Headers” tab “spreadsheet” then it is not present and the default will be used to compute the moveable oil.  To find the necessary measured values the user will need to go to the Headers Section of original electric log and enter the measured values from the log. The example header image for the Newby 2-28R was retrieved from the KGS Database & Server. Notice that the RMF, RMFT and TMAX are different than the default values above.

| Variable | Value | Units | Description |
| --- | --- | --- | --- |
| Rmf | 0.66 | ohm-m | Mud Filtrate Resistivity |
| Rmft | 74.0 | oF | Mud Filtrate Temperature |
| ST | 74.0 | oF | Surface Temperature |
| BHT | 101.0 | oF | Bottom Hole Temperature |
| TD | 3162.0 | Feet | Total Depth of Hole |

KGS has about 67,684 Kansas wells in its database as of 05 December 2014 (reference http://www.kgs.ku.edu/PRS/wellStats.html ).

To retrieve the electric log in Kansas, the user can search the Kansas Geological Survey Database (http://www.kgs.ku.edu/PRS/petroDB.html) for logs by clicking on the “Master list of oil and gas wells” and searching by Township-Range-Section, Partial Lease Name, or API-Number, i.e.

The RMF, RMFT and BHT are used to compute Rmfc, the corrected mud filtrate resistivity at the formation temperature, T. This is given by,

Rmfc = Rmf * ( Rmft + 6.77 ) / ( T + 6.77 ) Fahrenheit

Rmfc = Rmf * ( Rmft + 21.5 ) / ( T + 21.5 ) Celsius

where T = ST + depth * ( BHT - ST ) / TD 

         T = Formation Temperature 
         ST = Surface Temperature (Default is Rmft) 
         depth = depth within the flow unit 
         BHT = Bottom Hole Temperature 
         TD = Total Depth of Hole

PfEFFER-java automatically computes the temperature from the formation depth (interpolated from the surface and the bottom hole temperature) and then loads the SXO and BVF columns.

Now Click on the icon button will display the moveable oil plot.

The plot, BVW is represented as blue, BVF as green, and PHI as red. The differences between BVF and BVW represent the moveable hydrocarbon saturation, and the difference between PHI and BVF represents the residual hydrocarbon saturation.

Reference

Pfeffer-Pro (Petrofacies Evaluation of Formation for Engineering Reservoirs), Kansas Geological Survey, Release Date February 1998. Moveable Hydrocarbon Plot, pages 131 to 134.

Save Well Data as Log ASCII Standard (LAS) version 3.0 File

The LAS (Log ASCII Standard) is rapidly becoming the accepted industry standard for electronic transmission of digital wire-line logs. Earlier digital formats were commonly coded in binary (such as LIS) and so required specialized software to read them. The LAS standard was introduced by the Canadian Well Logging Society (http://www.cwls.org/) in 1989 to standardize the organization of digital log curve information for personal computer users. It did this very successfully and the standard became popular worldwide. Version 1.2 was the first version and was followed in September 1992 by version 2.0 to address some inconsistencies. A more versatile version LAS 3.0 was released in 1999 however at present LAS 2.0 remains the dominant product. LAS 3.0 clarify several of the poorly defined specifications of LAS 2.0 and provide expanded data storage capabilities, but have seen limited implementation.

The GEMINI Tools programs will read either a Log ASCII Standard (LAS) version 2.0 or 3.0 file and version 1.2 but the Well Information Section is backward in data definition and will not be parsed correctly in the GEMINI Tools web apps.

The sections defined for the LAS 2.0 standard are as follows (http://www.cwls.org/wp-content/uploads/2014/09/LAS_20_Update_Jan2014.pdf):

- "~V" (also known as "~VERSION INFORMATION SECTION") is a required section; has formatting requirements; must be the first section; identifies the version number and whether data is in "wrapped" or "un-wrapped" mode.

- "~W" (also known as "~WELL INFORMATION SECTION") is a required section; has formatting requirements; is preferably the second section; contains information on the well name, location, and start and stop values of the data in this file.

- "~C" (also known as ~CURVE INFORMATION SECTION") is a required section; has formatting requirements; contains curve mnemonics and their definitions in the order that they appear in the data section.

- "~P" (also known as ~PARAMETER INFORMATION SECTION") is an optional section; has formatting requirements; contains information on parameters or constants relevant to the wellbore such as mud resistivity, wire line engineer, truck number, elevation data, etc.

- "~O" (also known as "~OTHER") is an optional section; has no formatting requirements; contains other information or comments.

- "~A" (also known as ~ASCII LOG DATA") is a required section; has formatting requirements; is the last section in the file and also referred to as the data section. The index of the data columns is either Depth or Time. The index values always appear in the first column and each column of data must be separated by at least one space (ASCII 32). All values in the ASCII log data section must be floating point or integer (long) values. Other formats such as Text or Exponential values are not supported.

LAS 3.0 (http://www.cwls.org/wp-content/uploads/2014/09/LAS_3_File_Structure.pdf) will be used to save the well data for the GEMINI Tools web apps since it can hold all the well data in one file. You can even think of LAS 2.0 as a subset of LAS 3.0 since the LAS 2.0 is only concerned with the LOG Data. Note this section includes some of the referenced LAS 3 File Structure PDF; see the above URL for the complete LAS 3.0 File structure.

The ~Version and ~Well sections must appear in every LAS 3.0 file as the first and second sections respectively.

Other sections are grouped by data type. Each group consists of two or three sections; a Parameter Data section (optional for all but Log data), a Column Definition section, and a Column Data section, in that order.

For example, core analysis data would have the following three sections:

~Core_Parameter
~Core_Definition
~Core_Data.

At least one group or data type of either the defined LAS 3.0 data types or a user defined type must exist in every LAS 3.0 file.

The Column Definition and the Column Data sections for each data type are matched sets and must both appear in that order. The corresponding Parameter Data section is optional (except for Log data), but if used must appear before the corresponding Column Definition Section.

LAS 3.0 defines six specific well related data types and their root Section Title names. They are:

~Ascii or ~Log
~Core
~Inclinometry
~Drilling
~Tops
~Test

Additional data types can be defined by the user and content rules discussed elsewhere in the document may define other section titles.

Stand alone user defined Parameter Data sections can be included. Care must be taken to use standalone Parameter Data sections only when the data contained does not fit into any of the other defined data types.

When used, the section order of each set of the three sections for each data type must be Parameter, Definition, and then Data.

Blank lines and comment lines can appear within Column Data sections, but can only appear BEFORE the first Column Data line of that section, or after the LAST Column Data line of that section.

The names of each channel can optionally appear above each channel as a comment line immediately before, after or on the section title line of that section if space allows.

Note: Do not use the ~Other section recognized by LAS version 2.0. It is no longer allowed in LAS 3.0. Any data that can be stored in this section must now be stored properly in a user defined Parameter Data or Column Data section.

### The LAS version 3.0 file has the potential to hold all the well data that was collected, i.e. multiple log data files, core data, tops data, DST data, Perforation data, Cuttings Report data, etc.  As an example the Newby 2-28R has log, core, tops, perforation data as well as the PfEFFER data created from the log analysis tool, PfEFFER-java.  You can view the file at the following URL addresses,

### LAS 3.0 Example File: http://www.kgs.ku.edu/Gemini/Tools/documentation/Newby-2-28R_LAS3.las

### LAS 3.0 in WinZip File: http://www.kgs.ku.edu/Gemini/Tools/documentation/Newby-2-28R_LAS3.zip

### Standard LAS 3.0 Data Sections for the Newby-2-28R_LAS3.las

- ~Version - Version Section

- ~Well - Well Header Information Section

- ~Log - Log Data Section – for single log data file, see LAS 3.0 PDF reference below for multiple log data runs.

  - ~Parameter

  - ~Curve

  - ~ASCII, which must be the last section in the LAS File if the Log data is present.

- ~Tops - Formation Top (Stratigraphic Units) Picks Data Section

  - ~Tops_Parameter

  - ~Tops_Definition

  - ~Tops_Data | Tops_Definition

- ~Perforation – Perforation Data Section

  - ~Perforation_Parameter

  - ~Perforation_Definition

  - ~Perforation_Data | Perforation_Definition

### GEMINI Tools Defined LAS 3.0 Data Sections for the Newby-2-28R_LAS3.las

- ~IQ_Control - Recreate the Profile Plot Data Section

  - ~IQ_Control_Parameter

  - ~IQ_Control_Definition

  - ~IQ_Control_Data | IQ_Control_Definition

- ~IQ_Flow – This data holds the geologist cuttings report/core description

  - ~ IQ_Flow _Parameter

  - ~ IQ_Flow _Definition

  - ~ IQ_Flow | IQ_Flow_Definition

- ~IQ_Pfeffer – This data holds the file location of Core Image JPEG images.

  - ~ IQ_Pfeffer _Parameter

  - ~ IQ_Pfeffer _Definition

  - ~ IQ_Pfeffer | IQ_Pfeffer_Definition

- ~IQ_Las_Parameter - Selected LAS Curves – This Parameter Section was designed to remember the Log Curves selected by the user so the user does not have to map LAS File Curve Mnemonics to KGS Standard Tool Mnemonics when they run this file with other GEMINI Tools.

The Tabs at the top identifies the data that will be saved to the Log ASCII Standard (LAS) version 3.0 File.  This example saves the well info, log data, core data, brine data, tops data, profile plot control data, cuttings/core description and core image file locations on the KGS Server. The “LAS File Data Types” Dialog allows the user to modify the well header data that is being saved to the LAS version 3.0 file.  Also noted in the two images below are identified “required” fields for a valid LAS file.  The GEMINI Tools Web Apps doesn’t care that the LAS file is not valid, just that the data follows the basic rules for the well data sections in the LAS 3.0 file.

Save Cottonwood Flow Unit as a Comma Separated Values (CSV) File

To save the contents of a flow unit “spreadsheet” as a Comma Separated Values (CSV) File to be imported into Excel or another log analysis program, click on the  icon image button when the flow unit “spreadsheet” you wish to save as a CSV file is displayed, which will display the “Enter Directory Path & Filename” dialog, which the user can search their PC for the directory they wish to save the CSV file in.

Notice that the name of the flow unit appears in the “Enter Filename” text field with a “.csv” appended to it. You can change the Filename to anything you wish, e.g. Cottonwood_Limestone.csv will be altered to be Newby-2-28R-Cottonwood_Limestone.csv

Click on the “Continue” Button to save the contents of the flow unit “spreadsheet” as a CSV File. The following URL address will display the CSV file,

### LAS 3.0 Example File: http://www.kgs.ku.edu/Gemini/Tools/documentation/Newby-2-28R_Cottonwood_Limestone.csv

### LAS 3.0 in WinZip File: http://www.kgs.ku.edu/Gemini/Tools/documentation/Newby-2-28R_Cottonwood_Limestone.zip

### The contents of the above csv file can be imported directly into Excel as illustrated below,