LAS File Viewer Java Applet                                                                 by John R. Victorine

Introduction

The LAS File Viewer web app has 2 sources for importing well data, 1) the user’s PC or 2) the Kansas Geological Survey (KGS) Server & ORACLE Database.  This program allows the user to import Log, Tops, Core, and Geologist Cuttings Report/Core Description/Measured Sections Data.

There will be occasions when the user would like to inspect the contents of a LAS file. So, for example, the user may wish to find the identity of the well, the types of logs, depth range, well or log parameters. Alternatively, the user can suspect that the file is not coded correctly in LAS format if the Import LAS function fails, and wishes to examine the file to troubleshoot the problem. The LAS file is an ASCII file and can be read by any text editor, i.e. Notepad, WordPad, TextPad, etc. The CSV (Comma Separated Values) files are also ASCII and can be edited in the same way or with Microsoft Excel.

To access LAS File Viewer go to http://www.kgs.ku.edu/stratigraphic/LAS/.  At the top of the web page there is a menu "Main Page|Description|Applet|Help|Copyright & Disclaimer|".  Select the "Applet" menu option a "Warning - Security" Dialog will appear. The program has to be able to read and write to the user’s PC and access the Kansas Geological Survey (KGS) Database and File Server, ORACLE requires this dialog. The program does not save your files to KGS, but allows you to access the KGS for well information that may be missing in your Kansas logs. The program does not use Cookies or any hidden software it only reads the LAS and CSV files for the LAS File Viewer Session and writes a LAS 3.0 File to your PC to save your LAS File Viewer Session and the well data imported. The blue shield on the warning dialog is a symbol that the Java web app is created by a trusted source, which is the University of Kansas.  Select the "Run" Button, which will show the LAS File Viewer "Enter" Panel illustrated below,

# Contents

Loading Well Data   …………………………..………………………………………………………………………………………………….. 4 Data Source Panel	………………………………………………………………………………………………………………………... 4 Data Loaded Panel ………………………………………………………………………………………………………………………... 5

Importing KGS (Database & Server) Data …….…………………………………………………………………………………….… 6 Importing Well Data ……………………………………………………………………………………………………………………... 6   * LAS File Data …………………………………………………..………………………………………………………………………….  9 - Map Curves & Change Curve Selections …………………………..…….……………………….………………………... 11 * Tops Picks ………………………………………………………………………………….….………………………………………….. 16 - MKD Source Example ………………………………………………………………………………………………………………… 17 * Core Data …………………………………………………………………………………………………………………………………. 20 Importing Measured Sections ……………………………………………………………………………………………………… 23

Importing PC Data – Download Well Data from PC …….……………………………………………………………………… 28 Log ASCII Standard (LAS) version 2.0 File …………………………………………………………………………………….. 29 - Map Curves & Change Curve Selections …………………..………………………………………………………….……. 31     Tops CSV (Comma Separated Values) File …………………………………………………………………………….……… 42      - Tops CSV (Comma Separated Values) File Structure ………………………………………………………………….. 43     Core CSV (Comma Separated Values) File ……………………………………………………………………………………. 46      - Core Data CSV (Comma Separated Values) File Structure ………………………………………………………….. 48     Geologist Report (Cuttings/Core Descriptions, Measured Sections) ASCII Delimited File …………….. 51       - Geologist Report ASCII Delimited File Structure ………………………………………………………………………… 53        -- By Depth Range ………………………………………………………………………………………………………………………… 53        -- By Bedding Thickness ……………………………………………………………………………………………………………….. 53

LAS File Viewer Plot Control Dialog & Plot ………………………………………………………………………………….……… 58

Save well Data as Log ASCII Standard (LAS) version 3.0 File …………...…………………………………………………. 61

Data Entry Dialogs ………………………………………………………………………………………………………………………………. 68 Edit Header Information Dialog …………………………………………………..………………………………………………. 68 Change the Plot Track Limits ……………………………………………………………………………………………………….. 76 Changing the Colorlith – Porosity Imager Linear & Nonlinear Color Schema Tracks ……………...…….. 79 Adding & Modifying Tops/Sequence Stratigraphy/Depositional Environment …………………….………. 81   * Enter Horizon Data - Stratigraphic Units Panel ……………………………………………………………………….…. 84      - Add Lecompton Formation to the Stratigraphic Units List ……………………………………………….……..…. 86     - Add Unknown Bed (SG-A Bed) to the Stratigraphic Units List …………………………………………………….. 89    - Modify data for the Kanwaka Shale Formation in the Stratigraphic Units List …………….…………..…. 91

Quick Plot Buttons ………………………………………………………………………………………………………………..……..…….. 93 Plot Track Type ………………………………………………………………………………………………………………………..….. 93    - Default …………………………………………………………………………………………………………………………………....… 93    - Log/Colorlith ……………………………………………………………………………………………………………………………... 95      - Log Only …………………………………………………………………………………………………………………………………….. 96    - Georeport ………………………………………………………………………………………………………………..……………….. 97        Log Data Type ………………………………………………………………………………………………………………………..……. 98    - Litho/Phi ………………………………………………………………………………………………………………………………..….. 98    - Resistivity .……………………………………………………………………………………………………………………………….... 99    - Sonic …………………………………………………………………………………………………………………………………….…. 100    - Spectral Gamma Ray ……………………………………………………………………………………………………………….. 101    - Gamma Ray ……………………………………………………………………………………………………………………………… 102

Loading Well Data

Click the "LAS File Viewer Enter” Icon Button, which will show the "Load Data" Dialog. The dialog below displays an example of the Wellington KGS 1-32 well data loaded from the PC Data icon buttons with the data in the tables above.  The icon buttons in the Data Source Panel assists the user in loading well data into the LAS File Viewer Applet.

Data Source Panel

The Data Source Panel provides two methods of importing data into the LAS File Viewer Web App.  The Kansas Geological Survey (KGS) Database & File Server and the user’s PC.  A number of icon buttons are provided to assist the user in importing the specific data type of interest. When the user selects the icon button a search dialog is provided specific to the data type.  The CSV (Comma Separated Values) icon buttons under the “PC ASCII Delimited Data Files” Panel are expecting a general type of data presentation.  Although the order of the specific data columns is not important, the “Mnemonics” of the data column is.  Each data type in GEMINI Tools web apps have a data mnemonic list that will be presented later as each icon search dialog is presented. The CSV Search Dialog will use the first two lines of the CSV file to automatically match the file column data mnemonics with the web app curve mnemonics, but if the program does not recognize the file data mnemonic then it will leave it blank and expect the user to match the file data mnemonic to the web app curve mnemonics, this will be explained later.

Data Loaded Panel

The Data Loaded Panel provides a visual feedback that the data type was loaded, by the file names of the files that were accessed to load the data and by the data type that is loaded.  The data type is important in that it shows where the data came from.  The KGS data has the ORACLE Database from which the Tops Data, and Core Data, etc. are retrieved from a XML (Extensible Markup Language) data stream that is constructed using the ORACLE PL/SQL for each data type.  The user’s PC will load the data from CSV (Comma Delimited Values) files or a delimited file for the Geologist Report.  The LAS File can be downloaded automatically from the KGS Server in the program or from the user’s PC.  This program allows the user to import up to 3 Log ASCII Standard (LAS) version 2.0 or 3.0 files.  The LAS version 3.0 file can hold all the well data, but if the user wishes to add log curves from a LAS version 2.0 File it is advised that the LAS version 3.0 file be loaded last.  In most cases the user is importing multiple LAS version 2.0 files, which can be saved into a single LAS version 3.0 file.

Importing KGS (Database & Server) Data

KGS (Database & Server) - Importing Well Data

The Kansas Geological Survey (KGS) has a good collection of well data stored in the ORACLE Database and File Server as Files Log ASCII Standard (LAS) version 3.0 Files.  In this example the user will download the well data available from the KGS, Log data (LAS version 2.0 File), Tops Data, Measured Core Data, and Perforation Data.  The ORACLE Database is accessed by making Stored Procedure PL/SQL calls to the ORACLE Database from which an Extensible Markup Language (XML) data stream is created containing the well data that is passed back to the web app making the request.

Left Click on the “Well Data” Icon Button in the Data Source Panel of the Load Data Dialog.

This will display the “Search for Data on KGS Server” Dialog, see above image.  This dialog allows the user to search the KGS database for well data.  In this example, the well of interest will be the Newby 2-28R, this well contains all the well data that can be retrieved from the KGS Database, i.e. Log Data (LAS version 3.0 File), Tops Data, Core Data, and Perforations.

As the Summary image suggests there are 3 methods for searching for the well data within this dialog,

- By API-Number – KGS has a specific format for the API-Number, i.e.SS-CCC-99999 where SS is the state code for Kansas 15, CCC is the county code for Newby 2-28R it is 189 for Stevens County and the 5-Digit Well Number for Newby 2-28R it is 22225.

- By Partial Lease Name – The stored procedure used to retrieve the well header information allows the user to enter a partial phrase, in this example Newby.  The program places a ‘%’ in front and back of the phrase and sends the request to the Database, i.e. “%Newby%”.

- By Township Range Section – This search is by location in Kansas, this search also allows the user to enter just the Township and Range to search for wells, e.g. to look for the Newby 2-28R, enter Township as 31 set the S (South) Radio button and Range as 37 set the W (West) Radio button.

The user only needs to enter the above data and select the “Search” Button to display the list of Wells in the Kansas Database that match the search criteria.  In the image below the Lease Name “Newby” was entered to search for all wells in Kansas with the Phrase Newby in it. The user searches through the list until they find the well of interest.  In this example it is the Newby 2-28R, which is highlighted.

Notice that the LTCI represents the type of data that the well contains.  It is a visual aid that lets the user see what is available before trying to download the data.  If you require a LAS file you would want to see an L in that column.  The LTCI labels stand for the following,

- L – Log ASCII Standard (LAS) version 2.0 Files

- T – Tops Data ( Stratigraphic Unit Horizons )

- C – Measured Core Data

- I – Core Joint Photographic Experts Group (JPEG) Image Files

This dialog allows the user to now download each of the data types that are available.

Load KGS Well Data – LAS File Data

The “Search for Data on KGS Server” Dialog allows the user to download data from the KGS Database & Server to the web app.  The “LAS File Data” Button will display the “Select LAS File” Dialog with a list of LAS version 2.0 Files that are available.

In this example there are three LAS files available, with a table suggesting the log data type in the file.  In the beginning of the GEMINI Project (2000-2003) a precursor to the GEMINI Tools the KGS populated the Tool Types from every log that was in the KGS Server at that time.  Unfortunately KGS has not maintain that table for wells uploaded after 2003 so the LAS File may have “No” for all the log types, which is not accurate.  The user will need to open or download the file or search for the Well on the KGS Master List of Oil and Gas Wells in Kansas Web Page (http://www.kgs.ku.edu/Magellan/Qualified/index.html) to see what is in the File Header before deciding to download data from this program.  For this example the first log has most of the data necessary except the Spectral Gamma Ray Logs.  The Table above identifies the following log types,

- OHM-M – Resistivity Logs

- Neutron – Neutron Porosity Log

- Density – Bulk Density and/or Density Porosity Log

- Sonic – Acoustic Transit Time and/or Sonic Porosity Log

- GR – Gamma Ray (API units) Log

- PE – Photoelectric Factor Log

- THOR – Thorium Concentration

- URAN – Uranium Concentration

- POTA – Potassium Concentration)

In this example the first log contains the data needed, highlight the first log and click on the “Select” Button to display the “LAS File Curve Sections” Dialog. The “LAS File Curve Sections” Dialog allows the user to map unknown LAS Curve Mnemonics to the KGS “Standard” Curve Mnemonics so they will be plotted in the LAS File Viewer Plot.  This program reads the “LAS Tool Curve Mnemonics map to KGS Standard Mnemonics” XML File (http://www.kgs.ku.edu/software/gemini/data/las_standard_tools.xml ), which will automatically maps the Curve Mnemonics from the LAS file to one of 31 KGS “Standard“ Curve Mnemonics.

As you can see this log has all the log types of interest, Gamma Ray API, Resistivity, Neutron/Density,  Photoelectric Factor, Sonic and Permeability.  If a curve Mnemonic is not recognized the program will place a “?” in front of the Mnemonic, e.g. “?(MPERM16)” for the “. : Curve # 51” Log Curve.  If the user is satisfied with the automatic curve selections, which are checked and color coded, they only need to select the “Continue” Button at the bottom of the Dialog to import the file. The next section will take the user through a series of examples in changing the curve selections and mapping unknown curve mnemonics.

Notice that some of the check boxes are colored with different colors, which shows that the curves were automatically selected, but also to represent the curve type by color.The Curves are colored by type (data units) as follows,

- Orange -  OHM-M or Resistivity Logs

- Cyan – PU or porosity Logs, Neutron Porosity, Density Porosity, etc.

- Greenish yellow – BARNS/E or Photoelectric Factor Logs

- Green – GM/CC or Bulk Density Log

- Forest Green – USEC/FT or the Acoustic Transit Time Log

- Red – API, PPM or “%” as Radioactive logs, Gamma Ray, Spectral Gamma Ray, etc.

- Blue – MD or Permeability Logs

- Brown – F, FT or IN or Depth

- Middle yellow – FRAC, or other log curve types.

- Dark Violet – UNI or Unknown Linear Curves

- Medium Violet – UNL or Unknown Logrithum Curves

The color coding of the selected curves were added to also help the user visually recognize that a curve was selected or not.

Map Curves & Change Curve Selections

Some logs will have curve mnemonics that are not recognized as one of the KGS “Standard” Curve Mnemonics.  The user will need to map the log curve to one of the KGS standard curves if they want to display the curve.  Slide the scroll bar down to the Permeability Curves MPERM and ?(MPERM16).

Click on the “?(MPERM16)” Button to display the “Select KGS Standard Tools” Dialog.  This dialog provides a list of the KGS “Standard” Curve Mnemonics, from which the user can map an unrecognized log curve to one of the KGS standard curve mnemonics.  The KGS “Standard” Curve Mnemonics List was created as a way to standardize the alpha bit soup of Log Mnemonics.  Each logging company has their own curve mnemonics to represent similar tools.  The LAS File Viewer program is a later version of code from the GEMINI Project LAS File Viewer Module, which needed to standardize the log curves so the curves could be automatically read and assigned a plot track.  The “LAS Tool Curve Mnemonics map to KGS Standard Mnemonics” XML File was created to map the log curves from logs that were part of the KGS LAS File Collection which is not a complete list of possible curve mnemonics.

To map the unknown curve mnemonic “?(MPERM16)” you first notice that it similar to the MPERM curve above, which is a permeability curve.  In this example both curves will be plotted together, but the Permeability Plot Track can only plot one curve.  The web app has the ability to allow the user to plot up to 4 unknown logrithum curves and 4 unknown linear curves.  The permeability is usually plotted as logrithmic.  Click on the “?(MPERM16)” Button to display the “Select KGS Standard Tools” Dialog.  Slide the scroll bar down to the “LOG_1” Mnemonic – Semilog Track Curve 1 and highlight that curve.  Click on the “Select” Button to map the “?(MPERM16) to the Semilog Track Curve 1.

The ?(MPERM16) Curve has been changed to MPERM16 removing the ?() around the Curve Mnemonic.  Also select the check box next to it, which changes to a dark violet.

In order to plot both Permeability Curves on the same track, the MPERM must be assigned to the unknown Log Curves with MPERM16.  Click on the MPERM Mnemonic Button to display the

“Select KGS Standard Tools” Dialog.  Slide the scroll bar down to the “LOG_2” Mnemonic – Semilog Track Curve 2 and highlight that curve.  Click on the “Select” Button to map the “MPERM to the Semilog Track Curve 2.

Click the check box next to the MPERM Mnemonic Button you will notice that it has the same color as MPERM16, which now groups the two curves together.  Select the “Continue” Button to load the LAS File into the web app.

Load KGS Well Data – Top Picks

The “Search for Data on KGS Server” Dialog allows the user to download data from the KGS Database & Server to the web app.  The “Top Picks” Button will display the “Move/Merge KGS Data” Dialog with available top picks grouped by the sources creating the tops.

This dialog allows the user to add all or some the tops from each of the sources.  Both tables are set up so the user can use the “Ctrl” Key with the left click of mouse to select multiple tops, i.e.

Notice that the only some of the tops are selected.  You can then select the Add Button to move only the selected tops to the “User’s Stratigraphic Units” Table.

MKD Source Example:

Once the list of tops are in the “User’s Stratigraphic Units” Table the user can edit the list by removing any duplicate or invalid tops.  Notice that the “Council Grove” Top occurs 2 times in the list.  Highlight the one of the “Council Grove” tops.

Now select the “Remove” Button.

The MKD-07/2006 has only one top, so this dialog allows the user to add that top to the “User’s Stratigraphic Units” Table.  The MKD does not have this top and this is an extra top missing from the MKD data set.

Now with the data set complete select the “Load Data” Button to import the Tops data into the web app.

Load KGS Well Data – Core Data

The “Search for Data on KGS Server” Dialog allows the user to download data from the KGS Database & Server to the web app.  The “Core Data” Button will automatically load any measured core data that is in the KGS Database and import directly into the web app.

As the user accepted each data type the “Data Source Filenames:” Panel in the Load Data Dialog changes. The LAS File that was downloaded from the KGS Server to the LAS File Viewer Web App is added to the “Log ASCII Standard (LAS) Files” panel will show the filename downloaded.  The Log Data, Perforations, Tops Data, Measured Core Data, and the Geologist Report Data Type have been downloaded from KGS.

Figure: Example LAS File Viewer Plot of Newby 2-28R well data downloaded from the KGS Database and Server, which includes Log Data, Perforations, Tops Data, Measured Core Data with Core Descriptions and Core Images.

KGS (Database & Server) - Importing Measured Sections

The Kansas Geological Survey (KGS) has a good collection of Measured Section data stored in the ORACLE Database.  In this example the user will download the measured sections that are available, for each section will contain Measured Sections Data and Fossil Genera Names (Bio-Stratigraphy) and in some cases top picks.  The ORACLE Database is accessed by making Stored Procedure PL/SQL calls to the ORACLE Database from which an Extensible Markup Language (XML) data stream is created containing the measured section data that is passed back to the web app making the request.

To Display the “Search for KGS Measured Sections” left click the mouse on the “Measured Sections” Icon Button in the Data Source Panel of the Load Data Dialog.

This will display the “Search for KGS Measured Sections” Dialog, see above image.  This dialog allows the user to search the KGS database for measured sections data.  In this example, the well of interest will be the Marshall #1 this well contains all the brine samples that can be retrieved from the KGS Database.

As the Summary image suggests there are 3 methods for searching for the well data within this dialog,

- By Partial Directions Phrase – The stored procedure used to retrieve the list of measured sections from the KGS Database allows the user to enter a partial phrase in this example “line” is entered.  The program places a ‘%’ in front and back of the phrase and sends the request to the Database, i.e. “%line%”.

- By Township Range Section – This search is for measured sections in Kansas by, e.g. to look for the 3212 Measured Section, enter Section as 24, Township as 15 set the S (South) Radio button and Range as 4 set the E (East) Radio button.

- By County – This panel allows the user to search for measured sections by counties with measured section database table, e.g. select the DICKINSON County Button.

The user only needs to enter the above data and select the “Search” Button to display the list of measured sections in the Kansas Database that match the search criteria.  In the image below the Lease Name “line” was entered to search for all measured sections in Kansas with the Phrase “line” in it. The user searches through the list until they find the well of interest.

The “Load Data” will display the selection on the panel showing the data type by location of data. Notice that the Geologist Report Type is selected.  There are tops within the Measured Section Report, but the Tops are within the report and not part of the query process so this panel will not reflect that the tops are there.

Figure 1: KGS Measured Section (3212) "Center of Eastern Line" Township 15S - Range 4E - Section 24.

Importing PC Data - Download Well Data to PC

Download either the ASCII Text Files directly or the Zip files extracting the contents into a directory.  The problem with the ASCII Text Files being downloaded directly from a web page is that the web page will alter the contents so it does not retain the basic structure and add HTML text to the file.  The preferred method if you have Zip or WinZip is to download the zip files to your PC and extract.

Well Data: Wellington KGS 1-32, Sumner County, Kansas

| Type | ASCII Text Files |
| --- | --- |
| LAS 2.0 | http://www.kgs.ku.edu/Gemini/Tools/documentation/Wellington-KGS-1-32.las |
| Tops | http://www.kgs.ku.edu/Gemini/Tools/documentation/Wellington-KGS-1-32_Tops.csv |
| Core | http://www.kgs.ku.edu/Gemini/Tools/documentation/Wellington-KGS-1-32_Core_Data.csv |
| Report | http://www.kgs.ku.edu/Gemini/Tools/documentation/Wellington-KGS-1-32_geo.txt |

| Type | Zip Files |
| --- | --- |
| LAS 2.0 | http://www.kgs.ku.edu/Gemini/Tools/documentation/Wellington-KGS-1-32.zip |
| Tops | http://www.kgs.ku.edu/Gemini/Tools/documentation/Wellington-KGS-1-32_Tops.zip |
| Core | http://www.kgs.ku.edu/Gemini/Tools/documentation/Wellington-KGS-1-32_Core_Data.zip |
| Report | http://www.kgs.ku.edu/Gemini/Tools/documentation/Wellington-KGS-1-32_geo.zip |

Measured Section: C S Line, Township 16S-Range 7E-Section 10, Riley County, Kansas

| Type | ASCII Text Files |
| --- | --- |
| Report | http://www.kgs.ku.edu/Gemini/Tools/documentation/C_S_line_S10-T6S-R7E.txt |

| Type | Zip Files |
| --- | --- |
| Report | http://www.kgs.ku.edu/Gemini/Tools/documentation/C_S_line_S10-T6S-R7E.zip |

Importing PC Data – Log ASCII Standard (LAS) version 2.0 File

Most of the web apps will use the same input dialogs to import Log ASCII Standard (LAS) version 2.0 or 3.0 files.  The Load Data Dialog is basically the same for most of the Web Apps, except they only load a subset of the total data types.  In this example a LAS version 2.0 file is being imported into the web app.

Left Click on the “LAS File” Icon Button in the Data Source Panel of the Load Data Dialog.  This will display the “Select LAS File from your PC” Dialog.  This dialog allows the user to search their PC for the file of interest.  In this example it is the LAS version 2.0 file Wellington-KGS-1-32.las, highlighted below. Select the Open button to display the “LAS File Curve Sections” Dialog.

The “LAS File Curve Sections” Dialog allows the user to map unknown LAS Curve Mnemonics to the KGS “Standard” Curve Mnemonics so they will be plotted in the LAS File Viewer Plot.  This program reads the “LAS Tool Curve Mnemonics map to KGS Standard Mnemonics” XML File (http://www.kgs.ku.edu/software/gemini/data/las_standard_tools.xml ), which will automatically maps the Curve Mnemonics from the LAS file to one of 31 KGS “Standard“ Curve Mnemonics.  If a curve Mnemonic is not recognized the program will place a “?” in front of the Mnemonic, e.g. “?(BSZ)” for the “.in : Bit Size” Log Curve.  If the user is satisfied with the automatic curve selections, which are checked and color coded, they only need to select the “Continue” Button at the bottom of the Dialog to import the file. The next section will take the user through a series of examples in changing the curve selections and mapping unknown curve mnemonics.

Notice that some of the check boxes are colored with different colors, which shows that the curves were automatically selected, but also to represent the curve type by color.The Curves are colored by type (data units) as follows,

- Orange -  OHM-M or Resistivity Logs

- Cyan – PU or porosity Logs, Neutron Porosity, Density Porosity, etc.

- Greenish yellow – BARNS/E or Photoelectric Factor Logs

- Green – GM/CC or Bulk Density Log

- Forest Green – USEC/FT or the Acoustic Transit Time Log

- Red – API, PPM or “%” as Radioactive logs, Gamma Ray, Spectral Gamma Ray, etc.

- Blue – MD or Permeability Logs

- Brown – F, FT or IN or Depth

- Middle yellow – FRAC, or other log curve types.

- Dark Violet – UNI or Unknown Linear Curves

- Medium Violet – UNL or Unknown Logrithum Curves

The color coding of the selected curves were added to also help the user visually recognize that a curve was selected or not.

Map Curves & Change Curve Selections

Some logs will have curve mnemonics that are not recognized as one of the KGS “Standard” Curve Mnemonics.  The user will need to map the log curve to one of the KGS standard curves if they want to display the curve.  The first example is to map the Acoustic Transit Time (DT),which is labeled as “.uspf : WAVE DTC” log curve in the LAS File. Also notice that the button label “?(DTC)” is not recognized by the LAS FILE VIEWER web app.

Click on the “?(DTC)” Button to display the “Select KGS Standard Tools” Dialog.  This dialog provides a list of the KGS “Standard” Curve Mnemonics, from which the user can map an unrecognized log curve to one of the KGS standard curve mnemonics.  The KGS “Standard” Curve Mnemonics List was created as a way to standardize the alpha bit soup of Log Mnemonics.  Each logging company has their own curve mnemonics to represent similar tools.  The LAS File Viewer program is a later version of code from the GEMINI Project LAS File Viewer Module, which needed to standardize the log curves so the curves could be automatically read and assigned a plot track.  The “LAS Tool Curve Mnemonics map to KGS Standard Mnemonics” XML File was created to map the log curves from logs that were part of the KGS LAS File Collection which is not a complete list of possible curve mnemonics.

To map the unknown curve mnemonic “?(DTC)” you first notice that the unit is “uspf” (micro seconds per foot) a unit of time.  Also the Acoustic Transit Time Curve Mnemonic is similar to the KGS “Standard” Curve Mnemonic “DT”.  By selecting the “?(DTC)” Button you will display the “Select KGS Standard Tools” Dialog.

Highlight the “DT” Mnemonic Row and click on the “Select” Button to map the “?(DTC)” Curve Mnemonic to “DT” KGS Standard Curve Mnemonic.

The “?(DTC). .uspf : WAVE DTC” has changed to “DT.usec/ft : Acoustic transit time”. We want to change the selected “ITTT” Log Curve to “DT” Log Curve.  The reason is that “ITTT” is the wrong curve type for the Acoustic Transit Time.  The program found the curve mnemonic as similar to the “DT” Standard Curve Mnemonic, but this curve will not plot correctly in the LAS File Viewer Plot.  Just click on the green check box in front of the “ITTT” Mnemonic Button to deselect the curve and then click on the check box in front of the “DT” Mnemonic Button to select it.  Also notice that the ~Log_Definition Text Area was modified to show the change.

Moving the scroll bar down to the porosity curves, Neutron Porosity, and Density Porosity.

The LAS File Read will select the first curve that it recognizes and selects and color codes the curve.  In this case the Neutron porosity mnemonic selected is “NPHS”, which is a valid curve, but the “NPHI” curve is desired so like the Acoustic Transit Time, you can deselect the “NPHS” and then select the “NPHI” Curve.  Also the “DPHS” Density Porosity Curve can be deselected since the “RHOB” Bulk Density Curve has been selected.  The reason for deselecting the Density Porosity Curve, if the Bulk Density Curve is present, is to force the LAS FILE VIEWER program to recompute the Density Porosity using a Limestone Matrix.  If the Neutron Porosity, Bulk Density, Gamma Ray with/without a Photoelectric Factor Logs are present then the program will automatically compute a Lithology Composition Plot, but the Density Porosity has to be computed with a Limestone Matrix or the Lithology Composition Plot will not be computed corectly.

The above dialog represents the changes made for the neutron/density porosity logs. The last curves to be modified are the Array Induction Logs.  Haliburton uses a different curve mnemonic for these logs.  Move the scroll bar up to find the Array Induction Logs, RT90, RT60, etc.

Like the Acoustic Transit Time the “?(RT90).ohmm: 90in Resistivity 2ft Res” through “?(RT10).ohmm : 10in Resistivity 2ft Res” are not recognized.  These curves can be map to the “AHT90 Array Induction Resistivity-90” to “AHT10 Array Induction Resistivity-10” KGS Curves Respectively.  Click on the “?(RT90)” Mnemonic Button to display the “Select KGS Standard Tools” Dialog.

Highlight the AHT90 and click on the “Select” Button.

The “(?AHT90).ohmm : 90in Resistivity 2ft Res” has changed to “AHT90.OHM-M  : Array Induction Resistivity-90” and the orange check box is selected.  The rest of the Array Induction Log Curves each are mapped to the respective KGS Mnemonic Curve as follows,

(?RT90).ohmm : 90in Resistivity 2ft Res to AHT90.OHM-M  : Array Induction Resistivity-90 (?RT60).ohmm : 60in Resistivity 2ft Res to AHT60.OHM-M  : Array Induction Resistivity-60 (?RT30).ohmm : 30in Resistivity 2ft Res to AHT30.OHM-M  : Array Induction Resistivity-30 (?RT20).ohmm : 20in Resistivity 2ft Res to AHT20.OHM-M  : Array Induction Resistivity-20 (?RT10).ohmm : 10in Resistivity 2ft Res to AHT10.OHM-M  : Array Induction Resistivity-10

Select the Continue Button to read and parse the LAS log curves selected into the LAS FILE VIEWER Web App. Notice that the “Data Source Filenames:” Panel lists the LAS version 2.0 File that was just read in as well as the type of data, i.e. Log Data from LAS Data Type.

Importing PC Data – Tops CSV (Comma Separated Values) File.

Most of the web apps will use the same input dialogs to import tops CSV (Comma Separated Values) file.  The Load Data Dialog is basically the same for most of the Web Apps except they only load a subset of the total data types.  In this example a Tops CSV file is being imported into the web app.

Left Click on the “Tops Data” Icon Button in the Data Source Panel of the Load Data Dialog.  This will display the “Select Formation Tops Comma Delimited File from your PC” Dialog.  This dialog allows the user to search their PC for the file of interest.  In this example it is the Tops CSV file Wellington-KGS-1-32_Tops.csv, highlighted below. Select the Open button to display the “Map File Column Number to Region Column” Dialog.

The “Map File Column Number to Region Column” Dialog allows the user to map the file columns number to the web app tops data structure.  In this example the file has the well information in line one of the Tops CSV File and line two of the Tops CSV File has the file data columns.  In this case the chosen file columns match the Tops Mnemonics for the tops data structure.  The File Column Number is automatically assigned to the Region Column Names. The user only needs to select the “Load Data” Button to parse the Tops Data into the web app.

Tops CSV (Comma Separated Values) File Structure.

The Wellington KGS 1-32 Tops CSV example has two introduction lines, the first line is the well header information and the second line is the actual column labels for the tops data, illustrated below,

Figure: Partial Contents of the Wellington-KGS-1-32_Tops.csv File.

The “Map File Column Number to Region Column” Dialog allows the user to map the data in the Tops CSV File to the web app data structure variables.  The program first reads the first and second line of the CSV File looking for the data column headers.    The lines are each parsed to single out the data column headers and to match those headers to the tops data structure.  The program then assigns the column number to the Region Column Name starting at column 1,2,3, … if the file column name used matches the expected region column name.  The Column Names matrix used to parse the file column variables are listed below,

| Depth Top | Top | Start |
| --- | --- | --- |
| Depth Base | Base | End |
| Stratigraphic Unit Rank [ SYSTEM, GROUP, etc. ] | Rank |  |
| Stratigraphic Name | Name |  |
| Alternate Name | Alt Name |  |
| Era |  |  |
| System | Sys |  |
| Subsystem | subsys |  |
| Series | Ser |  |
| Subseries { Pennsylvanian & Mississippian Series } | Subseries | Subser |
| Stage | Stg |  |
| Group | Grp |  |
| Subgroup | subgrp |  |
| Formation | Form |  |
| Start Age (Ma) | Start Age |  |
| End Age (Ma) | End Age |  |

The Wellington KGS 1-32 Tops CSV File example above line 2 has only the Top, Tops Name, Rank, System, Subsystem, Series and Source as the column name variables.  The program was able to map each of the column headers to the tops data structure, except Source, i.e.

| Column | File Column Label | Tops Data Name |
| --- | --- | --- |
| 1 | Top | Depth Top |
| 2 | Name | Stratigraphic Name |
| 3 | Rank | Stratigraphic Unit Rank |
| 4 | System | System |
| 5 | Subsystem | Subsystem |
| 6 | Series | Series |
| 7 | Source |  |

When the user selects the “Load Data” Button on the “Map File Column Number to Region Column” Dialog the data is parsed into the LAS File Viewer Program, where the Tops CSV file name is entered into the “PC ASCII Files:” Panel as well as the data type source.

Importing PC Data – Core CSV (Comma Separated Values) File.

Most of the web apps will use the same input dialogs to import Core Data CSV (Comma Separated Values) file.  The Load Data Dialog is basically the same for most of the Web Apps, except they only load a subset of the total data types.  In this example a Core Data CSV file is being imported into the web app.

Left Click on the “Rock Data” Icon Button in the Data Source Panel of the Load Data Dialog.  This will display the “Select Rock Data Comma Delimited File from your PC” Dialog.  This dialog allows the user to search their PC for the file of interest.  In this example it is the Core CSV file Wellington-KGS-1-32_Core_Data.csv, highlighted below. Select the Open button to display the “Map File Column Number to Rock Column” Dialog.

The “Map File Column Number to Rock Data Column” Dialog allows the user to map the file columns number to the web app tops data structure.  In this example the file has the well information in line one of the Core Data CSV File and line two of the Core Data CSV File has the file data columns.  In this case the chosen file columns match the Core Mnemonics for the core data structure.  The File Column Number is automatically assigned to the Rock Data Column Names. The user only needs to select the “Load Data” Button to parse the Core Data into the web app.

Core Data CSV (Comma Separated Values) File Structure.

The Wellington KGS 1-32 Core Data CSV example has two introduction lines, the first line is the well header information and the second line is the actual column labels for the core data, illustrated below,

Figure: Partial Contents of the Wellington-KGS-1-32_Core_Data.csv File.

The “Map File Column Number to Rock Data Column” Dialog allows the user to map the data in the Core Data CSV File to the web app data structure variables.  The program first reads the first and second line of the CSV File looking for the data column headers.    The lines are each parsed to single out the data column headers and to match those headers to the core data structure.  The program then assigns the column number to the Rock Data Column Name starting at column 1,2,3, … if the file column name used matches the expected rock data column name.  The Column Names matrix used to parse the file column variables are listed below,

| Mnem | Description | Mnem | Description |
| --- | --- | --- | --- |
| Depth Data |  | Density Data |  |
| TOP | Depth Top | GMCC | Grain Density (gm/cc) |
| BASE | Depth Bottom | RHOD | Density of Rock Dry (gm/cc) |
| CORR | Depth Correction | RHOW | Density of Rock Wet (gm/cc) |
| Stratigraphic & Environment Descriptions |  | Permeability Data |  |
| STU | Stratigraphic Unit | KMAX | Whole Core Permeability Maximum |
| STN | Stratigraphic Name | K90 | Whole Core Permeability 90 deg |
| ENV | Depositional Environment | KVRT | Whole Core Permeability Vertical |
| LITHO | Lithofacies | KPLG | Core Plug Permeability Routine |
| Porosity Data |  | KKL | Core Plug Permeability KL Routine |
| PCORE | Whole Core Porosity | KINSI | Core Plug Permeability Insitu |
| PPLUG | Core Plug Porosity Routine | KKLIN | Core Plug Permeability KL Insitu |
| P800 | Core Plug Porosity 800 PSI | KPVRT | Core Plug Permeability Vertical |
| PINSI | Core Plug Porosity Insitu | Archie Constants |  |
| PEFF | Effective Rock Porosity | MAMB | Archie Cementation Ambient |
| Saturation Data |  | MINSI | Archie Cementation Insitu |
| SOIL | Oil Saturation | NAMB | Archie Saturation Ambient |
| SW | Water Saturation | NINSI | Archie Saturation Insitu |
| Mnem | Description | Mnem | Description |
| Radioactive Data |  | Computed Data Types |  |
| GR | Gamma Ray | COMPUTED | Th/U Thorium/Uranium Ratio |
| CGR | Gamma Ray Minus Uranium | COMPUTED | Th/K Thorium/Potassium Ratio |
| PGR | Pseudo Gamma Ray | COMPUTED | Gamma Ray |
| THOR | Thorium Concentration | COMPUTED | Grain Density (gm/cc) |
| URAN | Uranium Concentration | COMPUTED | Porosity |
| Unknown Linear Data |  | Unknown Log Data |  |
| LIN_1 | Linear Track Curve 1 | LOG_1 | Semilog Track Curve 1 |
| LIN_2 | Linear Track Curve 2 | LOG_2 | Semilog Track Curve 2 |
| LIN_3 | Linear Track Curve 3 | LOG_3 | Semilog Track Curve 3 |
| LIN_4 | Linear Track Curve 4 | LOG_4 | Semilog Track Curve 4 |

The Wellington KGS 1-32 Core Data CSV File example above line 2 has only the Top, Base KMAX, K90, KVRT, GMCC, PCORE, SW and SOIL as the column name variables.  The program was able to map each of the column headers to the core data structure, i.e.

| Column | File Column Label | Core Data Name |
| --- | --- | --- |
| 1 | NO |  |
| 2 | TOP | Depth Top |
| 3 | - |  |
| 4 | BASE | Depth Bottom |
| 5 | KMAX | Whole Core Permeability Maximum |
| 6 | K90 | Whole Core Permeability 90 deg |
| 7 | KVRT | Whole Core Permeability Vertical |
| 8 | GMCC | Grain Density (gm/cc) |
| 9 | PCORE | Whole Core Porosity |
| 10 | SW | Water Saturation |
| 11 | SOIL | Oil Saturation |

When the user selects the “Load Data” Button on the “Map File Column Number to Rock Data Column” Dialog the data is parsed into the LAS File Viewer Program, where the Core Data CSV file name is entered into the “PC ASCII Files:” Panel as well as the data type source.

Importing PC Data – Geologist Report (Cuttings/Core Descriptions, Measured Sections) ASCII Delimited File.

Some of the web apps will use the same input dialogs to import Geologist Report ASCII Delimited file.  The Load Data Dialog is basically the same for most of the Web Apps, except they only load a subset of the total data types.  In this example a Geologist Report ASCII Delimited file is being imported into the web app.

Left Click on the “Geologist Report” Icon Button in the Data Source Panel of the Load Data Dialog.  This will display the “Select Comments/Remarks/Notes Delimited File from your PC” Dialog.  This dialog allows the user to search their PC for the file of interest.  In this example it is the Geologist Delimited file Wellington-KGS-1-32_geo.txt, highlighted below. Select the Open button to display the “Parse Comments/Remarks/Notes ASCII Text File” Dialog.

The Select Comments/Remarks/Notes Delimited File from your PC Dialog allows the user to import the geologist report into the web app.  The data is parsed into the one of number of rock description data structures, i.e. Rock Color, Rock Lithology, Porosity, Sedimentary Structure, and General Fossils.

There are two sections to this dialog the Top controls how the depth data is found and interpreted.  The “User defined” Depth assumes that the first two columns will be the start and stop depth followed by the text as this example below shows.  The “Bedding Thickness” Depth assumes that there is one depth with the description. The user should take care to use a delimiter that does not appear in the description if the bedding thickness follows the bed descriptions.

The  “Delimiters, i.e. ,;:()” text field has default ‘,’ by default, which for the example below, showed be changed to ‘;’ to match the depth data separation delimiter shown. And the “Start at Row” text field should be changed to 6 since the data starts at line 6. The default depth is in feet, but it is possible that a measured section would be measured in inches, note this setting is for all depths in the file. This panel allows the user to see all the data and to edit the Depth Delimiter, Starting point of the Data and Bedding Depth Start text fields to match the data and then select the “Parse Data” to parse the text into the rock data structures.

Geologist Report ASCII Delimited File Structure.

By Depth Range:

The Wellington KGS 1-32 Geologist Report Delimited file example has a more relaxed format.  The well header information is at the top of the file with as many lines needed.  The data starts immediately after the header section.  The Geologist Report Example for the Wellington KGS 1-32 well is as follows,

Figure: Partial contents of the Wellington-KGS-1-32_geo.txt File.

In this example the depth range information is separated by semicolons (;) and is in the front of each description, e.g. “2340; 2344; Sh, gy, dark gy, frm”.  The depth range parse engine assumes that there will be two numbers at the beginning of each description.  The semicolon is not necessarily unique in the line, but the program expects to find two number fields at the front of the line.  The program will separate the description from the depth range using the delimiter ‘;’. Using an example line from above,

“2340; 2344; Sh, gy, dark gy, frm”

The parse engine will determine which part is the description and which is the depth information.  The depths are then cleaned of any other non-numeric characters leaving the numbers, e.g. 2340 and 2344.  The parse engine sets the starting depth and ending depths for the description and computes the thickness and adds to the cumulative total depth. The description is parsed later (Lithology, Rock Color, Porosity, Sedimentary Structure, Fossils and Fossil Genera/Species Names) when the user selects the “Parse Data” Button.

By Bedding Thickness:

The next example is a measured section done in Riley County, Kansas, see image below. Only the bedding thickness is available for each layer and is placed at the end of the description in parenthesis, e.g. “Shale, gray, red zone near top (10.8)”.  The user must chose the “Bedding Thickness” radio button in the “Depth Position By” Panel for this type of example. The bedding thickness parse engine assumes that the line can be divided in two strings.  In this example the parenthesis is the unique delimiter separating the text from the bedding description.  The program will separate the text from the bedding thickness using the delimiter ‘(‘.  Using the example above,

“Shale, gray, red zone near top (10.8)”

The parse engine will determine which part is the bedding description and which is the bedding thickness.  The bedding thickness is then cleaned of any other non-numeric characters leaving the number, e.g. 10.8.  The parse engine adds the thickness to the cumulative total depth and sets the starting depth and ending depth of the bed. The description is parsed later (Lithology, Rock Color, Porosity, Sedimentary Structure, and Fossils when the user selects the “Parse Data” Button.

Figure: Partial contents of the Measured Section in Riley County, Kansas ASCII Text File.

The Measured Section Example can be downloaded from the server to the user’s PC. Note that downloading the file directly, web page will insert HTML into the text and change the structure of the document. The URL Links are as follows,

ASCII Text: http://www.kgs.ku.edu/Gemini/Tools/documentation/C_S_line_S10-T6S-R7E.txt                                Zip File: http://www.kgs.ku.edu/Gemini/Tools/documentation/C_S_line_S10-T6S-R7E.zip

When the user selects the “Parse Data” Button on the “Parse Comments/Remarks/Notes ASCII Text File” Dialog the data is parsed into the LAS File Viewer Program, where the Geologist Report Delimited file name is entered into the “PC ASCII Files:” Panel as well as the data type source.

Select the “Continue” Button to create a LAS File Viewer Plot as illustrated below,

Figure: LAS File Viewer Plot of the Wellington KGS 1-32 with all the Data, Log, Tops, Core, and Cuttings/Core Descriptions.

Figure: Measured Section Center Section Line, Township 6S-Range 7E-Section 10, Riley County, Kansas.

LAS File Viewer Plot Control Dialog & Plot

The “Load Data” Dialog is the entry to the LAS File Viewer Plot.  The user searches the KGS Database for well data and/or from the User’s PC.  The Image below suggests that the well data came from both the user’s PC and the KGS Database. Once the well data is loaded the “Continue” Button at the bottom of this dialog becomes enabled. Click on the “Continue” Button to plot the wells data.

The “LAS File Viewer Plot Control” dialog allows the user to change the presentation of the LAS File Viewer Plot, by depth range, by depth scale, by data type, by log type, modify the track curve limits, or add, modify or delete data through data entry dialogs.

The Load Data is the primary source for the LAS File Viewer plot, but the LAS File Viewer Plot Dialogs allow the user to add, modify or delete certain well data types, i.e.

- LAS File Viewer Plot Control Dialog

  - Edit Header Information Button – This button will display the “Edit Well Header” Dialog, which allows the user to modify the default well header information from the Log ASCII Standard (LAS) File or the user can search the KGS Well Header Information Database for the well header information of the well.

- LAS File Viewer Plot Dialog – Horizons Plot Tracks

  - Porosity & Resistivity (Conductivity) Colorlith Color Schema Plot Track – The user can left click the mouse on the Porosity & Resistivity (Conductivity) Colorlith Track to change the log curve that will display the colorlith track and the limits to compute the linear color schema plot track.

  - Stratigraphic Units Plot Track – The user can left click the mouse on the stratigraphics units plot track to display the “Enter Horizon Data” Dialog with the “Stratigraphic Units” Data Entry Panel displayed.  This panel assists the user in adding, modifying or deleting tops from the LAS File Viewer plot.  This dialog has two buttons to set the Stratigraphic Units for a top, i.e.

    - ICS (International Commission on Stratigraphy) Chart Button displays the accepted stratigraphic units.

    - 1968 Kansas Chart Button displays the Accepted Kansas stratigraphic units.

Figure: Wellington KGS 1-32 LAS File Viewer Plot with Log, Tops, Core, and Cuttings/Core Descriptions.

Save Well Data as Log ASCII Standard (LAS) version 3.0 File

The LAS (Log ASCII Standard) is rapidly becoming the accepted industry standard for electronic transmission of digital wire-line logs. Earlier digital formats were commonly coded in binary (such as LIS) and so required specialized software to read them. The LAS standard was introduced by the Canadian Well Logging Society (http://www.cwls.org/) in 1989 to standardize the organization of digital log curve information for personal computer users. It did this very successfully and the standard became popular worldwide. Version 1.2 was the first version and was followed in September 1992 by version 2.0 to address some inconsistencies. A more versatile version LAS 3.0 was released in 1999 however at present LAS 2.0 remains the dominant product. LAS 3.0 clarify several of the poorly defined specifications of LAS 2.0 and provide expanded data storage capabilities, but have seen limited implementation.

The GEMINI Tools programs will read either a Log ASCII Standard (LAS) version 2.0 or 3.0 file and version 1.2 but the Well Information Section is backward in data definition and will not be parsed correctly in the GEMINI Tools web apps.

The sections defined for the LAS 2.0 standard are as follows (http://www.cwls.org/wp-content/uploads/2014/09/LAS_20_Update_Jan2014.pdf):

- "~V" (also known as "~VERSION INFORMATION SECTION") is a required section; has formatting requirements; must be the first section; identifies the version number and whether data is in "wrapped" or "un-wrapped" mode.

- "~W" (also known as "~WELL INFORMATION SECTION") is a required section; has formatting requirements; is preferably the second section; contains information on the well name, location, and start and stop values of the data in this file.

- "~C" (also known as ~CURVE INFORMATION SECTION") is a required section; has formatting requirements; contains curve mnemonics and their definitions in the order that they appear in the data section.

- "~P" (also known as ~PARAMETER INFORMATION SECTION") is an optional section; has formatting requirements; contains information on parameters or constants relevant to the wellbore such as mud resistivity, wire line engineer, truck number, elevation data, etc.

- "~O" (also known as "~OTHER") is an optional section; has no formatting requirements; contains other information or comments.

- "~A" (also known as ~ASCII LOG DATA") is a required section; has formatting requirements; is the last section in the file and also referred to as the data section. The index of the data columns is either Depth or Time. The index values always appear in the first column and each column of data must be separated by at least one space (ASCII 32). All values in the ASCII log data section must be floating point or integer (long) values. Other formats such as Text or Exponential values are not supported.

LAS 3.0 (http://www.cwls.org/wp-content/uploads/2014/09/LAS_3_File_Structure.pdf) will be used to save the well data for the GEMINI Tools web apps since it can hold all the well data in one file. You can even think of LAS 2.0 as a subset of LAS 3.0 since the LAS 2.0 is only concerned with the LOG Data. Note this section includes some of the referenced LAS 3 File Structure PDF; see the above URL for the complete LAS 3.0 File structure.

The ~Version and ~Well sections must appear in every LAS 3.0 file as the first and second sections respectively.

Other sections are grouped by data type. Each group consists of two or three sections; a Parameter Data section (optional for all but Log data), a Column Definition section, and a Column Data section, in that order.

For example, core analysis data would have the following three sections:

~Core_Parameter
~Core_Definition
~Core_Data.

At least one group or data type of either the defined LAS 3.0 data types or a user defined type must exist in every LAS 3.0 file.

The Column Definition and the Column Data sections for each data type are matched sets and must both appear in that order. The corresponding Parameter Data section is optional (except for Log data), but if used must appear before the corresponding Column Definition Section.

LAS 3.0 defines six specific well related data types and their root Section Title names. They are:

~Ascii or ~Log
~Core
~Inclinometry
~Drilling
~Tops
~Test

Additional data types can be defined by the user and content rules discussed elsewhere in the document may define other section titles.

Stand alone user defined Parameter Data sections can be included. Care must be taken to use standalone Parameter Data sections only when the data contained does not fit into any of the other defined data types.

When used, the section order of each set of the three sections for each data type must be Parameter, Definition, and then Data.

Blank lines and comment lines can appear within Column Data sections, but can only appear BEFORE the first Column Data line of that section, or after the LAST Column Data line of that section.

The names of each channel can optionally appear above each channel as a comment line immediately before, after or on the section title line of that section if space allows.

Note: Do not use the ~Other section recognized by LAS version 2.0. It is no longer allowed in LAS 3.0. Any data that can be stored in this section must now be stored properly in a user defined Parameter Data or Column Data section.

### The LAS version 3.0 file has the potential to hold all the well data that was collected, i.e. multiple log data files, core data, tops data, DST data, Perforation data, Cuttings Report data, etc.  As an example the Newby 2-28R has log, core, tops, perforation data as well as the PfEFFER data created from the log analysis tool, PfEFFER-java.  You can view the file at the following URL addresses,

### LAS 3.0 Example File: http://www.kgs.ku.edu/Gemini/Tools/documentation/ Wellington_KGS_1-32_LAS3.las

### LAS 3.0 in WinZip File: http://www.kgs.ku.edu/Gemini/Tools/documentation/ Wellington_KGS_1-32_LAS3.zip

### Standard LAS 3.0 Data Sections for the Wellington_KGS_1-32_LAS3.las

- ~Version - Version Section

- ~Well - Well Header Information Section

- ~Log - Log Data Section – for single log data file, see LAS 3.0 PDF reference below for multiple log data runs.

  - ~Parameter

  - ~Curve

  - ~ASCII, which must be the last section in the LAS File if the Log data is present.

- ~Core - Core Data Section

  - ~Core_Parameter

  - ~Core_Definition

  - ~Core_Data | Core_Definition

- ~Tops - Formation Top (Stratigraphic Units) Picks Data Section

  - ~Tops_Parameter

  - ~Tops_Definition

  - ~Tops_Data | Tops_Definition

### GEMINI Tools Defined LAS 3.0 Data Sections for the Wellington_KGS_1-32_LAS3.las

- ~IQ_Control - Recreate the LAS File Viewer Plot Data Section

  - ~IQ_Control_Parameter

  - ~IQ_Control_Definition

  - ~IQ_Control_Data | IQ_Control_Definition

- ~IQ_Geo_Report – This data holds the geologist cuttings report/core description

  - ~ IQ_Geo_Report _Parameter

  - ~ IQ_Geo_Report _Definition

  - ~ IQ_Geo_Report | IQ_Geo_Report_Definition

- ~IQ_Images – This data holds the file location of Core Image JPEG images.

  - ~ IQ_Images _Parameter

  - ~ IQ_Images _Definition

  - ~ IQ_Images | IQ_Images_Definition

- ~IQ_Brine – This data holds the measured brine data.

  - ~ IQ_Brine _Parameter

  - ~ IQ_Brine _Definition

  - ~ IQ_Brine | IQ_Brine_Definition

- ~IQ_Las_Parameter - Selected LAS Curves – This Parameter Section was designed to remember the Log Curves selected by the user so the user does not have to map LAS File Curve Mnemonics to KGS Standard Tool Mnemonics when they run this file with other GEMINI Tools.

The Tabs at the top identifies the data that will be saved to the Log ASCII Standard (LAS) version 3.0 File.  This example saves the well info, log data, core data, brine data, tops data, LAS File Viewer plot control data, cuttings/core description and core image file locations on the KGS Server. The “LAS File Data Types” Dialog allows the user to modify the well header data that is being saved to the LAS version 3.0 file.  Also noted in the two images are identified “required” fields for a valid LAS file.  The GEMINI Tools Web Apps doesn’t care that the LAS file is not valid, just that the data follows the basic rules for the well data sections in the LAS 3.0 file.

Data Entry Dialogs

Edit Header Information Dialog

The Well Header Information Summary is displayed in the “Header Information” Panel on the LAS File Viewer Control Dialog as well as at the top of the LAS File Viewer Plot.

It is obvious that there are fields missing in the “Header Information” Panel, i.e. Status, Latitude & Longitude, etc.  The “Edit Header Information” Button allows the user to edit the header information that is in the program.  This missing information applies to the LAS File Viewer Plot.

The user can select the “Edit Headers Information” Button to display the Edit Header Information Dialog.  The data displayed holds the initial information stored in the Log ASCII Standard (LAS) file and the Geologist Report ASCII Delimited file if the user loads the files from their PC.  If the user loads the Log ASCII Standard (LAS) File from the KGS Server then the Well Header Information is automatically downloaded from the KGS Well Header Database Table.

The Header Information Dialog displays the contents of the header information data structure.  The user can edit the fields and select the “Ok” Button to transfer the information back to the LAS File Viewer Program and any summary information will be updated in the LAS File Viewer Control and Plot.

As this example illustrates there are missing fields in the header information data.  The user can select the “Search KGS Database for Well Header Information” Button, which will display a “Search for Data on KGS Server” Dialog that will allow the user to build a query that will download all wells that match the query.

This will display the “Search for Data on KGS Server” Dialog, see image below.  This dialog allows the user to search the KGS database for well header data.  In this example, the well of interest will be the Wellington KGS 1-32.

As the Summary image suggests there are 3 methods for searching for the well header information within this dialog,

- By API-Number – KGS has a specific format for the API-Number, i.e.SS-CCC-99999 where SS is the state code for Kansas 15, CCC is the county code for Wellington KGS 1-32 it is 191 for Sumner County and the 5-Digit Well Number for the Wellington KGS 1-32 is 22591.

- By Partial Lease Name – The stored procedure used to retrieve the well header information allows the user to enter a partial phrase, in this example Wellington.  The program places a ‘%’ in front and back of the phrase and sends the request to the Database, i.e. “%Wellington%”.

- By Township Range Section – This search is by location in Kansas, this search also allows the user to enter just the Township and Range to search for wells, e.g. to look for the Wellington KGS 1-32, enter Township as 31 set the S (South) Radio button and Range as 1 set the E (East) Radio button.

The user only needs to enter the above data and select the “Search” Button to display the list of Wells in the Kansas Database that match the search criteria.  In the image below the Lease Name “Wellington” is entered to search for all wells in Kansas with the Phrase Wellington in it. The user searches through the list until they find the well of interest.  In this example it is the Wellington KGS 1-32, which is highlighted.

The user clicks on the “Select” button to transfer the header information to the Edit Header Information Dialog.

The “Show KGS Well Header Data” radio button will become enabled if the KGS well header information has been downloaded.  The user can move between the initial header information by selecting the “Show Initial Header Data” radio button the KGS well header information by selecting the “Show KGS Well Header Data” radio button.

There a number of buttons on the panel that will allow the user to change or compute data in the header information.  The “Kansas TRS to Latitude, Longitude & Elevation” Button computes the latitude, longitude and elevation from the township, range and section by making an Oracle PL/SQL Stored Procedure, e.g.

http://chasm.kgs.ku.edu/ords/iqstrat.TRS2LL_pkg.getXML?iTownship=31&sTownship=S&iRange=1&sRange=E&iSection=32

The call will return a XML with the latitude, longitude and ground level elevation.

<?xml version="1.0"?>

<!DOCTYPE headers [

<!ELEMENT headers (data*)>

<!ATTLIST headers records CDATA #IMPLIED>

<!ELEMENT data (info*,

other?,

loc?,

xy?,

z?,

comments?,

misc?,

cnt?)>

<!ELEMENT info  EMPTY>

<!ATTLIST info  kid   CDATA #IMPLIED

well_kid   CDATA #IMPLIED

key   CDATA #IMPLIED

type  CDATA #IMPLIED

api   CDATA #IMPLIED

name   CDATA #IMPLIED

status CDATA #IMPLIED

error  CDATA #IMPLIED>

<!ELEMENT other EMPTY>

<!ATTLIST other operator  CDATA #IMPLIED

oper_kid  CDATA #IMPLIED

field     CDATA #IMPLIED

field_kid CDATA #IMPLIED>

<!ELEMENT loc EMPTY>

<!ATTLIST loc state     CDATA #IMPLIED

state_cd  CDATA #IMPLIED

county    CDATA #IMPLIED

county_cd CDATA #IMPLIED

loc       CDATA #IMPLIED

town      CDATA #IMPLIED

town_dir  CDATA #IMPLIED

range     CDATA #IMPLIED

range_dir CDATA #IMPLIED

section   CDATA #IMPLIED>

<!ELEMENT xy  EMPTY>

<!ATTLIST xy  latitude  CDATA #IMPLIED

longitude CDATA #IMPLIED

zone      CDATA #IMPLIED

utm_x     CDATA #IMPLIED

utm_y     CDATA #IMPLIED>

<!ELEMENT z   EMPTY>

<!ATTLIST z   depth     CDATA #IMPLIED

gl        CDATA #IMPLIED

kb        CDATA #IMPLIED

df        CDATA #IMPLIED>

<!ELEMENT comments (#PCDATA)>

<!ELEMENT misc EMPTY>

<!ATTLIST misc user      CDATA #IMPLIED

access    CDATA #IMPLIED

source    CDATA #IMPLIED

date      CDATA #IMPLIED>

<!ELEMENT cnt EMPTY>

<!ATTLIST cnt  las       CDATA #IMPLIED

tops      CDATA #IMPLIED

core      CDATA #IMPLIED

images    CDATA #IMPLIED>]>

<headers records="1">

<data>

<loc town="31" town_dir="S" range="1" range_dir="E" section="32" />

<xy latitude="37.311703" longitude="-97.339619" />

<z gl="1277" />

</data>

</headers>

The “UTM” Button will compute the UTM XY coordinates from the latitude & longitude. The analysis uses A Working Manual by John P. Snyder, U.S. Geological Survey Professional Paper 1395, USG Printing Office, Washington, DC, 1987 (http://pubs.er.usgs.gov/djvu/PP/PP_1395.pdf ).

The Status button allows the user to change the well status.

Select the “Ok” Button to update the Header Information in LAS File Viewer program.  The “Header Information” Summary Panel in the Control Dialog will change if any of the fields were modified, e.g. the latitude, longitude, status, depth and elevation and the LAS File Viewer Plot.

Change the Plot Track Limits

The “Change Plot Limits” Button on the LAS File Viewer Control Dialog allows the user to change the limits of the log curves.

Notice that the color boxes next to the curve limits of the curves are colored with different colors, which shows the curve type by unit.The Curves are colored by type (data units) as follows,

- Orange -  OHM-M or Resistivity Logs

- Cyan – PU or porosity Logs, Neutron Porosity, Density Porosity, etc.

- Greenish yellow – BARNS/E or Photoelectric Factor Logs

- Green – GM/CC or Bulk Density Log

- Forest Green – USEC/FT or the Acoustic Transit Time Log

- Red – API, PPM or “%” as Radioactive logs, Gamma Ray, Spectral Gamma Ray, etc.

- Blue – MD or Permeability Logs

- Brown – F, FT or IN or Depth

- Middle yellow – FRAC, or other log curve types.

- Dark Violet – UNI or Unknown Linear Curves

- Medium Violet – UNL or Unknown Logrithum Curves

The color coding is the same as the “LAS File Curve Sections” Dialog that helps the user distinguish the type of curves available at a glance.

The user can change as many curves as they wish, understanding that each plot curves are grouped together, i.e. Porosity.  The Resistivity curves are grouped by Plot Track so that if you change the Micro Normal Resistivity (MNOR) and Micro Inverse Resistivity (MINV) the Array Induction Resistivity (AHT) Curves will not automatically change unless the user wishes.

The above changes above are entered, i.e.

- The Photoelectric factor (PE) curve is changed from “0.0 to 20.0” to 0.0 to 10.0”

- The Neutron Porosity (NPHI) curve is changed from “-0.1 to 0.3” to “0.0 to 0.5” which also modifies the Density Porosity (DPHI) and the Sonic Porosity (SPHI) to the same limits.

As noted in the image the LAS File Viewer plot is automatically modified as the user makes changes to each text field.  View the “Litho-Density” Plot track (see image below) reflects the changes made to the plot curves.

Changing the Colorlith – Porosity Imager Linear & Nonlinear Color Schema Tracks

Adding & Modifying Tops

To begin the example uses the measured section ASCII delimited text file.  The Measured_Section-Sec_24-T12S-R18E.txt File should be downloaded to your PC.  As an option the Measured_Section-Sec_24-T12S-R18E_las.las is the Log ASCII Standard (LAS) version 3.0 generated with the all the examples for this help section.

Download either the ASCII Text Files directly or the Zip files extracting the contents into a directory.  The problem with the ASCII Text Files being downloaded directly from a web page is that the web page will alter the contents so it does not retain the basic structure and add HTML text to the file.  The preferred method if you have Zip or WinZip is to download the zip files to your PC and extract.

Measured Section: Strata southeast of Lecompton Kansas, Douglas County, Kansas

| Type | ASCII Text Files |
| --- | --- |
| Measured Section | http://www.kgs.ku.edu/Gemini/Tools/documentation/Measured_Section-Sec_24-T12S-R18E.txt |
| LAS 3.0 | http://www.kgs.ku.edu/Gemini/Tools/documentation/Measured_Section-Sec_24-T12S-R18E_las.las |

| Type | Zip Files |
| --- | --- |
| Measured Section | http://www.kgs.ku.edu/Gemini/Tools/documentation/Measured_Section-Sec_24-T12S-R18E.zip |
| LAS 3.0 | http://www.kgs.ku.edu/Gemini/Tools/documentation/Measured_Section-Sec_24-T12S-R18E_las.zip |

This example will only require the ASCII Delimited File Measured Section (Measured_Section-Sec_24-T12S-R18E.txt) File to be loaded into the Load Data Dialog. The purpose of this exercise is to show how to enter data into the Stratigraphic Units (Tops) Panel.

Presently the LAS File Viewer Plot assumes that the user will load some or all of the data from the “Load Data” Dialog and modify the data in the LAS File Viewer plot data entry dialogs.

Notice that this data has Tops Data and Geologist Report loaded into the LAS File Viewer Program.

To display the LAS File Viewer Plot select the “Continue” button.

The measured section is from the Bulletin 169: Paleoecological Aspects of Kansas Pennsylvanian and Permian Cyclothems, by Raymond C. Moore, University of Kansas, Lawrence, Kansas, http://www.kgs.ku.edu/Publications/Bulletins/169/Moore/.

Paleoecological Aspects of Kansas Pennsylvanian and Permian Cyclothems, by Raymond C. Moore, “Figure 16--Typical section of uppermost Kanwaka Shale and most of Lecompton Limestone southeast of Lecompton, Kansas, showing stratigraphic occurrence of some ecosystems ("A," Avoca-type; "B," Beil-type; "H," Heebner-type; "S," Speiser-type; "ST," Stranger-type; "T," Tarkio-type)”

## Some of the beds where modified to expand the implied sub-beds, i.e. Bed 16. “Limestone (Beil), very shaly in upper part, bluish-gray …” implies that the upper half is shaly, so 2 beds were created to represent this one bed description.  As you noticed in the text the tops are in parenthesis within each bed description.  The text file was also modified to automatically load the tops by placing the top above the first bed to mention it, i.e. Bed 12 has Doniphan in parenthesis; Doniphan Shale was placed just above this bed.  Some generic fossils were also added to Bed 16 to illustrate the type of Beil-type fossils and for the Heebner Shale Phosphatic are also added to the lithology.

### Enter Horizon Data - Stratigraphic Units Panel

The Stratigraphic Units Panel allows the user to Add, Modify or Remove Stratigraphic Units. There are two files that are used to assist in mapping Stratigraphic Units to a specific top name.

- The 2010 International Commission on Stratigraphy Stratigraphic Units and RGB color for the Stratigraphic Units XML File (http://www.kgs.ku.edu/software/gemini/data/ISC.xml) are used to display the tops in the Stratigraphics Plot Track by Age (RGB Color).

- The Kansas Geological Survey (KGS) Stratigraphic Succession in Kansas, edited by D.E. Zeller, December 1968, updated 2012 (http://www.kgs.ku.edu/software/gemini/data/kansas.xml) will help map the Kansas Top Names to Stratigraphic Units, System, Sub-System, Series, etc. and to map the top depth of one top pick to the base depth of another top pick.

The KGS Stratigraphic Succession of Kansas edited by D. E. Zeller, Dec. 1968, updated 2012 (http://www.kgs.ku.edu/PRS/Ozark/TYPE_LOG/Stratigraphic/index.html).

Figure: The 2010 International Commission on Stratigraphy Stratigraphic Units and RGB colors for the Stratigraphic Units.

### Add Lecompton Formation to the Stratigraphic Units List

### This first example is to add a Lecompton Formation, which is part of the KGS Stratigraphic Succession in Kansas. First place the mouse in the Stratigraphic Units Plot Track and left click with the mouse to display the “Enter Horizon Data” Dialog with the “Stratigraphic Units” Panel.

### For this example all tops from Avoca Limestone to Spring Branch Limestone belongs to the Lecompton Limestone Formation.  The user can manually enter the tops with as much information as they wish to display.  If the stratigraphic unit is part of the Kansas Stratigraphic Units List all necessary fields can be loaded by using the “1968 Kansas Chart” button to display the list of Kansas Stratigraphic Units.

### Add Unknown Bed (SG-A Bed) to the Stratigraphic Units List

This second example is to add an unknown bed (SG-A Bed) to the Stratigraphic Units List, which is not part of the KGS Stratigraphic Succession in Kansas. Bed 6 “Limestone (Spring Branch), bluish gray, weathers rich Brown…” has Fusulinid fossils and implies a change in sea level to deeper waters. This example will enter the basic information to the Depth Range text fields, the Stratigraphic Name text filed and setting the Rank as a BED.  The Stratigraphic Units will be provided by the International Commission on Stratigraphy Dialog by selecting the “ICS Chart” Button.  The “International Stratigraphic Chart” Dialog displays the stratigraphic units as a series of colored boxes, each stratigraphic unit is has the recommended RGB (Red-Green-Blue) Color defined by the International Commission on Stratigraphy.  The user only needs to click the colored box to select all the stratigraphic data associated with the selected stratigraphic unit and transfer the data back to the “Stratigraphic Units” Panel in the “Enter Horizon Data” Dialog.

### Modify data for the Kanwaka Shale Formation in the Stratigraphic Units List

This third example is to modify the Kanwaka Shale Formation Data to include an ending depth and to add some other missing terms.

Quick Plot Buttons – Plot Track Type

- Default

Quick Plot Buttons – Plot Track Type

- Log/Colorlith

Quick Plot Buttons – Plot Track Type

- Log Only

Quick Plot Buttons – Plot Track Type

- Georeport

Quick Plot Buttons – Log Data Type

- Litho/Phi

Quick Plot Buttons – Log Data Type

- Resistivity

Quick Plot Buttons – Log Data Type

- Sonic

Quick Plot Buttons – Log Data Type

- Spectral GR

Quick Plot Buttons – Log Data Type

- Gamma Ray